| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0231r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0047r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Signalling and use of HRD parameters for bitstream partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Hendry 5775 Morehouse Drive San Diego, CA 92121, USA Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA Miska M. Hannuksela Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com fhendry@qti.qualcomm.com +1 360 817 8486 sdeshpande@sharplabs.com +358 10 4488000 miska.hannuksela@nokia.com |
| Source: | Qualcomm Incorporated, Sharp Labs of America, Inc., and Nokia |  |  |

_____________________________

# Abstract

This document proposes some changes to the signalling of HRD parameters for bitstream partitions, including changes to the VPS VUI bitstream partition HRD parameters syntax and semantics, the bitstream partition initial arrival time SEI message syntax and semantics, and their use in other places such as HRD operations.

# Introduction

In the bitstream partition based HRD operation in the AHG10 output text in JCTVC-R0010, HRD parameters are signalled for each bitstream partition. The method for signalling of HRD parameters for bitstream partitions has the following shortcomings:

- For each bitstream partition, the HRD parameters for only the highest sub-layer are signalled, thus the conformance of a temporal subset of the bitstream partition cannot be defined and temporal scalability does not fully work.

- In the bitstream partition initial arrival time SEI message, there is a bug in the syntax such that both of the following two cases that should not occur can occur:

  - When NalHrdBpPresentFlag is 1, the initial arrival delay for VCL HRD parameters through vcl_initial_arrival_delay[ i ] syntax elements are not signalled even if VclHrdBpPresentFlag is equal to 1. In this case, the VCL HRD conformance cannot be defined.

  - When NalHrdBpPresentFlag is 0 the initial arrival delay for VCL HRD parameters through vcl_initial_arrival_delay[ i ] syntax elements are signalled even if VclHrdBpPresentFlag is equal to 0. In this case, those signalling are purely wasting bits.

- Each hrd_parameters( ) syntax structure contains information for vps_max_sub_layer_minus1 + 1 sub-layers, even the syntax structure applies to a bitstream has a number of sub-layers that is less than vps_max_sub_layer_minus1 + 1.

- The CPB size and bit rate values for the delivery schedules from different hrd_parameters( ) syntax structures for a bitstream partition should be in certain order to ensure interpolated schedules also work. However, such restrictions are missing.

- For the bitstream-partition-specific CPB operatoins, the values of BitRate and CpbSize as well as the delivery schedule index are not correctly derived.

Also, there are some other bugs, mostly editorial, in HRD operations and other places that use the signalled HRD parameters for bitstream partitions.

We thus propose changes to resolve these issues.

# Proposal

The changes are summarized as follows:

- Each hrd_parameters( ) syntax structure contains information for the number of sub-layers as needed, indicated by a new syntax element num_sub_layer_hrd_minus1[ i ].

- The bug in the bitstream partition initial arrival time SEI message syntax is fixed.

- For each bitstream partition, the HRD parameters for each sub-layer are signalled.

- Constraints are proposed that the HRD parameter schedules signaled for bitstream partition buffers in the vps_vui_bsp_hrd_parameters( ) structure are ordered in decreasing values of cpb_size_value_minus1[ i ] and cpb_size_du_value_minus1[ i ] and in increasing values of bit_rate_value_minus1[ i ] and bit_rate_du_value_minus1[ i ].

It is asserted that these constraints not only solve the problem mentioned above, but also allow for finding a matching bitrate from the schedule list more systematically without requiring to parse all num_bsp_sched_combinations_minus1[ h ] [ i ] schedules. Also it is asserted that these constraints are needed to support consistent derivation of interpolated delivery schedules.

- Constraints are proposed that at least one CpbSize and BitRate value for the BPB delivery schedules is respectively less than or equal to MaxCPB and MaxBR multiplied by a scale factor.

These constraints make sure that at least one delivery schedule can be used based on the level limits for a correct bitream-partition-specific HRD operation.

- In Annex C for bitstream-partition-specific CPB operations the SchedSelIdx value derivation for related delivery schedules is proposed.

- In Annex C for bitstream-partition-specific CPB operation derivation of BitRate and CpbSize is proposed.

- Valid value ranges for some syntax elements are defined.

- Various improvements or corrections to HRD operations and other places that use the signalled HRD parameters for bitstream partitions.

- An improvement in the decoder conformance section using VPS signalled DPB parameters instead of SPS DPB parameters since the DPB operation is based on VPS signalled parameters.

The proposed text changes are in the attachment, marked based on variant 2 of the AHG10 output text in JCTVC-R0010v2.

The specification text for revision 1 contains a few clarifications of the specification of the first option presented in the original submission of this contribution. In revision 1, option 2 of the original submission is no longer proposed.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).