| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0232 |
| --- | --- |

| Title: | Non-SCCE3: High throughput palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes high throughput palette coding method by reducing the number of context coded bins for palette coding. Specifically, for each CU, the number of context coded bins for palette_run is counted and context bins are replaced by bypass bins when the number of context coded bins exceeds a predetermined threshold. The proposed method doesn’t change binarization method for palette_run reducing the worst case context coded bins per pixel for palette_run by 42%.  The results show that the average coding performance is 0.0% and BD rate variation has been observed between 0.2% and -0.2%.

# Problem description and proposed solution

The palette coding tool is a new tool under consideration for screen content coding which operates in the pixel domain. For each CU, palette table including a set of representative pixel values is transmitted and pixel values within the CU are signaled using the palette table. Specifically, each pixel value is represented by index information in the palette table and the special correlation in the index map is removed by using spatial prediction such as horizontal prediction (‘INDEX_MODE’) and vertical prediction (‘COPY_ABOVE_MODE’). Considering index map coding is similar to residual coding in the existing HEVC (and range extension) this contribution identifies throughput issue in current index map coding compared with existing HEVC (and range extension).

1.1 Index map coding

The index map coding structure of the current palette coding is shown in Figure 1. The following is representative syntax elements;

palette_run_type_flag equal to 1 specifies that the decoding process is COPY_ABOVE_MODE where the decoded pixel value is equal to the pixel at the same location in the above row. palette_run_type_flag equal to 0 specifies that the decoding process mode is INDEX_MODE where the pixel’s palette index is coded in the bitstream.

palette_index is an index to the palette entries. If palette_index is equal to palette_size, the pixel is coded as ESCAPE_PIXEL.

palette_run the number of consecutive locations minus 1 with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_MODE or represents the number of consecutive locations minus 1 with the same palette index when palette_run_type_flag is equal to INDEX_MODE.

Since the palette_run_type_flag is context coded and can be operated at each pixel level in worst case, the corresponding syntax element requires about 1 context bin/pixel. (currently, for the first line, palette_run_type_flag is not coded so that the exact worst case would be slightly smaller than 1. For example, this case occurs when CU size is equal to 64 and palette_run_type_flag is coded every pixel position except the first row. It gives 64*63 context bins/64x64 pixels, which gives 0.984375 context bins/pixel). Palette_index map is all bypass coded and palette_run information is context coded in part. In Figure 2, the struture of palette_run coding is dipicted.  The number of context coded bins for palette run is shown as follows.

| Input level | # context coded bins | The number of pixels coded | Average context bins/pixel |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
| 1 | 2 | 2 | 1 |
| 2 | 3 | 3 | 1 |
| 3 | 3 | 4 | ¾ = 0.75 |
| >3 | 3 | N | 3/N |

Therefore, total worst case context bins for index_map coding becomes 1. 984375 bins/pixel ( 1 context coded bin/pixel for palette_run and 0.984375 bins/pixel for palette_run_type_flag.

Figure 1 The structure of Index Map Coding

Figure 2 The coding structure of ‘palette_run’

# Proposed high throughput index map coding

In order to reduce worst case context coded for palette_run coding (ex: from 1 bin/pixel to 0.575 bin/pixel), the change from palette_run coding to palette_run_b coding is determined based on the given condition depicted in Figure 3. More detailed conditions are as follows;

N indicates the number of accumulated context coded bins for palette_run. N is initialized to zero at each CU level and updated after palette_run coding.

TH: indicate the threshold value to determine whether use bypass coding or context coding for palette run coding.  It is depends on CU size as follows

TH = 0.575*(CU_SIZE*CU_SIZE) -3

Here, 0.575*(CU_SIZE*CU_SIZE) represents the total number of context bins for the given CU to guarantee the worst case context bin is bounded by 0.575 bins/pixel. Since palette_run can use upto 3 context bins when it is called, TH value is reduced by 3.

Figure 3 Proposed high throughput index map coding

# Results

Full results are provided in the attached Excel sheets. Table 1 and Table 2 summarize lossy coding results with respect to the SCCE3 palette anchor for full Intra Block Copy and 2CTU search ranges, respectively.

Table 1: Results with respect to the SCCE3 palette anchor for full frame Intra Block Copy (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot.,1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, text & grap. with mot.,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot., 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot.,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% | 0.3% | -0.1% | -0.8% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 103% |  |  |

Table 2: Results with respect to the SCCE3 palette anchor for 2 CTU Intra Block Copy search (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot.,1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| RGB, text & grap. with mot.,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot., 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot.,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 103% |  |  |

Table 3 and Table 4 summarize lossless coding results with respect to the SCCE3 palette anchor for full Intra Block Copy and 2CTU search ranges, respectively.

Table 3: Results with respect to the SCCE3 palette anchor for full frame Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | -0.1% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | -0.1% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  | 100% |  |  |  | 100% |  |  |  |

Table 4: Results with respect to the SCCE3 palette anchor for 2CTU Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | -0.1% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | -0.1% | -0.1% |  |  | -0.2% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, T & G. w/mot.,720p | 0.0% | 0.0% |  |  | -0.1% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | -0.1% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  | 100% |  |  |  | 100% |  |  |  |

# Conclusion

In conclusion, the proposed method reduces the worst case context coded bins per pixel for current palette coding from 1.984375 bins/pixel to 1.5625 bins/pixel.  The results show that the proposed change doesn’t give significant coding performance impact.  Therefore we propose to have it adopt into Screen content coding draft specifications.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed draft text changes:

7.3.8.5 Coding unit syntax

| … |  |
| --- | --- |
| num_ctx = 0; |  |
| while( scanPos < nCbS * nCbS ) { |  |
| if ( yC != 0 && previous_run_type_flag != COPY_ABOVE_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = INDEX_MODE |  |
| previous_run_type_flag = palette_run_type_flag[ xC ][ yC ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) { |  |
| palette_index | ae(v) |
| if( palette_index = = palette_size ) { /* ESCAPE_PIXEL */ |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| scanPos++ |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_ABOVE_MODE ¦ ¦ ( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE && palette_index < palette_size ) ) { |  |
| if(num_ctx > TH) { |  |
| palette_run_bypass | ae(v) |
| } |  |
| else palette_run | ae(v) |
| runPos = 0 |  |
| … |  |
| } |  |
| … |  |