| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0233 |
| --- | --- |

| Title: | Modifications to palette coding for tiles/slices/dependent slices/wavefronts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modification to palette coding for harmonized operation with tiles/slices/dependent slices/wavefronts. The list of proposed changes is given below:

1.	Prevent palette table prediction across tile/slice boundary

2.	For dependent slices, define storage and synchronization process for palette table predictor

3.	For wavefronts, define storage and synchronization process for palette table predictor

# Problem description and proposed solution

The palette coding tool is a new tool under consideration for screen content coding. It uses palette table based signaling of pixel values, spatial prediction of pixel values and palette table prediction to achieve significant compression gains. This contribution identifies points of interaction between palette coding with tiles/slices/dependent slices/wavefronts which leads to these tools breaking and not serving their intended design. Listed below are the cause of these tools breaking and their proposed solution:

- Palette coding uses a palette table prediction process where the previously coded palette table is used as a predictor for the current palette. However, this behavior can lead to prediction across slice/tile boundaries introducing dependencies between tiles/slices. We propose that the palette table prediction be disabled across slice/tile boundary. To achieve this goal we propose to set the previous palette size to 0 during the CABAC initialization process.

- Previous palette tables used for prediction have similar characteristics to context’s of a CABAC. We therefore propose that they be included in the storage and synchronization process of context variables and rice parameter initialization states for dependent slices and wavefronts.

The proposed specification text for the above listed changes is detailed in section 3 below.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed draft text changes:

Listed below is the proposed specification text change. The changes are color-coded to help identify which item they correspond to:

- Prevent palette table prediction across tile/slice boundary

- For dependent slices, define storage and synchronization process for palette table

- For wavefronts, define storage and synchronization process for palette table

Changes common to items 2 and 3 are highlighted using dark green color.

9.3	CABAC parsing process for slice segment data

9.3.1	General

…

The storage process for context variables is applied as follows:

  - When ending the parsing of the coding tree unit syntax in subclause 7.3.8.2, entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 1 or both CtbAddrInRs is greater than 1 and TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 2 ] ], the storage process for context variables, Palette table predictor, and Rice parameter initialization states as specified in subclause 9.3.2.3 is invoked with TableStateIdxWpp, TableMpsValWpp, TablePreviousPaletteEntriesWpp, PreviousPaletteSizeWpp and TableStatCoeffWpp as outputs.

  - When ending the parsing of the general slice segment data syntax in subclause 7.3.8.1, dependent_slice_segments_enabled_flag is equal to 1 and end_of_slice_segment_flag is equal to 1, the storage process for context variables, Palette table predictor, and Rice parameter initialization states as specified in subclause 9.3.2.3 is invoked with TableStateIdxDs, TableMpsValDs, TablePreviousPaletteEntriesDs, PreviousPaletteSizeDs, and TableStatCoeffDs as outputs.

…

9.3.2	Initialization process

9.3.2.1	General

Outputs of this process are initialized CABAC internal variables, and the initialized Rice parameter initialization states StatCoeff, previous palette table previousPaletteEntries, and previous palette size previousPaletteSize.

…

The context variables of the arithmetic decoding engine are initialized as follows:

  - If the coding tree unit is the first coding tree unit in a tile, the following applies:

  - The initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set equal to 0.

  - Otherwise, if entropy_coding_sync_enabled_flag is equal to 1 and either CtbAddrInRs % PicWidthInCtbsY is equal to 0 or TileId[ CtbAddrInTs ] is not equal to TileId[ CtbAddrRsToTs[ CtbAddrInRs − 1 ] ], the following applies:

  - The location ( xNbT, yNbT ) of the top-left luma sample of the spatial neighbouring block T (Figure 9-2) is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding tree block as follows:

( xNbT, yNbT ) = ( x0 + CtbSizeY, y0 − CtbSizeY )	(9-3)

  - The availability derivation process for a block in z-scan order as specified in subclause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbT, yNbT ) as inputs, and the output is assigned to availableFlagT.

  - The synchronization process for context variables is invoked as follows:

  - If availableFlagT is equal to 1, the synchronization process for context variables and Rice parameter initialization states as specified in subclause 9.3.2.4 is invoked with TableStateIdxWpp, TableMpsValWpp, TablePreviousPaletteEntriesWpp, PreviousPaletteSizeWpp, and TableStatCoeffWpp as inputs.

  - Otherwise, the following applies:

  - The initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set equal to 0.

  - Otherwise, if CtbAddrInRs is equal to slice_segment_address and dependent_slice_segment_flag is equal to 1, the synchronization process for context variables and Rice parameter initialization states as specified in subclause 9.3.2.4 is invoked with TableStateIdxDs, TableMpsValDs, TablePreviousPaletteEntriesDs, PreviousPaletteSizeDs, and TableStatCoeffDs as inputs.

  - Otherwise, the following applies:

  - The initialization process for context variables is invoked as specified in subclause 9.3.2.2.

  - The variables StatCoeff[ k ] are set equal to 0, for k in the range 0 to 3, inclusive.

  - The variable previousPaletteSize is set equal to 0.

The initialization process for the arithmetic decoding engine is invoked as specified in subclause 9.3.2.5.

…

9.3.2.3	Storage process for context variables, Palette table predictor and Rice parameter initialization states

Inputs to this process are:

–	The CABAC context variables indexed by ctxTable and ctxIdx.

–	The previous palette table indexed by cIdx and n, and the size of the previous index table.

–	The Rice parameter initialization states indexed by k.

Outputs of this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the initialization process of context variables and Rice parameter initialization states that are assigned to all syntax elements in subclauses 7.3.8.1 through 7.3.8.11, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variables tablePreviousPaletteEntriesSync, PreviousPaletteSizeSync containing the values of the variables previousPaletteEntries[ cIdx ][ n ], previousPaletteSize used in the initialization process of context variables, Palette table predictor and Rice parameter initialization states.

–	The variables tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the initialization process of context variables, Palette table predictor, and Rice parameter initialization states.

For each context variable, the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync are initialized to the corresponding pStateIdx and valMps.

For each Previous palette table entry, where cIdx varies from 0 to ( ChromaArrayType != 0 ? 2 : 0 ), inclusive, and n varies from 0 to previous PaletteSize-1, inclusive, each entry of the table tablePreviousPaletteEntriesSync is initialized to the corresponding value of previousPaletteEntries[ cIdx ][ n ].

For each Rice parameter initialization state k, each entry of the table tableStatCoeffSync is initialized to the corresponding value of StatCoeff[ k ].

The storage process for context variables is illustrated in the flowchart of Figure 9 4.

…

- 9.3.2.4	Synchronization process for context variables, Palette table predictor and Rice parameter initialization states

Inputs to this process are:

–	The variables tableStateSync and tableMPSSync containing the values of the variables pStateIdx and valMps used in the storage process of context variables that are assigned to all syntax elements in subclauses 7.3.8.1 through 7.3.8.11, except end_of_slice_segment_flag, end_of_subset_one_bit, and pcm_flag.

–	The variable tablePreviousPaletteEntriesSync and previousPaletteSizeSync containing the values of the variables previousPaletteEntries[ cIdx ][ n ] and previousPaletteSize used in the storage process of context variables, Palette table predictor and Rice parameter initialization states.

–	The variable tableStatCoeffSync containing the values of the variables StatCoeff[ k ] used in the storage process of context variables, Palette table predictor and Rice parameter initialization states.

Outputs of this process are:

–	The initialized CABAC context variables indexed by ctxTable and ctxIdx.

–	The initialized Previous palette table entries indexed by cIdx and n, and Previous palette table size.

–	The initialized Rice parameter initialization states StatCoeff indexed by k.

For each context variable, the corresponding context variables pStateIdx and valMps are initialized to the corresponding entries pStateIdx and valMps of tables tableStateSync and tableMPSSync.

For each Previous palette table entry n, where cIdx varies from 0 to ( ChromaArrayType != 0 ? 2 : 0 ), inclusive,, each variable previousPaletteEntries[ cIdx ][ n ] is initialized to the corresponding entry of table tablePreviousPaletteEntriesSync. The variable previousPaletteSize is initialized to previousPaletteSizeSync.

For each Rice parameter initialization state, each variable StatCoeff[ k ] is initialized to the corresponding entry of table tableStatCoeffSync.