| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0234 |
| --- | --- |

| Title: | Palette coding and scaling lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modification to palette coding for harmonized operation with scaling lists. The ESCAPE mode in palette coding uses the existing HEVC dequantizer. However, this dequantization process includes within it the use of scaling list. It is proposed that when palette coding mode is used then scaling lists is set to a flat scaling factor array.

# Problem description and proposed solution

Palette coding is a new tool under consideration for screen content coding which operates in the pixel domain. When using ESCAPE mode in palette coding the existing HEVC dequantizer is used. However, this dequantization process includes within it the use of scaling list. The use of scaling list while decoding a coding unit coded in palette coding mode may result in visual artifacts since palette coding operates in the pixel domain. It is therefore proposed to use a flat scaling factor array for palette coding, with each element of the scaling factor array set to 16.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).