| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0236 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0049 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On access unit boundary detection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes some changes into subclause 7.4.2.4.4 "Order of NAL units and coded pictures and their association to access units".

# Introduction

JCT-VC/JCT-3V decided the following as response to JCTVC-P0043 (copied from JCTVC-P meeting notes):

However, in the discussion, it was remarked that non-nested HRD parameters and AU boundary detection for version 1 decoders must consider EL-only AUs to not be separate AUs.

It was remarked that the version 1 text may not be fully clear in that regard, and that this should be clarified.

Decision (BF/Corrigendum): Clarify the text such that decoders conforming to profiles specified in Annex A do not use NAL units with nuh_layer_id > 0 for AU boundary detection and that non-nested HRD parameters describe Annex C operation for this type of AU boundary detection.

This decision was not implemented in the MV-HEVC/SHVC specification text until the Valencia output. However, it is asserted that the following issues remain in the Valencia output text:

- The phrasing "The first of any of the following NAL units after the last VCL NAL unit of a coded picture with nuh_layer_id equal to 0 specifies the start of a new access unit" is used in subclause 7.4.2.4.4 and then the list of these NAL units include e.g. SPS, PPS and prefix SEI NAL units. However, within a "multi-layer" access unit at least an SPS, PPS, or prefix SEI NAL unit may be present after the last VCL NAL unit of the picture with nuh_layer_id equal to 0. In such a case, a version 1 or RExt decoder would determine an access unit boundary at the SPS/PPS/SEI NAL unit (at a wrong location).

- The phrasing in in subclause 7.4.2.4.4 is not explicit whether nuh_layer_id equal to 0 or any nuh_layer_id value is considered in determining the first NAL unit of an access unit.

# Overview of the proposal

The changes proposed into subclause 7.4.2.4.4 can be summarized as follows, while the detailed proposal is indicated with change marks in the accompanying specification text document:

- Rather than using phrasing "The first of any of the following NAL units after the last VCL NAL unit of a coded picture with nuh_layer_id equal to 0 specifies the start of a new access unit", it is proposed to use the following phrasing: "Let firstBlPicNalUnit be the first VCL NAL unit of a coded picture with nuh_layer_id equal to 0. The first of any of the following NAL units preceding firstBlPicNalUnit in decoding order and succeeding the VCL NAL unit preceding the firstBlPicNalUnit in decoding order, if any,"

The proponent suggests careful consideration if this proposed change is editorial or technical from the version 1 and RExt point of view. (If it is judged to be a technical change, another solution to the asserted issue should be found.)

- It is explicitly stated that NAL units with nuh_layer_id equal to 0 only are considered in determining the first NAL unit of an access unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).