Replace subclauses 7.4.2.4.4 with the following (with differences indicated in turquois):

7.4.2.4.4	Order of NAL units and coded pictures and their association to access units

This subclause specifies the order of NAL units and coded pictures and their association to access units for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 10.

NOTE 1 – The association of NAL units to access units may differ depending on which profiles the CVSs conform to or which decoding process is used for decoding the bitstream.

An access unit consists of one coded picture with nuh_layer_id equal to 0, zero or more VCL NAL units with nuh_layer_id greater than 0, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause 7.4.2.4.5.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

Let firstBlPicNalUnit be the first VCL NAL unit of a coded picture with nuh_layer_id equal to 0. The first of any of the following NAL units preceding firstBlPicNalUnit in decoding order and succeeding the VCL NAL unit preceding the firstBlPicNalUnit in decoding order, if any, specifies the start of a new access unit: [Ed. (YK): Within an access unit at least an SPS, PPS, or prefix SEI NAL unit may be present after the last VCL NAL unit of the picture with nuh_layer_id equal to 0. Maybe other NAL units as well. Check the need of more changes.] [Ed. (MH): I agree, although version 1 specification considers any NAL unit of the following list as the start of a new access unit when it appears after the last VCL NAL unit of a coded picture (with nuh_layer_id equal to 0). It is therefore not clear if the change proposed here is editorial.]

–	access unit delimiter NAL unit with nuh_layer_id equal to 0 (when present),

–	VPS NAL unit with nuh_layer_id equal to 0 (when present),

–	SPS NAL unit with nuh_layer_id equal to 0 (when present),

–	PPS NAL unit with nuh_layer_id equal to 0 (when present),

–	Prefix SEI NAL unit with nuh_layer_id equal to 0 (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 and with nuh_layer_id equal to 0 (when present), [Ed. (MH): It should be clarified if only the NAL units with nuh_layer_id equal to 0 are considered here or if nuh_layer_id can have any value in this condition. (YK): Also for other NAL units that may have nuh_layer_id greater than 0. (MH): Version 1 states: "For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (i.e., remove from the bitstream and discard) all NAL units with values of nuh_layer_id not equal to 0." JCT-VC decided earlier as response to JCTVC-P0043 that nuh_layer_id values greater than 0 are not considered when determining access unit boundaries.]

–	NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC55 and with nuh_layer_id equal to 0 (when present),

–	first VCL NAL unit of a coded picture with nuh_layer_id equal to 0 (always present).

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any prefix SEI NAL units are present, they shall not follow the last VCL NAL unit of the access unit.

–	NAL units having nal_unit_type equal to FD_NUT or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63 shall not precede the first VCL NAL unit of the coded picture with nuh_layer_id equal to 0.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

NOTE 2 – VPS NAL units with nuh_layer_id equal to 0, SPS NAL units with nuh_layer_id equal to 0, PPS NAL units with nuh_layer_id equal to 0, prefix SEI NAL units with nuh_layer_id equal to 0, or NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 or UNSPEC48..UNSPEC55, and with nuh_layer_id equal to 0 may be present in an access unit, but cannot follow the last VCL NAL unit of the access unit, as this condition would specify the start of a new access unit.

The structure of access units not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, or RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63 is shown in Figure 8-1.

Figure 8-1 – Structure of an access unit not containing any NAL units with nal_unit_type equal to FD_NUT, SUFFIX_SEI_NUT, VPS_NUT, SPS_NUT, PPS_NUT, RSV_VCL_N10, RSV_VCL_R11, RSV_VCL_N12, RSV_VCL_R13, RSV_VCL_N14, RSV_VCL_R15, RSV_IRAP_VCL22, or RSV_IRAP_VCL23, or in the range of RSV_VCL24..RSV_VCL31, RSV_NVCL41..RSV_NVCL47, or UNSPEC48..UNSPEC63