| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0239 |
| --- | --- |

| Title: | SCE1: Test 5.1 results on improved weighted prediction for color gamut scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Döne Bugdayci, Alireza Aminlou, Kemal Ugur, Miska M. Hannuksela (Nokia) Visiokatu 3, Tampere, Finland E. Alshina (Samsung) | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia, Samsung |  |  |

_____________________________

# Abstract

SCE1 (JCTVC-Q1101) tests several encoder side algorithms on improved Weighted Prediction (WP) for color gamut scalability and also 8x2x2 non-uniform regions matrix based color difference prediction. Test 5.1 of SCE1 proposes more accurate estimation of WP parameters by considering luma and chroma values, and also using more ILR pictures in the reference list of the enhancement layer. Different weigh and offset values are signaled for each of those pictures.

# Introduction

Color Gamut Scalability has been mentioned in the requirements of Scalable Coding Extension of HEVC. It allows addressing the cases the original Enhancement Layer uses a different color gamut than the Base Layer. This can be useful for instance in case of deployment of UHD services compatible with legacy HD devices: HD is using the Rec.709, while UHD is likely to use some of the parameters defined in the Rec.2020. Another realistic use case described in Error: Reference source not found considers UHDTV forward compatibility where color gamut is different between base and enhancement layers.

SCE1 (JCTVC-Q1101) aims to test utilizing Weighted Prediction (WP) design for more efficient coding of color gamut scalability and 8x2x2 non-uniform regions matrix based color difference prediction.

Encoder only algorithmError: Reference source not found that improves the coding efficiency of weighted prediction enabled in SHVC for inter-layer reference was proposed in JCTVC-Q0056.

In this test, following encoder only algorithms for WP were tested:

- More accurate estimation of WP parameters: The separation criteria is improved to take into account the triplet of luma and chroma values.

- Using more ILR pictures: More inter-layer reference pictures are added to reference picture lists and separate weight and offset parameters are signaled for each one of those pictures

# Encoder changes and improvements

## More ILR picture insertion

Intra coded enhancement layer pictures are set as B_SLICE instead of P_SLICE. Inter layer reference pictures are repeated using reference picture list modification syntax in order to signal the additional weight-offset pairs. We tested adding 1, 2, 4 and 8 inter layer reference pictures to each list which corresponds to 2, 4, 8 and 16 regions. We keep the number of inter layer reference pictures at two (one in each list) for the enhancement layer pictures that use temporal reference pictures also.

In order to test 16 and 32 regions, we have removed the constraint on the maximum number of weight parameters to be signalled.

## Weight-offset calculation

According to the total number of ILR pictures in each EL picture, pixels of the upsampled BL picture are divided into several regions. Then for each region, weight and offset parameters are calculated by the least square error (LSE) method using BL pixels in that region and the corresponding EL pixels. Then each weight and offset pair is assigned to an ILR picture. It should be noted that according to the HEVC standard, the number of reference frames for a picture is restricted to 8. Hence, this constraint has been relaxed when 16 or 32 ILR frames are used.

Definitions of different regions when using different numbers of ILR pictures are as below, where AvrY, AvrU and AvrV are the average value of the Y, U and V components of the upsampled BL picture.

### 2 ILR pictures

The UV space is separated into two regions as follows:

Region_Index = ((U+V) < (AvrU+AvrV)) ? 0 : 1

### 4 ILR pictures

Y space is separated into two regions and the UV space is separated into two regions. More specifically:

Region_Index =

2*(((U+V) < (AvrU+AvrV)) ? 0 : 1) +

((Y < AvrY) ? 0 : 1)

### 8 ILR pictures

Y, U and V space are separately separated into two regions. More specifically:

Region_Index =

4*((Y < AvrY) ? 0 : 1)+

2*((U < AvrU) ? 0 : 1)+

1*((V < AvrV) ? 0 : 1)

### 16 ILR pictures

Y space is separated into four regions, U and V spaces are separately separated into two regions. More specifically:

Region_Index =

4*min(3, (Y / (AvrY>>1)))+

2*((U < AvrU) ? 0 : 1)+

1*((V < AvrV) ? 0 : 1)

### 32 ILR pictures

Y space is separated into 8 regions, U and V spaces are separately separated into two regions. More specifically:

Region_Index =

8*min(7, (Y / (AvrY>>2)))+

2*((U < AvrU) ? 0 : 1)+

1*((V < AvrV) ? 0 : 1)

## Rate-distortion optimization

As we use more inter-layer reference pictures, being able to test also 2Nx2N in merge mode helps more in our case than in anchor, so we set ENCODER_FAST_MODE to 2. We have also modified the uni-prediction mode in order to explicitly test each reference picture in each reference picture list. This way, each weight and offset pair obtained from each region is tested in RDO.

# Experimental results

## Summary

In order to provide an overview of the results of different schemes, a summary is provided here that shows the gains of different methods for AI 1x 10bit case:

| Configuration | Gain AI 1x 10bit |
| --- | --- |
| 2 ILR pictures | -2.3% |
| 4 ILR pictures | -3.1% |
| 8 ILR pictures | -3.6% |
| 16 ILR pictures | -3.8% |
| 32 ILR pictures | -- |

## Detailed results

As experiments were run on a heterogeneous cluster, the encoding/decoding times are not very reliable.

2 ILR pictures

4 ILR pictures

8 ILR pictures

16 ILR pictures

32 ILR pictures

Full results will be made available after few remaining simulations are finished.

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).