| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0240 |
| --- | --- |

| Title: | SCCE1 subtest 3.2:Ping-Pong block vector predictor test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lihua Zhu Jizheng Xu Gary J. Sullivan YongJun Wu Sridhar Sankuratri B. Anil Kumar Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Email: | lizhu@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution reports the results of SCCE3 test 3.2 on "Ping-Pong" block vector (BV) prediction in the intra block copy (BC) mode, corresponding to the JCT-VC proposal Q0134 presented during the 17th meeting in Valencia. The "ping-pong" approach with an encoder-selectable predictor for the BV value is described. Compared to SCCE3 intra BC anchor, the prediction method provides gains of 2.0% for RGB text and graphics with motion (TGM) 1080p AI lossy coding.

# Proposed approach

The "Ping-Pong" block vector predictor approach was proposed in [1]. Two block vector prediction values are maintained as state information. We refer to these as PMV0 and PMV1. For each block vector to be sent, a flag is sent to select between them for the block vector prediction value.

As previously proposed, after a new block vector value is produced, the prior PMV1 will be replaced by the prior value of PMV0, and PMV0 will be replaced by the new block vector value.

On the encoder side, the encoder will select between the two PMVs as the block vector prediction from the Ping-Pong block vector predictor pair, and will send an additional PMV flag to indicate which BV predictor should be used.

On the decoder side, the decoder parses the PMV flag to decide which PMV should be selected, and uses the selected PMV as the BV predictor.

Fig.1 Ping-Pong motion vector predictor

We need one additional MV memory to keep one more PMV stored for this approach.

This process could be applied in the same way as each block vector is produced. We refer to that approach as "Solution 2". Here we also describe a "Solution 1" that has a minor modification for the NxN case in which the updating process is skipped for the lower NxN PUs of a CU. Test results are provided for "Solution 1" and will soon be provided for "Solution 2".

  - Using the same update rule for all block vectors

When we code a PU, we select between PMV0 and PMV1 with the flag to form the prediction. After determining the final value of MV for the current PU, we update the ping-pong cache with the MV value.

The process for updating the ping-pong cache with a particular MV value and for coding the selection indicator flag is the same as described above.

The process for updating the ping-pong cache with a particular mv value is as follows:

if (mv != PMV0) {

PMV1 = PMV0;

PMV0 = mv;

} else {

do nothing;

}

In all the above descriptions, the statement "we select between A and B" means that when the flag is 0, we select A, and when the flag is 1 we select B. The flag that indicates which of the two candidate prediction values is used is coded with CABAC using an additional probability context (initialized to equal probability at the slice level).

Before coding the first CU of each CTU, the value of the two PMVs is initialized. As tested, the value of PMV0 is initialized to (−2W, 0) and PMV1 is initialized to (0, −2W).

## Slight modification and change of encoder search

Two very small modifications were requested to the proposed scheme and a change to the encoding search was requested. The changes to the proposed scheme were as follows, motivated by a desire to harmonize the scheme with what is done in the inter prediction case:

- Switching the order of the BV prediction selection flag and the BV itself (harmonizing the syntax with the inter case)

- Using the same CABAC context for coding the BV prediction selection flag as is used for MV prediction selection (harmonizing the entropy coding with the inter case and avoiding the need for an extra context)

The encoder search was simplified to the point where it became actually somewhat simpler (faster) than the anchor encoder search technique – roughly 5% faster than the anchor for all-intra coding. The simplification was to avoid exhaustive and redundant error measurements in the search process. The cross-checker has been given a copy of the software with the simplified search scheme.

# Simulation results summary

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [2] is followed.

### Lossy coding results

### Lossless coding results

|  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
|  | RGB, text & graphics with motion, 1080p | 1.7% | 1.7% | 1.3% | 2.2% |  |
|  | RGB, text & graphics with motion,720p | 0.4% | 0.5% | 0.1% | 1.2% |  |
|  | RGB, mixed content, 1440p | 0.1% | 0.2% | 0.1% | 0.2% |  |
|  | RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |  |
|  | RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, text & graphics with motion, 1080p | 1.7% | 1.7% | 1.3% | 2.2% |  |
|  | YUV, text & graphics with motion,720p | 0.5% | 0.6% | 0.1% | 1.4% |  |
|  | YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |  |
|  | YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |  |
|  | YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | Enc Time[%] | 109% |  |  |  |  |
|  | Dec Time[%] | 103% |  |  |  |  |
|  |  | Random Access |  |  |  |  |
|  |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
|  | RGB, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.9% | 1.7% |  |
|  | RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |  |
|  | RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |  |
|  | RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, text & graphics with motion, 1080p | 1.1% | 1.3% | 1.0% | 1.7% |  |
|  | YUV, text & graphics with motion,720p | 0.1% | 0.4% | 0.1% | 1.1% |  |
|  | YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |  |
|  | YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | Enc Time[%] | 100% |  |  |  |  |
|  | Dec Time[%] | 104% |  |  |  |  |
|  |  | Low Delay B |  |  |  |  |
|  |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
|  | RGB, text & graphics with motion, 1080p | 1.1% | 1.1% | 0.7% | 1.4% |  |
|  | RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.3% |  |
|  | RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |  |
|  | RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, text & graphics with motion, 1080p | 1.0% | 1.1% | 0.9% | 1.3% |  |
|  | YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.1% | 0.4% |  |
|  | YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | Enc Time[%] | 101% |  |  |  |  |
|  | Dec Time[%] | 106% |  |  |  |  |

# Proposed text modifications (for solution 2)

NOTE: the syntax name "bv_flag" used below could alternatively use the same name as what is used for the coding of the predictor selection flag for inter coding. Here, however, we have used a different name for clarity. This is only a matter of editorial preference and of no real consequence.

### Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

- bv_flag specifying the index of the block vector predictor for the current luma coding block, where bv_flag is a binary value which might be 0 or 1.

Output of this process is the (nCbS)x(nCbX) array of block vectors bvIntra.

The variables nCbS, nCbSw, nCbSh are derived as follows:

nCbS = 1  <<  log2CbSize									(8-1)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-2)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-3)

The variable BvpIntra[ bv_flag ][ compIdx ] specifies the block vector predictor from two block vector predictor candidates: BvpIntra[ 0 ] and BvpIntra[ 1 ]. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. The initialized values for BvpIntra are set as follows:

BvpIntra[ 0 ][ 0 ] = − 2 * nCbS, BvpIntra[ 0 ][ 1 ] = 0.

BvpIntra[ 1 ][ 0 ] = 0, BvpIntra[ 1 ][ 1 ] = − 2 * nCbS.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- The value of bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ bv_flag ][ 0 ]	(8-4)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ bv_flag ][ 1 ]	(8-5)

- When bvIntra[ xPb ][ yPb ][ compIdx ] is not equal to BvpIntra[ 0 ][ compIdx ]), the value of BvpIntra[ 1 ][ compIdx ] is set equal to BvpIntra[ 0 ][ compIdx ] and the value of BvpIntra[ 0 ][ compIdx ] is set equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]

#### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| bv_flag | ae(v) |
| if( PartMode = = PART_2NxN ) { |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| bv_flag | ae(v) |
| } else if( PartMode = = PART_Nx2N ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| bv_flag | ae(v) |
| } else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| bv_flag | ae(v) |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| bv_flag | ae(v) |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| bv_flag | ae(v) |
| } |  |

bv_flag equal to 1 specifies that the current coding block  uses the second motion vector candidate as the motion vector predictor. bv_flag equal to 0 specifies that the current coding block  uses the first motion vector candidate as the motion vector predictor.

# References

- L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft), “Ping-Pong block vector predictor for intra block copy ”, JCTVC-Q0134, JCT-VC Meeting, Valencia, Mar 2014.

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).