| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0243 |
| --- | --- |

| Title: | Cross-check of JCTVC-R0203 Non-SCCE1: Default vector selection for Intra block copy BV prediction, simulation B |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Xingyu Zhang Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7500 xzhang@merl.com cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document provides the results of a crosscheck of Simulation B from JCTVC-R0203, Non-SCCE1: Default vector selection for Intra block copy BV prediction. Two cases were crosschecked: Simulation B using the last 2 stored BV as a default predictor, and Simulation B using the last stored BV as a default predictor. The results of this crosscheck match the corresponding Simulation B results in JCTVC-R0203.

# Cross-check overview

JCTVC-R0203 uses one or two previously-coded BVs as the default BV predictor for intra block copy mode. This cross-check verifies Simulation B from JCTVC-R0203. For the first case, the last two BVs are used to generate the predictor. The second case uses just the last BV for the predictor. The software provided by the authors of JCTVC-R0203 was used for this cross-check, and results are compared to the SCM-1.0 reference using the common conditions for screen content coding JCTVC-Q1015.

# Experimental Results

## Case 1: Simulation B, use last 2 stored BV as default predictor.

### Lossy coding results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.7% | -7.8% | -7.8% |
| RGB, text & graphics with motion,720p | -4.4% | -4.7% | -4.7% |
| RGB, mixed content, 1440p | -3.1% | -3.2% | -3.2% |
| RGB, mixed content, 1080p | -3.7% | -3.6% | -3.7% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -8.3% | -8.4% | -8.3% |
| YUV, text & graphics with motion,720p | -5.2% | -5.3% | -5.4% |
| YUV, mixed content, 1440p | -3.6% | -3.7% | -3.7% |
| YUV, mixed content, 1080p | -4.0% | -4.0% | -4.2% |
| YUV, Animation, 720p | -0.3% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.9% | -3.9% | -3.9% |
| RGB, text & graphics with motion,720p | -2.9% | -2.9% | -2.9% |
| RGB, mixed content, 1440p | -2.0% | -2.0% | -1.9% |
| RGB, mixed content, 1080p | -2.3% | -2.3% | -2.3% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -3.8% | -3.8% | -3.9% |
| YUV, text & graphics with motion,720p | -3.3% | -3.3% | -3.3% |
| YUV, mixed content, 1440p | -2.1% | -2.2% | -2.1% |
| YUV, mixed content, 1080p | -2.8% | -2.6% | -2.8% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -1.1% | -1.0% | -0.9% |
| RGB, mixed content, 1440p | -1.1% | -1.2% | -1.1% |
| RGB, mixed content, 1080p | -1.1% | -1.1% | -1.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.3% | -2.3% | -2.4% |
| YUV, text & graphics with motion,720p | -1.2% | -0.9% | -1.4% |
| YUV, mixed content, 1440p | -1.2% | -1.2% | -1.4% |
| YUV, mixed content, 1080p | -0.8% | -1.0% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

### Lossless coding results

Encoding times for lossless coding results should be ignored as they were run on a system different from that used for the anchors.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.7% | 6.6% | 4.7% | 9.1% |
| RGB, text & graphics with motion,720p | 1.4% | 2.0% | 0.4% | 4.7% |
| RGB, mixed content, 1440p | 1.2% | 1.2% | 1.1% | 1.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.0% | 6.9% | 5.0% | 9.5% |
| YUV, text & graphics with motion,720p | 1.7% | 2.4% | 0.5% | 5.9% |
| YUV, mixed content, 1440p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 66% (NA) |  |  |  |
| Dec Time[%] | 103% (NA) |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.8% | 4.5% | 2.0% | 6.5% |
| RGB, text & graphics with motion,720p | 0.4% | 1.2% | 0.2% | 3.6% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.6% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.8% | 4.7% | 2.2% | 6.8% |
| YUV, text & graphics with motion,720p | 0.4% | 1.5% | 0.3% | 4.5% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.6% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 66% (NA) |  |  |  |
| Dec Time[%] | 112% (NA) |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.8% | 3.7% | 1.5% | 5.2% |
| RGB, text & graphics with motion,720p | 0.3% | 0.5% | 0.1% | 1.3% |
| RGB, mixed content, 1440p | 0.2% | 0.3% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.7% | 3.8% | 1.8% | 5.2% |
| YUV, text & graphics with motion,720p | 0.3% | 0.6% | 0.1% | 1.6% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 65% (NA) |  |  |  |
| Dec Time[%] | 114% (NA) |  |  |  |

## Case 2: Simulation B, use last 1 stored BV as default predictor.

### Lossy coding results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.5% | -7.6% | -7.6% |
| RGB, text & graphics with motion,720p | -4.4% | -4.6% | -4.5% |
| RGB, mixed content, 1440p | -3.0% | -3.1% | -3.1% |
| RGB, mixed content, 1080p | -3.6% | -3.6% | -3.6% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -8.2% | -8.3% | -8.3% |
| YUV, text & graphics with motion,720p | -5.1% | -5.2% | -5.2% |
| YUV, mixed content, 1440p | -3.5% | -3.5% | -3.5% |
| YUV, mixed content, 1080p | -3.9% | -3.9% | -4.1% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.6% | -3.7% | -3.7% |
| RGB, text & graphics with motion,720p | -2.8% | -2.9% | -2.9% |
| RGB, mixed content, 1440p | -1.8% | -1.7% | -1.8% |
| RGB, mixed content, 1080p | -2.2% | -2.3% | -2.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.6% | -3.6% | -3.7% |
| YUV, text & graphics with motion,720p | -3.3% | -3.3% | -3.4% |
| YUV, mixed content, 1440p | -2.0% | -2.1% | -2.2% |
| YUV, mixed content, 1080p | -2.7% | -2.6% | -2.6% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.3% | -2.2% |
| RGB, text & graphics with motion,720p | -1.1% | -1.0% | -1.1% |
| RGB, mixed content, 1440p | -0.7% | -0.9% | -0.9% |
| RGB, mixed content, 1080p | -0.9% | -0.8% | -0.9% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.0% | -2.0% | -2.1% |
| YUV, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| YUV, mixed content, 1440p | -0.9% | -1.0% | -1.0% |
| YUV, mixed content, 1080p | -1.0% | -1.8% | -0.6% |
| YUV, Animation, 720p | -0.2% | -0.6% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

### Lossless coding results

Encoding times for lossless coding results should be ignored as they were run on a system different from that used for the anchors.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.7% | 6.5% | 4.8% | 9.1% |
| RGB, text & graphics with motion,720p | 1.4% | 2.0% | 0.4% | 4.8% |
| RGB, mixed content, 1440p | 1.1% | 1.1% | 1.1% | 1.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.0% | 6.9% | 5.1% | 9.5% |
| YUV, text & graphics with motion,720p | 1.6% | 2.4% | 0.5% | 5.9% |
| YUV, mixed content, 1440p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 66% (NA) |  |  |  |
| Dec Time[%] | 103% (NA) |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.6% | 4.4% | 1.9% | 6.3% |
| RGB, text & graphics with motion,720p | 0.3% | 1.2% | 0.2% | 3.7% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.5% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.7% | 4.6% | 2.2% | 6.6% |
| YUV, text & graphics with motion,720p | 0.4% | 1.5% | 0.3% | 4.6% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.5% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 66% (NA) |  |  |  |
| Dec Time[%] | 112% (NA) |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.6% | 3.5% | 1.5% | 5.0% |
| RGB, text & graphics with motion,720p | 0.2% | 0.5% | 0.1% | 1.3% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.5% | 3.6% | 1.7% | 5.0% |
| YUV, text & graphics with motion,720p | 0.3% | 0.6% | 0.1% | 1.6% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 65% (NA) |  |  |  |
| Dec Time[%] | 113% (NA) |  |  |  |

# Conclusions

The results from this cross-check match the corresponding results for Simulation B in JCTVC-R0203.