| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0244 |
| --- | --- |

| Title: | Cross check of ‘SCCE1: Test 1.1 – Intra block copy with different search areas’ (JCTVC-R0184) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of SCCE1 Test 1.1 on intra-block copy search area. The results match those reported by the proponents.

# Cross check procedure

The source code for test 1.1 of Screen Content Core Experiment 1 (SCCE1), as defined in [1], was provided by Qualcomm. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code provided by Qualcomm was studied and found to correspond to the test description in [1].

In particular, for the 2x3 and 3x5 searches, the search area is restricted to the corresponding region.  For the hash search, candidates outside of the corresponding region are skipped.

# Coding performance measurement

The full-frame anchor is used for reporting each cross-check simulation result.  The simulation conditions are defined in [2].  The BD-rate summary tables for each sub-test are provided in Appendix A.

Table 1 and Table 2 provide summarization (i.e. luma only) of the lossy and lossless simulation results respectively for the sub-tests of test 1.1. For the lossless simulation results, the total bit-rate reduction is reported in Table 2.  Note that results for ‘full-frame’ cases correspond to the anchor, so no BD-rate change is observed.

Table 1. Summary of lossy simulation results for test 1.1

| AI | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| --- | --- | --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 34.7% | 11.4% | 20.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 22.6% | 8.1% | 14.2% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 15.1% | 5.2% | 8.6% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 20.3% | 4.7% | 10.5% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 35.2% | 11.5% | 20.5% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 22.4% | 8.1% | 14.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 16.5% | 5.5% | 9.3% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 21.8% | 5.1% | 11.3% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% | 98% | 97% | 100% | 103% |
| Dec Time[%] | 105% | 103% | 104% | 100% | 102% |
| RA | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| RGB, text & graphics with motion, 1080p | 17.5% | 6.0% | 10.5% | 0.0% | 7.7% |
| RGB, text & graphics with motion,720p | 15.2% | 5.6% | 9.8% | 0.0% | 1.3% |
| RGB, mixed content, 1440p | 8.3% | 2.7% | 4.3% | 0.0% | 2.1% |
| RGB, mixed content, 1080p | 12.0% | 3.0% | 6.7% | 0.0% | 2.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 16.9% | 5.6% | 10.0% | 0.0% | 6.9% |
| YUV, text & graphics with motion,720p | 15.1% | 5.7% | 9.9% | 0.0% | 1.2% |
| YUV, mixed content, 1440p | 10.0% | 3.2% | 5.3% | 0.0% | 2.1% |
| YUV, mixed content, 1080p | 14.4% | 3.4% | 7.8% | 0.0% | 2.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 102% | 102% | 100% | 101% |
| Dec Time[%] | 101% | 100% | 101% | 100% | 101% |
| LB | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| RGB, text & graphics with motion, 1080p | 11.3% | 3.4% | 6.5% | 0.0% | 9.4% |
| RGB, text & graphics with motion,720p | 5.8% | 2.6% | 4.1% | 0.0% | 3.9% |
| RGB, mixed content, 1440p | 4.4% | 1.2% | 2.5% | 0.0% | 2.0% |
| RGB, mixed content, 1080p | 4.4% | 1.3% | 2.6% | 0.0% | 3.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | 10.1% | 3.0% | 5.8% | 0.0% | 8.3% |
| YUV, text & graphics with motion,720p | 5.0% | 2.3% | 3.6% | 0.0% | 3.3% |
| YUV, mixed content, 1440p | 5.3% | 1.5% | 2.5% | 0.0% | 1.9% |
| YUV, mixed content, 1080p | 5.8% | 1.9% | 3.3% | 0.0% | 2.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 103% | 102% | 100% | 102% |
| Dec Time[%] | 101% | 101% | 100% | 100% | 101% |

Table 2. Summary of lossless simulation results for test 1.1

| AI | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| --- | --- | --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | -33.0% | -12.1% | -20.4% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -5.9% | -1.9% | -3.5% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | -12.8% | -9.8% | -10.9% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -11.1% | -5.1% | -8.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -35.2% | -13.1% | -22.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -6.7% | -2.2% | -4.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | -13.0% | -9.4% | -10.7% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -12.1% | -5.1% | -8.4% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% | 95% | 98% | 100% | 100% |
| Dec Time[%] | 105% | 108% | 111% | 100% | 99% |
| RA | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| RGB, text & graphics with motion, 1080p | -21.4% | -6.5% | -12.3% | 0.0% | -20.0% |
| RGB, text & graphics with motion,720p | -1.3% | -0.4% | -0.8% | 0.0% | -0.6% |
| RGB, mixed content, 1440p | -2.3% | -1.6% | -1.8% | 0.0% | -1.0% |
| RGB, mixed content, 1080p | -2.0% | -0.9% | -1.4% | 0.0% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.9% | -6.5% | -12.1% | 0.0% | -19.3% |
| YUV, text & graphics with motion,720p | -1.3% | -0.5% | -0.8% | 0.0% | -0.5% |
| YUV, mixed content, 1440p | -2.4% | -1.5% | -1.8% | 0.0% | -1.1% |
| YUV, mixed content, 1080p | -2.2% | -1.0% | -1.5% | 0.0% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 102% | 102% | 100% | 100% |
| Dec Time[%] | 101% | 100% | 101% | 100% | 101% |
| LB | 2 CTU | 3x5 CTU | 2x3 CTU | Full-frame | Intra: Full-frame Inter: 2CTU |
| RGB, text & graphics with motion, 1080p | -21.2% | -6.3% | -11.9% | 0.0% | -21.0% |
| RGB, text & graphics with motion,720p | -0.7% | -0.2% | -0.4% | 0.0% | -0.6% |
| RGB, mixed content, 1440p | -1.3% | -0.8% | -0.9% | 0.0% | -1.1% |
| RGB, mixed content, 1080p | -0.8% | -0.4% | -0.5% | 0.0% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.2% | -6.1% | -11.4% | 0.0% | -20.0% |
| YUV, text & graphics with motion,720p | -0.7% | -0.2% | -0.4% | 0.0% | -0.6% |
| YUV, mixed content, 1440p | -1.4% | -0.7% | -0.9% | 0.0% | -1.1% |
| YUV, mixed content, 1080p | -1.0% | -0.4% | -0.6% | 0.0% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 101% | 103% | 100% | 100% |
| Dec Time[%] | 101% | 100% | 101% | 100% | 101% |

# Conclusion

The performance of intra-block copy with different search areas is tested in test 1.1 of SCCE1.  The results provided by the proponents are verified in this cross-check report.

# References

- J. Sole, S. Liu, “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”, JCTVC-Q1121, Valencia, ES, March 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, Joint Collaborative Team on Video Coding, JCTVC-Q1015, Valencia, ES, March 2014.

- C. Pang, J. Sole, M. Karczewicz, “SCCE1: Test 1.1 – Intra block copy with different search areas”, JCTVC-R0184, Sapporo, JP, June 2014.

# Appendix A. Full results.

## Search area is 2 CTU (left and current)

Table 3 and Table 4 show the lossy and lossless simulation results respectively for the 2CTU search area sub-test.

Table 3. Lossy simulation result for 2CTU search area.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 34.7% | 34.5% | 34.5% |
| RGB, text & graphics with motion,720p | 22.6% | 22.0% | 22.2% |
| RGB, mixed content, 1440p | 15.1% | 15.0% | 15.2% |
| RGB, mixed content, 1080p | 20.3% | 20.5% | 20.0% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 35.2% | 34.8% | 34.7% |
| YUV, text & graphics with motion,720p | 22.4% | 21.7% | 21.7% |
| YUV, mixed content, 1440p | 16.5% | 16.6% | 16.6% |
| YUV, mixed content, 1080p | 21.8% | 22.1% | 21.6% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 17.5% | 17.2% | 17.2% |
| RGB, text & graphics with motion,720p | 15.2% | 14.8% | 14.9% |
| RGB, mixed content, 1440p | 8.3% | 8.2% | 8.3% |
| RGB, mixed content, 1080p | 12.0% | 12.1% | 12.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 16.9% | 16.6% | 16.6% |
| YUV, text & graphics with motion,720p | 15.1% | 14.8% | 14.9% |
| YUV, mixed content, 1440p | 10.0% | 9.8% | 9.9% |
| YUV, mixed content, 1080p | 14.4% | 13.3% | 13.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 11.3% | 10.8% | 10.9% |
| RGB, text & graphics with motion,720p | 5.8% | 5.6% | 5.5% |
| RGB, mixed content, 1440p | 4.4% | 4.1% | 4.2% |
| RGB, mixed content, 1080p | 4.4% | 4.3% | 3.8% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.1% | 9.8% | 9.7% |
| YUV, text & graphics with motion,720p | 5.0% | 4.7% | 4.9% |
| YUV, mixed content, 1440p | 5.3% | 4.9% | 4.8% |
| YUV, mixed content, 1080p | 5.8% | 1.9% | 3.4% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4. Lossless simulation result for 2CTU search area.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -33.0% | -31.6% | -55.6% | -11.1% |
| RGB, text & graphics with motion,720p | -5.9% | -11.0% | -33.5% | -0.8% |
| RGB, mixed content, 1440p | -12.8% | -11.9% | -16.2% | -7.5% |
| RGB, mixed content, 1080p | -11.1% | -11.1% | -11.1% | -11.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -35.2% | -34.1% | -61.6% | -12.3% |
| YUV, text & graphics with motion,720p | -6.7% | -13.6% | -43.9% | -1.0% |
| YUV, mixed content, 1440p | -13.0% | -12.5% | -16.2% | -8.8% |
| YUV, mixed content, 1080p | -12.1% | -12.1% | -12.1% | -12.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 105% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -21.4% | -21.8% | -38.0% | -4.1% |
| RGB, text & graphics with motion,720p | -1.3% | -6.9% | -25.5% | -0.4% |
| RGB, mixed content, 1440p | -2.3% | -2.4% | -4.2% | -0.6% |
| RGB, mixed content, 1080p | -2.0% | -2.0% | -2.0% | -2.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.9% | -23.5% | -42.4% | -5.3% |
| YUV, text & graphics with motion,720p | -1.3% | -8.9% | -33.5% | -0.4% |
| YUV, mixed content, 1440p | -2.4% | -2.5% | -4.4% | -0.7% |
| YUV, mixed content, 1080p | -2.2% | -2.2% | -2.2% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -21.2% | -17.0% | -24.0% | -3.3% |
| RGB, text & graphics with motion,720p | -0.7% | -2.7% | -9.4% | -0.2% |
| RGB, mixed content, 1440p | -1.3% | -1.4% | -2.6% | -0.1% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.2% | -18.1% | -27.2% | -4.4% |
| YUV, text & graphics with motion,720p | -0.7% | -3.4% | -12.4% | -0.2% |
| YUV, mixed content, 1440p | -1.4% | -1.5% | -2.9% | -0.2% |
| YUV, mixed content, 1080p | -1.0% | -1.0% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

## Search area is surrounding the current CTU (3×5 CTUs, 2×3 CTUs)

Table 5 and Table 6 provide the lossy and lossless simulation result respectively for the 3x5 CTU search area sub-test. Table 7 and Table 8 provide the lossy and lossless simulation result respectively  for the 2x3 CTU search area sub-test.

Table 5. Lossy simulation result for 3x5 CTU search area.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 11.4% | 11.3% | 11.3% |
| RGB, text & graphics with motion,720p | 8.1% | 7.9% | 7.9% |
| RGB, mixed content, 1440p | 5.2% | 5.1% | 5.3% |
| RGB, mixed content, 1080p | 4.7% | 4.6% | 4.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.5% | 11.5% | 11.4% |
| YUV, text & graphics with motion,720p | 8.1% | 7.8% | 7.7% |
| YUV, mixed content, 1440p | 5.5% | 5.6% | 5.7% |
| YUV, mixed content, 1080p | 5.1% | 5.1% | 5.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 6.0% | 5.9% | 5.9% |
| RGB, text & graphics with motion,720p | 5.6% | 5.6% | 5.6% |
| RGB, mixed content, 1440p | 2.7% | 2.6% | 2.7% |
| RGB, mixed content, 1080p | 3.0% | 2.9% | 2.9% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.6% | 5.7% | 5.6% |
| YUV, text & graphics with motion,720p | 5.7% | 5.6% | 5.8% |
| YUV, mixed content, 1440p | 3.2% | 3.1% | 3.3% |
| YUV, mixed content, 1080p | 3.4% | 3.5% | 3.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 3.4% | 3.2% | 3.4% |
| RGB, text & graphics with motion,720p | 2.6% | 2.3% | 2.6% |
| RGB, mixed content, 1440p | 1.2% | 0.9% | 1.1% |
| RGB, mixed content, 1080p | 1.3% | 1.7% | 1.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | 3.0% | 3.0% | 2.9% |
| YUV, text & graphics with motion,720p | 2.3% | 2.3% | 2.2% |
| YUV, mixed content, 1440p | 1.5% | 1.1% | 1.2% |
| YUV, mixed content, 1080p | 1.9% | 0.7% | 1.5% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

Table 6. Lossless simulation result for 3x5 CTU search area.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -12.1% | -11.6% | -23.4% | -3.3% |
| RGB, text & graphics with motion,720p | -1.9% | -3.7% | -11.7% | -0.4% |
| RGB, mixed content, 1440p | -9.8% | -8.5% | -14.7% | -2.4% |
| RGB, mixed content, 1080p | -5.1% | -5.1% | -5.1% | -5.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -13.1% | -12.7% | -25.1% | -4.8% |
| YUV, text & graphics with motion,720p | -2.2% | -4.7% | -15.5% | -0.4% |
| YUV, mixed content, 1440p | -9.4% | -8.5% | -14.2% | -2.7% |
| YUV, mixed content, 1080p | -5.1% | -5.1% | -5.1% | -5.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |  |
| Dec Time[%] | 108% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -6.5% | -7.7% | -14.4% | -1.8% |
| RGB, text & graphics with motion,720p | -0.4% | -2.5% | -9.3% | -0.2% |
| RGB, mixed content, 1440p | -1.6% | -1.6% | -3.1% | -0.2% |
| RGB, mixed content, 1080p | -0.9% | -0.9% | -0.9% | -0.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.5% | -8.5% | -15.7% | -3.0% |
| YUV, text & graphics with motion,720p | -0.5% | -3.3% | -12.3% | -0.1% |
| YUV, mixed content, 1440p | -1.5% | -1.6% | -3.1% | -0.2% |
| YUV, mixed content, 1080p | -1.0% | -1.0% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -6.3% | -5.2% | -7.1% | -1.6% |
| RGB, text & graphics with motion,720p | -0.2% | -1.2% | -4.1% | -0.1% |
| RGB, mixed content, 1440p | -0.8% | -0.8% | -1.5% | 0.0% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.1% | -5.8% | -8.1% | -2.7% |
| YUV, text & graphics with motion,720p | -0.2% | -1.5% | -5.4% | -0.1% |
| YUV, mixed content, 1440p | -0.7% | -0.8% | -1.6% | 0.0% |
| YUV, mixed content, 1080p | -0.4% | -0.4% | -0.4% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 7. Lossy simulation result for the 2x3 CTU search area sub-test.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 20.1% | 19.9% | 19.9% |
| RGB, text & graphics with motion,720p | 14.2% | 13.9% | 14.0% |
| RGB, mixed content, 1440p | 8.6% | 8.5% | 8.7% |
| RGB, mixed content, 1080p | 10.5% | 10.3% | 10.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 20.5% | 20.2% | 20.1% |
| YUV, text & graphics with motion,720p | 14.1% | 13.8% | 13.7% |
| YUV, mixed content, 1440p | 9.3% | 9.3% | 9.3% |
| YUV, mixed content, 1080p | 11.3% | 11.5% | 10.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 10.5% | 10.3% | 10.3% |
| RGB, text & graphics with motion,720p | 9.8% | 9.7% | 9.7% |
| RGB, mixed content, 1440p | 4.3% | 4.3% | 4.3% |
| RGB, mixed content, 1080p | 6.7% | 6.6% | 6.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.0% | 9.8% | 9.7% |
| YUV, text & graphics with motion,720p | 9.9% | 9.7% | 9.7% |
| YUV, mixed content, 1440p | 5.3% | 5.2% | 5.2% |
| YUV, mixed content, 1080p | 7.8% | 7.8% | 7.6% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 6.5% | 6.2% | 6.3% |
| RGB, text & graphics with motion,720p | 4.1% | 3.9% | 4.0% |
| RGB, mixed content, 1440p | 2.5% | 2.0% | 2.2% |
| RGB, mixed content, 1080p | 2.6% | 2.7% | 2.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | 5.8% | 5.6% | 5.6% |
| YUV, text & graphics with motion,720p | 3.6% | 3.3% | 3.8% |
| YUV, mixed content, 1440p | 2.5% | 2.5% | 2.6% |
| YUV, mixed content, 1080p | 3.3% | 2.0% | 2.7% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 8. Lossless simulation result for 2x3 CTU search area sub-test.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -20.4% | -19.5% | -37.2% | -6.0% |
| RGB, text & graphics with motion,720p | -3.5% | -6.8% | -21.0% | -0.7% |
| RGB, mixed content, 1440p | -10.9% | -9.8% | -15.2% | -4.3% |
| RGB, mixed content, 1080p | -8.0% | -8.0% | -8.0% | -8.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -22.0% | -21.3% | -40.8% | -7.6% |
| YUV, text & graphics with motion,720p | -4.0% | -8.5% | -27.9% | -0.8% |
| YUV, mixed content, 1440p | -10.7% | -10.0% | -14.9% | -5.0% |
| YUV, mixed content, 1080p | -8.4% | -8.4% | -8.4% | -8.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 111% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -12.3% | -13.4% | -24.3% | -2.7% |
| RGB, text & graphics with motion,720p | -0.8% | -4.4% | -16.4% | -0.3% |
| RGB, mixed content, 1440p | -1.8% | -1.9% | -3.5% | -0.3% |
| RGB, mixed content, 1080p | -1.4% | -1.4% | -1.4% | -1.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.1% | -14.6% | -27.1% | -4.0% |
| YUV, text & graphics with motion,720p | -0.8% | -5.8% | -21.7% | -0.3% |
| YUV, mixed content, 1440p | -1.8% | -1.9% | -3.5% | -0.4% |
| YUV, mixed content, 1080p | -1.5% | -1.5% | -1.5% | -1.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -11.9% | -9.9% | -14.2% | -2.3% |
| RGB, text & graphics with motion,720p | -0.4% | -2.0% | -6.9% | -0.2% |
| RGB, mixed content, 1440p | -0.9% | -1.0% | -1.9% | -0.1% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -11.4% | -10.8% | -16.2% | -3.5% |
| YUV, text & graphics with motion,720p | -0.4% | -2.5% | -9.2% | -0.2% |
| YUV, mixed content, 1440p | -0.9% | -1.0% | -2.0% | -0.1% |
| YUV, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

## Search area is the entire frame

Table 9 and Table 10 show the lossy and lossless simulation results respectively for the full-frame CTU search area sub-test.

Table 9. Lossy simulation result for full-frame CTU search area sub-test.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 10. Lossless simulation result for full-frame CTU search area sub-test.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

## Search area is the entire frame for intra frames and local search (i.e., 2 CTUs) for inter frames

Table 11 and Table 12 show the lossy and lossless simulation results respectively for full-frame for intra and 2 CTU for inter search area sub-test.

Table 11. Lossy simulation result of full-frame for intra and 2CTU for inter search area sub-test.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 7.7% | 7.3% | 7.4% |
| RGB, text & graphics with motion,720p | 1.3% | 1.3% | 1.3% |
| RGB, mixed content, 1440p | 2.1% | 2.1% | 2.1% |
| RGB, mixed content, 1080p | 2.4% | 2.4% | 2.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.9% | 6.6% | 6.7% |
| YUV, text & graphics with motion,720p | 1.2% | 1.2% | 1.2% |
| YUV, mixed content, 1440p | 2.1% | 2.0% | 2.1% |
| YUV, mixed content, 1080p | 2.5% | 2.4% | 2.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 9.4% | 8.8% | 9.0% |
| RGB, text & graphics with motion,720p | 3.9% | 3.6% | 3.6% |
| RGB, mixed content, 1440p | 2.0% | 2.0% | 2.0% |
| RGB, mixed content, 1080p | 3.1% | 2.9% | 2.8% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 8.3% | 8.1% | 8.1% |
| YUV, text & graphics with motion,720p | 3.3% | 3.1% | 3.3% |
| YUV, mixed content, 1440p | 1.9% | 2.0% | 2.0% |
| YUV, mixed content, 1080p | 2.9% | 1.9% | 2.1% |
| YUV, Animation, 720p | -0.2% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 12. Lossless simulation result for full-frame for intra and 2 CTU for inter search area sub-test.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -20.0% | -11.7% | -22.7% | -3.0% |
| RGB, text & graphics with motion,720p | -0.6% | -0.7% | -1.7% | -0.2% |
| RGB, mixed content, 1440p | -1.0% | -1.1% | -2.2% | 0.0% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -19.3% | -12.4% | -22.2% | -4.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -2.1% | -0.1% |
| YUV, mixed content, 1440p | -1.1% | -1.2% | -2.3% | 0.0% |
| YUV, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -21.0% | -14.7% | -23.5% | -3.2% |
| RGB, text & graphics with motion,720p | -0.6% | -1.8% | -6.0% | -0.2% |
| RGB, mixed content, 1440p | -1.1% | -1.1% | -2.3% | 0.0% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.0% | -15.6% | -22.6% | -4.3% |
| YUV, text & graphics with motion,720p | -0.6% | -2.3% | -7.8% | -0.2% |
| YUV, mixed content, 1440p | -1.1% | -1.2% | -2.4% | 0.0% |
| YUV, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |