| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0246 |
| --- | --- |

| Title: | Cross check of ‘SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4’ (JCTVC-R0191) by Mediatek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of SCCE1 combination test 5.1. Two variants of the combination test are cross-checked, both variants treat the intra-block copy operation as an inter-prediction process and include a merge mode for block vector prediction.  Variant 5.1b includes asymmetric motion partitions (AMP).  The cross-check confirms the simulation results reported by the proponent.

# Cross check procedure

The source code for combination test 5.1 of Screen Content Core Experiment 1 (SCCE1), as defined in [1], was provided by Mediatek. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was provided by Mediatek and configured for combination tests 5.1a and 5.1b.  These configuration are as follows:

5.1a is 2.1 (IBC as inter PU) + 2.3 (Merge mode)

5.1b is 2.1 (IBC as inter PU ) + 2.3 (Merge mode ) + 2.4 (AMP)

# Coding performance measurement

Table 1 and Table 2 show the lossy and lossless simulation result respectively for combination test 5.1a of SCCE1.

Table 1. Lossy simulation result for test 5.1a.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -9.3% | -10.3% | -10.4% |
| RGB, text & graphics with motion,720p | -5.2% | -6.0% | -5.9% |
| RGB, mixed content, 1440p | -4.2% | -4.4% | -4.5% |
| RGB, mixed content, 1080p | -4.5% | -5.0% | -5.0% |
| RGB, Animation, 720p | -0.3% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -9.7% | -10.7% | -10.7% |
| YUV, text & graphics with motion,720p | -5.7% | -5.9% | -6.1% |
| YUV, mixed content, 1440p | -4.1% | -4.7% | -4.8% |
| YUV, mixed content, 1080p | -4.6% | -5.4% | -5.4% |
| YUV, Animation, 720p | -0.2% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.8% | -7.4% | -7.5% |
| RGB, text & graphics with motion,720p | -3.8% | -4.7% | -4.8% |
| RGB, mixed content, 1440p | -2.6% | -3.1% | -3.3% |
| RGB, mixed content, 1080p | -3.0% | -4.1% | -4.1% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -5.6% | -7.2% | -7.3% |
| YUV, text & graphics with motion,720p | -3.8% | -4.5% | -4.7% |
| YUV, mixed content, 1440p | -2.4% | -3.9% | -3.8% |
| YUV, mixed content, 1080p | -3.2% | -5.0% | -4.5% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.9% | -6.3% | -6.2% |
| RGB, text & graphics with motion,720p | -2.1% | -3.0% | -3.0% |
| RGB, mixed content, 1440p | -2.0% | -2.6% | -2.5% |
| RGB, mixed content, 1080p | -1.7% | -3.3% | -3.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | 0.2% |
| YUV, text & graphics with motion, 1080p | -4.4% | -5.9% | -5.7% |
| YUV, text & graphics with motion,720p | -1.4% | -2.5% | -3.2% |
| YUV, mixed content, 1440p | -1.4% | -2.7% | -2.9% |
| YUV, mixed content, 1080p | -1.6% | -4.2% | -3.8% |
| YUV, Animation, 720p | 0.3% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Lossless simulation result for test 5.1a.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.8% | 6.8% | 5.7% | 8.9% |
| RGB, text & graphics with motion,720p | 1.3% | 1.9% | 0.8% | 4.7% |
| RGB, mixed content, 1440p | 2.1% | 1.9% | 1.1% | 2.8% |
| RGB, mixed content, 1080p | 2.3% | 2.3% | 2.3% | 2.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.9% | 6.9% | 5.5% | 9.2% |
| YUV, text & graphics with motion,720p | 1.4% | 2.3% | 0.7% | 5.9% |
| YUV, mixed content, 1440p | 2.2% | 2.0% | 1.3% | 2.8% |
| YUV, mixed content, 1080p | 2.2% | 2.2% | 2.2% | 2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.7% | 4.9% | 2.8% | 7.2% |
| RGB, text & graphics with motion,720p | 0.4% | 1.2% | 0.3% | 3.7% |
| RGB, mixed content, 1440p | 0.4% | 0.5% | 0.1% | 0.8% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.7% | 5.0% | 2.5% | 7.5% |
| YUV, text & graphics with motion,720p | 0.4% | 1.4% | 0.2% | 4.7% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.8% |
| YUV, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.0% | 4.4% | 2.3% | 5.7% |
| RGB, text & graphics with motion,720p | 0.2% | 0.5% | 0.1% | 1.4% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.9% | 4.4% | 2.1% | 5.7% |
| YUV, text & graphics with motion,720p | 0.2% | 0.6% | 0.1% | 1.9% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 3 and Table 4 show the lossy and lossless simulation result respectively for combination test 5.1b of SCCE1.

Table 3. Lossy simulation result for test 5.1b.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -9.9% | -10.9% | -11.0% |
| RGB, text & graphics with motion,720p | -5.7% | -6.6% | -6.4% |
| RGB, mixed content, 1440p | -4.6% | -4.8% | -4.9% |
| RGB, mixed content, 1080p | -4.8% | -5.3% | -5.3% |
| RGB, Animation, 720p | -0.4% | -0.4% | -0.4% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -10.6% | -11.5% | -11.4% |
| YUV, text & graphics with motion,720p | -6.2% | -6.5% | -6.8% |
| YUV, mixed content, 1440p | -4.5% | -5.2% | -5.2% |
| YUV, mixed content, 1080p | -5.0% | -5.8% | -5.8% |
| YUV, Animation, 720p | -0.5% | -0.8% | -0.8% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 166% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.4% | -8.0% | -8.1% |
| RGB, text & graphics with motion,720p | -4.2% | -5.2% | -5.3% |
| RGB, mixed content, 1440p | -3.0% | -3.5% | -3.6% |
| RGB, mixed content, 1080p | -3.2% | -4.3% | -4.4% |
| RGB, Animation, 720p | -0.2% | -0.3% | -0.1% |
| RGB, camera captured, 1080p | 0.2% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -6.2% | -7.8% | -7.9% |
| YUV, text & graphics with motion,720p | -4.3% | -4.9% | -5.0% |
| YUV, mixed content, 1440p | -2.6% | -4.2% | -3.9% |
| YUV, mixed content, 1080p | -3.5% | -5.4% | -5.0% |
| YUV, Animation, 720p | -0.1% | -0.8% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.3% |
| Enc Time[%] | 116% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.4% | -6.8% | -6.8% |
| RGB, text & graphics with motion,720p | -2.2% | -3.2% | -3.1% |
| RGB, mixed content, 1440p | -2.2% | -3.0% | -2.7% |
| RGB, mixed content, 1080p | -2.0% | -3.5% | -3.8% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.2% | 0.1% | 0.2% |
| YUV, text & graphics with motion, 1080p | -4.9% | -6.6% | -6.4% |
| YUV, text & graphics with motion,720p | -1.9% | -2.8% | -3.5% |
| YUV, mixed content, 1440p | -1.5% | -3.6% | -3.1% |
| YUV, mixed content, 1080p | -1.8% | -5.7% | -4.0% |
| YUV, Animation, 720p | 0.1% | -0.5% | -0.3% |
| YUV, camera captured, 1080p | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 101% |  |  |

Table 4. Lossless simulation result for test 5.1b.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.1% | 7.1% | 5.9% | 9.3% |
| RGB, text & graphics with motion,720p | 1.4% | 2.0% | 0.9% | 4.8% |
| RGB, mixed content, 1440p | 2.1% | 2.0% | 1.1% | 2.8% |
| RGB, mixed content, 1080p | 2.3% | 2.3% | 2.3% | 2.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.2% | 7.1% | 5.8% | 9.7% |
| YUV, text & graphics with motion,720p | 1.5% | 2.4% | 0.7% | 6.1% |
| YUV, mixed content, 1440p | 2.2% | 2.1% | 1.3% | 2.8% |
| YUV, mixed content, 1080p | 2.2% | 2.2% | 2.2% | 2.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 196% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 4.8% | 5.1% | 2.9% | 7.5% |
| RGB, text & graphics with motion,720p | 0.4% | 1.2% | 0.3% | 3.8% |
| RGB, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.8% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.9% | 5.2% | 2.7% | 7.8% |
| YUV, text & graphics with motion,720p | 0.4% | 1.5% | 0.2% | 4.9% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.1% | 0.8% |
| YUV, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.2% | 4.6% | 2.4% | 5.9% |
| RGB, text & graphics with motion,720p | 0.3% | 0.6% | 0.1% | 1.5% |
| RGB, mixed content, 1440p | 0.2% | 0.3% | 0.0% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.0% | 4.6% | 2.2% | 6.0% |
| YUV, text & graphics with motion,720p | 0.2% | 0.7% | 0.1% | 2.0% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

# Conclusion

The performance of intra-block copy configured as an inter-prediction mode with merge mode and with and without AMP was tested in combination tests 5.1a and 5.1b of SCCE1.  The results provided by the proponent is verified in this cross-check report.

# References

- J. Sole, S. Liu, “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”, JCTVC-Q1121, Valencia, ES, March 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Valencia, ES, March 2014.

- X. Xu, S. Liu, S. Lei, “SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4”, JCTVC-R0191, Sapporo, JP, June 2014.