| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0248 |
| --- | --- |

| Title: | SCCE1: Cross-check results of Test 3.4 (JCTVC-R0061) |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | M. Xu, F. Duanmu, Z. Ma, W. Wang, H. Yu 2330 Central Expy, Santa Clara, CA 95050 | Tel: Email: |  | +1 408 330 5142 m.xu@huawei.com |
| Source: | Huawei R&D USA |  |  |  |

_____________________________

# Algorithm Description

JCTVC-R0061 [1] proposes to use spatially adjacent and IntraBC coded blocks BVs to predict the current PU in IntraBC mode. Two candidate BV lists are constructed and when one or two of the lists is empty, some preset default vectors will be used. Cross-check results perfectly match the data provided by the proponent.

The proposed technique uses spatial AMVP for IntraBC BV prediction. Two candidate BV lists are constructed. A flag is used to signal which of the predictors to be used. When there is only one unique candidate BV available, (-2*CU_width, 0) will be used for the second candidate. If both candidates are not available, (-2*CU_width, 0) and (-CU_width, 0) will be used.

The proposed method is integrated on HM-14.0+RExt-7.0+SCM1.0 [1].	The simulations are performed under the updated CTC [2].

Table 1: Results for lossy configuration (w/ full frame IBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.8% | -3.9% | -3.8% |
| RGB, text & graphics with motion,720p | -1.9% | -2.0% | -2.0% |
| RGB, mixed content, 1440p | -1.4% | -1.4% | -1.4% |
| RGB, mixed content, 1080p | -1.8% | -1.7% | -1.8% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.5% | -4.6% | -4.6% |
| YUV, text & graphics with motion,720p | -2.4% | -2.5% | -2.5% |
| YUV, mixed content, 1440p | -1.7% | -1.7% | -1.7% |
| YUV, mixed content, 1080p | -2.1% | -2.0% | -2.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.6% | -1.6% |
| RGB, text & graphics with motion,720p | -1.2% | -1.3% | -1.2% |
| RGB, mixed content, 1440p | -0.8% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -1.0% | -1.0% | -1.0% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.7% | -1.7% | -1.8% |
| YUV, text & graphics with motion,720p | -1.5% | -1.4% | -1.6% |
| YUV, mixed content, 1440p | -0.9% | -0.9% | -1.0% |
| YUV, mixed content, 1080p | -1.3% | -1.4% | -1.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.4% | -0.6% | -0.4% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.9% | -0.8% |
| YUV, text & graphics with motion,720p | -0.4% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -0.5% |
| YUV, mixed content, 1080p | -0.1% | -1.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2: Results for lossless configuration (w/ full frame IBC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.9% | 1.9% | 0.9% | 2.8% |
| RGB, text & graphics with motion,720p | 0.2% | 0.5% | 0.0% | 1.4% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.2% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 2.1% | 1.0% | 2.8% |
| YUV, text & graphics with motion,720p | 0.3% | 0.5% | 0.1% | 1.6% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.2% | 0.3% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.2% | 0.6% | 2.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.1% | 1.1% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 1.2% | 0.8% | 2.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.4% | 0.1% | 1.2% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.4% | 1.3% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.5% | 1.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

Detailed data is enclosed as the excel files where additional simulations using 2CTU IBC are also provided.

# Reference

[1] HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-14.0%2BRExt-7.0%2BSCM-1.0

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Apr. 2014

[3] X. Xu, S. Liu, “SCCE1 Test 3.4: IntraBC BV prediction”, JCTVC-R0061, Sapporo, Japan, July 2014