| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0249 |
| --- | --- |

| Title: | SCCE2: Cross-check of Line-based Intra Copy with 2-D BV and with Constrained PU-level Splitting (Test C) (JCTVC-R0131) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Xu, F. Duanmu, Z. Ma, W. Wang, H. Yu | Tel: Email: | +1 408 330 5142 m.xu@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Description

This contribution reports the cross-check of SCCE2 Test C with Line-based intra block copy using 2-D Block Vector and Constrained PU-level splitting. This mode extends the basic processing unit for intra copy from blocks to lines, which divides a prediction block horizontally or vertically into 1-pixel-wide lines and then operates intra copying line-by-line from part of the reconstructed pixels, to the left or on the top of the lines, in the current frame.

The following experiments were conducted using the HM-14.0+RExt-7.0+SCM-1.0 software and the screen content common test conditions [1] to collect the BD-rate, encoding and decoding time of the proposed techniques in the All-Intra, Random Access, Low Delay B configurations. Results are summarized in Table 1 and Table 2. Results perfectly match the data provided by the proponent.

Table 1. BD-rate reduction and processing time ratio of Test C in lossy coding.

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U | R/V |  |
| RGB, text & graphics with motion, 1080p | -7.0% |  | -7.0% | -6.9% |  |
| RGB, text & graphics with motion,720p | -3.2% |  | -3.2% | -3.2% |  |
| RGB, mixed content, 1440p | -2.8% |  | -2.7% | -2.8% |  |
| RGB, mixed content, 1080p | -3.7% |  | -3.8% | -3.8% |  |
| RGB, Animation, 720p | -0.1% |  | -0.1% | -0.1% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -6.6% |  | -6.5% | -6.5% |  |
| YUV, text & graphics with motion,720p | -2.5% |  | -3.0% | -3.3% |  |
| YUV, mixed content, 1440p | -2.2% |  | -2.7% | -2.9% |  |
| YUV, mixed content, 1080p | -3.2% |  | -3.9% | -3.7% |  |
| YUV, Animation, 720p | 0.0% |  | -0.1% | -0.1% |  |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% |  |
| Enc Time[%] | 114% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U | R/V |  |
| RGB, text & graphics with motion, 1080p | -4.1% |  | -4.0% | -4.0% |  |
| RGB, text & graphics with motion,720p | -2.3% |  | -2.2% | -2.4% |  |
| RGB, mixed content, 1440p | -2.1% |  | -1.9% | -1.9% |  |
| RGB, mixed content, 1080p | -2.6% |  | -2.7% | -2.7% |  |
| RGB, Animation, 720p | -0.1% |  | -0.1% | -0.1% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -3.6% |  | -3.9% | -3.9% |  |
| YUV, text & graphics with motion,720p | -2.1% |  | -2.6% | -2.6% |  |
| YUV, mixed content, 1440p | -1.3% |  | -2.4% | -2.1% |  |
| YUV, mixed content, 1080p | -2.4% |  | -3.1% | -3.0% |  |
| YUV, Animation, 720p | -0.1% |  | -0.2% | 0.0% |  |
| YUV, camera captured, 1080p | 0.0% |  | -0.1% | 0.1% |  |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U | R/V |  |
| RGB, text & graphics with motion, 1080p | -2.5% |  | -2.4% | -2.3% |  |
| RGB, text & graphics with motion,720p | -1.5% |  | -1.4% | -1.5% |  |
| RGB, mixed content, 1440p | -1.1% |  | -1.3% | -1.1% |  |
| RGB, mixed content, 1080p | -1.3% |  | -1.3% | -1.3% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -2.2% |  | -2.4% | -2.5% |  |
| YUV, text & graphics with motion,720p | -0.9% |  | -1.1% | -1.8% |  |
| YUV, mixed content, 1440p | -0.7% |  | -1.7% | -1.8% |  |
| YUV, mixed content, 1080p | -0.5% |  | -2.0% | -0.4% |  |
| YUV, Animation, 720p | -0.1% |  | -0.5% | -0.2% |  |
| YUV, camera captured, 1080p | 0.0% |  | 0.1% | -0.1% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |

Table 2. BD-rate reduction and processing time ratio of Test C in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.6% | 6.5% | 4.3% | 10.2% |
| RGB, text & graphics with motion,720p | 1.5% | 1.7% | 0.5% | 3.2% |
| RGB, mixed content, 1440p | 0.9% | 1.0% | 0.6% | 1.4% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.6% | 6.5% | 4.3% | 9.9% |
| YUV, text & graphics with motion,720p | 1.6% | 2.1% | 0.4% | 4.6% |
| YUV, mixed content, 1440p | 0.9% | 1.0% | 0.5% | 1.4% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.9% | 4.0% | 1.7% | 7.4% |
| RGB, text & graphics with motion,720p | 0.4% | 1.0% | 0.3% | 2.7% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.9% | 4.1% | 2.0% | 7.5% |
| YUV, text & graphics with motion,720p | 0.4% | 1.3% | 0.3% | 3.9% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.6% | 3.0% | 1.3% | 4.9% |
| RGB, text & graphics with motion,720p | 0.3% | 0.7% | 0.2% | 1.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.5% | 3.1% | 1.6% | 5.1% |
| YUV, text & graphics with motion,720p | 0.3% | 0.9% | 0.2% | 2.6% |
| YUV, mixed content, 1440p | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Conditions for Screen Content Coding Tests,” JCTVC-Q1015, Valencia, Mar. 2014.