| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0251 |
| --- | --- |

| Title: | SCCE3: Cross-check results of Test B.1 (JCTVC-R0121) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Xu, F. Duanmu, Z. Ma, W. Wang, H. Yu 2330 Central Expy, Santa Clara, CA 95050 | Tel: Email: | +1 408 330 5142 m.xu@huawei.com |
| Source: | Huawei R&D USA. |  |  |

_____________________________

# Abstract

This information document reports the cross-check results of SCCE 3 Test B.1. Results perfectly match the data provided by the proponent.

# Introduction

In SCCE3 Test B.1, the line-based palette color index map coding method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

The color indices of all samples in the CU are coded line by line. For each sample line, an “index line mode” is signaled to indicate the coding mode. The index line mode includes vertical mode, horizontal mode, and normal mode:

- Vertical mode: current line is copied from the above line.

- Horizontal mode: signal color index itself.

- Normal mode: the color indices of the line are signaled sample by sample with predictions.

# Experimental results

The tests were conducted under the test conditions defined in SCCE3 [1]. Two kinds of configurations are tested, which are the IntraBC with 2-CTU search range and the IntraBC with full-frame search range. Table 1 - 4 shows the lossy and the lossless results of the proposed method.

Table 1 Lossless results vs. SCM 1.0 using 2CTU IBC

Table 2 Lossy results vs. SCM 1.0 using 2CTU IBC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -19.1% | -19.4% | -19.2% |
| RGB, text & graphics with motion,720p | -10.3% | -9.7% | -10.0% |
| RGB, mixed content, 1440p | -2.0% | -1.7% | -1.7% |
| RGB, mixed content, 1080p | -4.0% | -4.3% | -3.8% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -18.6% | -21.0% | -20.0% |
| YUV, text & graphics with motion,720p | -7.4% | -10.7% | -13.0% |
| YUV, mixed content, 1440p | -1.5% | -4.3% | -4.1% |
| YUV, mixed content, 1080p | -3.9% | -7.1% | -6.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.1% | -11.8% | -11.9% |
| RGB, text & graphics with motion,720p | -10.0% | -9.1% | -9.6% |
| RGB, mixed content, 1440p | -2.2% | -1.7% | -1.7% |
| RGB, mixed content, 1080p | -3.6% | -4.1% | -3.4% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -11.2% | -14.4% | -13.9% |
| YUV, text & graphics with motion,720p | -7.3% | -10.7% | -13.0% |
| YUV, mixed content, 1440p | -1.5% | -5.4% | -5.1% |
| YUV, mixed content, 1080p | -3.5% | -8.1% | -7.9% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.1% | -7.9% | -7.9% |
| RGB, text & graphics with motion,720p | -7.7% | -7.2% | -7.6% |
| RGB, mixed content, 1440p | -2.3% | -1.9% | -1.8% |
| RGB, mixed content, 1080p | -3.0% | -3.9% | -3.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.0% | -9.5% | -9.0% |
| YUV, text & graphics with motion,720p | -5.3% | -8.3% | -10.3% |
| YUV, mixed content, 1440p | -1.7% | -6.0% | -5.4% |
| YUV, mixed content, 1080p | -2.8% | -6.9% | -6.8% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 108% |  |  |

Table 3 Lossless results vs. SCM 1.0 using Full Frame IBC

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 9.7% | 10.0% | 4.0% | 15.3% |
| RGB, text & graphics with motion,720p | 9.2% | 7.2% | 0.3% | 20.1% |
| RGB, mixed content, 1440p | 0.6% | 0.6% | 0.4% | 0.8% |
| RGB, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 12.0% | 12.0% | 3.2% | 20.1% |
| YUV, text & graphics with motion,720p | 8.0% | 7.2% | 0.3% | 17.0% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.2% | 0.8% |
| YUV, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.6% | 5.4% | 2.3% | 11.2% |
| RGB, text & graphics with motion,720p | 1.3% | 3.6% | 0.1% | 7.6% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.4% | 6.5% | 1.8% | 14.6% |
| YUV, text & graphics with motion,720p | 1.1% | 4.1% | 0.1% | 9.3% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.2% | 3.8% | 1.7% | 7.7% |
| RGB, text & graphics with motion,720p | 0.8% | 2.6% | 0.1% | 5.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.8% | 4.4% | 1.6% | 9.9% |
| YUV, text & graphics with motion,720p | 0.6% | 2.8% | 0.0% | 6.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 106% |  |  |  |

Table 4 Lossy results vs. SCM 1.0 using Full Frame IBC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -14.0% | -14.1% | -13.9% |
| RGB, text & graphics with motion,720p | -7.6% | -6.8% | -7.1% |
| RGB, mixed content, 1440p | -1.6% | -1.5% | -1.4% |
| RGB, mixed content, 1080p | -3.0% | -3.1% | -2.7% |
| RGB, Animation, 720p | 0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -14.5% | -16.2% | -15.1% |
| YUV, text & graphics with motion,720p | -6.2% | -9.1% | -11.2% |
| YUV, mixed content, 1440p | -1.3% | -3.4% | -3.3% |
| YUV, mixed content, 1080p | -3.6% | -5.2% | -5.0% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.3% | -8.6% | -8.7% |
| RGB, text & graphics with motion,720p | -7.5% | -6.4% | -6.9% |
| RGB, mixed content, 1440p | -1.8% | -1.4% | -1.3% |
| RGB, mixed content, 1080p | -2.9% | -3.1% | -2.7% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -8.5% | -11.1% | -10.6% |
| YUV, text & graphics with motion,720p | -6.1% | -8.6% | -10.8% |
| YUV, mixed content, 1440p | -1.3% | -4.6% | -4.2% |
| YUV, mixed content, 1080p | -3.1% | -6.4% | -5.9% |
| YUV, Animation, 720p | 0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.6% | -6.1% | -6.1% |
| RGB, text & graphics with motion,720p | -5.6% | -4.9% | -5.3% |
| RGB, mixed content, 1440p | -2.0% | -1.9% | -1.6% |
| RGB, mixed content, 1080p | -2.5% | -2.8% | -2.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.8% | -7.6% | -7.3% |
| YUV, text & graphics with motion,720p | -4.2% | -6.5% | -9.0% |
| YUV, mixed content, 1440p | -1.4% | -5.0% | -4.5% |
| YUV, mixed content, 1080p | -1.7% | -5.8% | -4.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 106% |  |  |

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, Mar. 2014.