| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0254 |
| --- | --- |

| Title: | SCCE5: Cross-check result of Test 3.2.2 (JCTVC-R0099) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Xu, F. Duanmu, Z. Ma, W. Wang, H. Yu 2330 Central Expy, Santa Clara, CA 95050 | Tel: Email: | +1 408 330 5142 m.xu@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Abstract

This document crosschecks the proposal in SCCE5 test 3.2.2, where adaptive color space is designed to improve the coding efficiency of RGB (GBR) video. Simulation results match the data shared by the proponent.

# Introduction

- Lossy coding: besides GBR, coding in YCoCg, RGB, and BGR spaces are enabled for intra predicted CU for lossy coding.

- Lossless coding, only reordering (e.g. RGB, BGR) is allowed.

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [1] is followed. The summarized All Intra experimental results (full picture Intra BC) are shown in Table 1~6. More detailed experimental results (including 2CTU Intra BC results) could be found in the attached spreadsheets.

Table 1 Experimental result for lossy coding (both reorder and YCoCg, full picture Intra BC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.6% | -3.3% | -3.4% |
| RGB, text & graphics with motion,720p | -8.4% | -3.7% | -6.0% |
| RGB, mixed content, 1440p | -13.0% | -7.6% | -7.7% |
| RGB, mixed content, 1080p | -7.8% | -4.3% | -4.7% |
| RGB, Animation, 720p | -20.0% | -17.9% | -15.6% |
| RGB, camera captured, 1080p | -17.6% | -5.2% | -10.3% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 138% |  |  |
| Dec Time[%] | 111% |  |  |

Table 2 Experimental results for lossless coding (both reorder and YCoCg both reorder and YCoCg, full picture Intra BC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.2% | 1.1% |
| RGB, text & graphics with motion,720p | 0.7% | 0.8% | 0.5% | 1.2% |
| RGB, mixed content, 1440p | 0.6% | 0.5% | 0.4% | 0.7% |
| RGB, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, Animation, 720p | 3.2% | 3.2% | 3.2% | 3.2% |
| RGB, camera captured, 1080p | 0.2% | 0.2% | 0.2% | 0.3% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 119% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3 Experimental result for lossy coding (reorder both reorder and YCoCg, full picture Intra BC)

Table 4 Experimental results for lossless coding (reorder both reorder and YCoCg, full picture Intra BC)

Table 5 Experimental result for lossy coding (YCoCg both reorder and YCoCg, full picture Intra BC)

Table 6 Experimental results for lossless coding (YCoCg both reorder and YCoCg, full picture Intra BC)

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.