| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0257 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0060 |
| --- | --- |

| Title: | Language indication for Overlay information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The contribution proposes to add an optional language indication to the Overlay information SEI message. In case multiple overlays are available with the same content in different languages, a set-top-box can make a selection, based on the user's language setting. It is proposed to code the language according to ISO 639-3.

# Introduction

The overlay information SEI message contains the overlay_name and overlay_element_name syntax elements as UTF-8 strings. Those can be shown in the user interface as menu options for overlay selection. Also the overlay itself may contain rendered text. For multi-language broadcast, a language indication would be helpful for the user to select the proper overlay.

| overlay_info( payloadSize ) { | Descriptor |
| --- | --- |
| overlay_info_cancel_flag | u(1) |
| if( !overlay_info_cancel_flag ) { |  |
| overlay_content_aux_id_minus128 | ue(v) |
| overlay_label_aux_id_minus128 | ue(v) |
| overlay_alpha_aux_id_minus128 | ue(v) |
| num_overlays_minus1 | ue(v) |
| for( i = 0; i <= num_overlays_minus1; i++ ) { |  |
| overlay_idx[ i ] | ue(v) |
| overlay_language_present_flag[ i ] | u(1) |
| if( overlay_language_present_flag[ i ]) |  |
| overlay_language[ i ] | st(v) |
| overlay_name[ i ] | st(v) |
| overlay_content_layer_id[ i ] | u(6) |
| overlay_label_present_flag[ i ] | u(1) |
| if( overlay_label_present_flag[ i ] ) |  |
| overlay_label_layer_id[ i ] | u(6) |
| overlay_alpha_present_flag[ i ] | u(1) |
| if( overlay_alpha_present_flag[ i ] ) |  |
| overlay_alpha_layer_id[ i ] | u(6) |
| if( overlay_label_present_flag[ i ] ) { |  |
| num_overlay_elements_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_overlay_elements_minus1[ i ]; j++ ) { |  |
| overlay_element_name[ i ][ j ] | st(v) |
| overlay_element_label_min[ i ][ j ] | u(v) |
| overlay_element_label_max[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| overlay_info_persistence_flag | u(1) |
| } |  |
| } |  |

overlay_language_present_flag[ i ] equal to 1 indicates that overlay_language is present. overlay_language_present_flag equal to 0 indicates that overlay_language is not present and that the language of the overlay is unspecified.

overlay_language[ i ] indicates the language of the i-th overlay indicates the name of the i-th overlay. The language is coded according to ISO 639-3. The length of the syntax element overlay_language[ i ] shall be equal to 4 bytes.

# References

- ISO 639-3:2007, Codes for the representation of names of languages — Part 3: Alpha-3 code for comprehensive coverage of languages

- JCTVC-Q1008, High efficiency video coding (HEVC) scalable extension Draft 6, J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang (Editors)

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).