| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0262 |
| --- | --- |

| Title: | Cross check of ‘Non-SCCE1: Intra block copy deblocking’ (JCTVC-R0189) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of non-SCCE1 proposal JCTVC-R0189 on intra-block copy deblocking by Qualcomm.  The cross-check confirms the result reported by the proponent.

# Cross check procedure

The source code for JCTVC-R0189 [1] was provided by Qualcomm. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code provided by Qualcomm was studied and the following observations were made:

- The deblocking filter boundary strength setting is set to either 1 or 0, in accordance with inter-prediction modes.

- If the block vectors of two adjacent intra-block copy blocks are equal, then the boundary strength is set to 0, otherwise it is set to 1.

These observations are consistent with the description in [1].

# Coding performance measurement

Table 1 shows the simulation results for JCTVC-R0189 under lossy test conditions.

Table 1. Lossy simulation result.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | 0.0% | -0.7% | -0.6% |
| RGB, mixed content, 1440p | 0.0% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | 0.0% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.4% |
| YUV, mixed content, 1080p | 0.0% | -0.8% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -1.7% | -1.6% |
| RGB, text & graphics with motion,720p | -0.1% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.1% | -1.0% | -1.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | 0.0% | -0.7% | -0.8% |
| YUV, mixed content, 1440p | 0.0% | -1.1% | -1.0% |
| YUV, mixed content, 1080p | -0.1% | -1.7% | -1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -1.8% | -1.6% |
| RGB, text & graphics with motion,720p | -0.2% | -1.1% | -1.1% |
| RGB, mixed content, 1440p | -0.3% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -0.3% | -1.6% | -1.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -1.6% | -1.3% |
| YUV, text & graphics with motion,720p | 0.0% | -0.7% | -1.0% |
| YUV, mixed content, 1440p | -0.1% | -1.2% | -1.2% |
| YUV, mixed content, 1080p | -0.3% | -3.5% | -3.4% |
| YUV, Animation, 720p | 0.0% | -0.5% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

The boundary strength setting of the deblocking filter for intra block copy blocks is modified to treat block vectors as motion vectors in JCTVC-R0189.  The results provided by the proponents are verified in this cross-check report.

# References

- C. Pang, Y. Chen, J. Sole, M. Karczewicz, “Non-SCCE1: Intra block copy deblocking”, Joint Collaborative Team on Video Coding, JCTVC-R0189, Sapporo, JP, June 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, Joint Collaborative Team on Video Coding, JCTVC-Q1015, Valencia, ES, March 2014.