| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0263 |
| --- | --- |

| Title: | Cross-check of 'Non-SCCE3: Encoder modification of palette mode for escape pixels' (JCTVC-R0075) by ITRI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of non-SCCE3 proposal JCTVC-R0075 on palette mode escape code encoder modification by ITRI and CU-level escape flag presence signalling.  The cross-check confirms the result reported by the proponent.

# Cross check procedure

The source code for JCTVC-R0075 [1] was provided by ITRI. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code provided by ITRI was studied and the following observations were made:

- When preparing the palette, if a given colour is used twice or once, the colour is not added to the palette and is instead escape coded (i.e. even if unused entries are available in the palette).

- The CU-based escape presence flag of JCTVC-R0051 is also incorporated into this contribution.  In the encoder search, this flag is disabled initially and can be enabled in derivePLT().

The observations above are consistent with the description in [1].

# Coding performance measurement

Table 1 shows the simulation results for JCTVC-R0189 under lossy test conditions, with full-frame intra-block copy enabled.  The anchor is the SCCE3 palette base software.

Table 1. Lossy simulation result.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.1% | -1.2% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.5% | -0.7% |
| YUV, mixed content, 1080p | -0.5% | -1.2% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.3% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.7% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.6% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | -0.4% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.3% | -1.4% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

The results provided by the proponents are verified in this cross-check report.

# References

- Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung, “Non-SCCE3: Encoder modification of palette mode for escape pixels”, Joint Collaborative Team on Video Coding, JCTVC-R0075, Sapporo, JP, June 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, Joint Collaborative Team on Video Coding, JCTVC-Q1015, Valencia, ES, March 2014.