| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 29 June –11 July. 2014 | Document: JCTVC-R0266 |
| --- | --- |

| Title: | Non-SCCE3: Palette Table Prediction by the Nearest Palette Table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chao-Hsiung Hung 2Chun-Lung Lin 3Yao-Jen Chang 4Ching-Chieh Lin 5Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | 1+886-3-5917964 1chhung@itri.org.tw 2Chunlung@itri.org.tw 3britpablo@itri.org.tw 4JackLin@itri.org.tw 5SunriseJSTu@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

Palette coding [1][2] provides good coding performance for SCC (screen content coding) test sequences. It analyzes major colors of a CU. Then it creates a palette table and presents each pixel by corresponding color index. Palette tables from neighboring CUs may have many consistent major colors. We propose a prediction algorithm to predict major color table by the nearest ones. Compared with SCCE3 reference software (SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047 [7]) under All intra, lossy, and 2 left CTUs IBC search, the BD rates are -1.2% and -1.1% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” respectively. For All intra, lossy, and full frame IBC search condition, BD rates of above two test sequence groups are all -0.8%.

# Introductions

SCC test sequences are usually composed of few major colors. Palette coding considers this characteristic and compresses them efficiently. It presents a CU by a palette table and a color index map. Coding palette table consumes many bits. Predicting current palette table by previous decoded one improves coding performance [3]. In this document, we propose to predict current palette table by the nearest ones.

# Proposed Algorithm

For a palette-coded CU, it stores its palette table and center location. These two information will be propagated together by other non-palette-coded CUs. For a non-palette-coded CU, it propagates above two information stored in the nearest CU.

For current CU, the nearest CU is selected among left and above CUs. The center coordinate of current CU is (XC, YC). The center coordinates stored in left CU and above CU are (XL, YL) and (XA, YA). If left CU is palette-coded, (XL, YL) is its center information. Otherwise, (XL, YL) is propagated from the nearest CU of left CU. (XA, YA) stored in above CU is processed similarly. Left distance and above distance are abs(XC - XL) + abs(YC - YL) and abs(XC – XA) + abs(YC – YA). The nearest CU is the one resulting in minimum distance.

For a palette-coded CU, we select palette table stored in the nearest CU as palette table predictor.

# Experiment results

The proposed method is simulated based on SCM 1.0 and SCCE3 reference software (SCM-1.0 + JCTVC-Q0094[4] + bug-fixes of JCTVC-Q0047[5]) under the common test condition [6][7]. Table 1 and Table 2 show the lossless and lossy results.

Table 1. Anchor is SCM 1.0, IBC searches 2 left CTUs, lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -14.2% | -14.4% | -14.3% |
| RGB, text & graphics with motion,720p | -7.9% | -7.3% | -7.6% |
| RGB, mixed content, 1440p | -1.5% | -1.4% | -1.3% |
| RGB, mixed content, 1080p | -3.1% | -3.4% | -2.9% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -13.4% | -15.5% | -14.7% |
| YUV, text & graphics with motion,720p | -5.7% | -8.2% | -10.2% |
| YUV, mixed content, 1440p | -1.1% | -3.3% | -3.0% |
| YUV, mixed content, 1080p | -2.7% | -5.1% | -4.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 135% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.5% | -9.1% | -9.1% |
| RGB, text & graphics with motion,720p | -7.7% | -6.9% | -7.3% |
| RGB, mixed content, 1440p | -1.8% | -1.5% | -1.5% |
| RGB, mixed content, 1080p | -2.9% | -3.3% | -2.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -8.6% | -11.2% | -10.7% |
| YUV, text & graphics with motion,720p | -5.8% | -8.3% | -10.3% |
| YUV, mixed content, 1440p | -1.3% | -4.3% | -4.0% |
| YUV, mixed content, 1080p | -2.8% | -6.5% | -6.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.3% | -5.9% | -5.9% |
| RGB, text & graphics with motion,720p | -5.8% | -5.5% | -5.8% |
| RGB, mixed content, 1440p | -1.8% | -1.5% | -1.4% |
| RGB, mixed content, 1080p | -2.7% | -3.3% | -2.6% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.5% | -7.2% | -6.8% |
| YUV, text & graphics with motion,720p | -4.1% | -6.2% | -8.1% |
| YUV, mixed content, 1440p | -1.5% | -4.7% | -4.0% |
| YUV, mixed content, 1080p | -2.0% | -5.2% | -4.9% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 109% |  |  |

Table 2. Anchor is SCM 1.0, IBC searches full frame, lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.1% | -11.1% | -11.0% |
| RGB, text & graphics with motion,720p | -6.3% | -5.6% | -6.0% |
| RGB, mixed content, 1440p | -1.2% | -1.1% | -1.0% |
| RGB, mixed content, 1080p | -2.3% | -2.4% | -2.2% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -11.3% | -12.8% | -11.9% |
| YUV, text & graphics with motion,720p | -5.3% | -7.4% | -9.5% |
| YUV, mixed content, 1440p | -0.9% | -2.5% | -2.3% |
| YUV, mixed content, 1080p | -2.4% | -3.5% | -3.3% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 134% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.7% | -6.9% | -7.1% |
| RGB, text & graphics with motion,720p | -6.2% | -5.2% | -5.7% |
| RGB, mixed content, 1440p | -1.4% | -1.1% | -1.0% |
| RGB, mixed content, 1080p | -2.2% | -2.4% | -2.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -7.0% | -9.1% | -8.7% |
| YUV, text & graphics with motion,720p | -5.2% | -7.3% | -9.1% |
| YUV, mixed content, 1440p | -1.1% | -3.6% | -3.1% |
| YUV, mixed content, 1080p | -2.4% | -5.0% | -4.6% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.2% | -4.7% | -4.7% |
| RGB, text & graphics with motion,720p | -4.3% | -3.9% | -4.2% |
| RGB, mixed content, 1440p | -1.5% | -1.4% | -1.2% |
| RGB, mixed content, 1080p | -2.0% | -2.4% | -2.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -3.7% | -5.9% | -5.7% |
| YUV, text & graphics with motion,720p | -3.3% | -5.2% | -6.7% |
| YUV, mixed content, 1440p | -1.1% | -4.0% | -3.4% |
| YUV, mixed content, 1080p | -1.5% | -4.5% | -2.9% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 106% |  |  |

Table 3. Anchor is SCCE3, IBC searches 2 left CTUs, lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.1% | -1.1% |
| YUV, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 112% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | -0.7% | -0.7% | -0.8% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.3% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.2% | -0.9% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 103% |  |  |

Table 4. Anchor is SCCE3, IBC searches full frame, lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -0.8% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.5% | -0.5% | -0.6% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.3% |
| RGB, text & graphics with motion,720p | -0.3% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.4% | -0.8% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 142% |  |  |
| Dec Time[%] | 173% |  |  |

‘

Table 5. Anchor is SCM 1.0, IBC searches 2 left CTUs, lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 13.8% | 13.1% | 4.6% | 23.9% |
| RGB, text & graphics with motion,720p | 9.9% | 9.7% | 0.2% | 20.3% |
| RGB, mixed content, 1440p | 1.6% | 1.5% | 1.0% | 1.9% |
| RGB, mixed content, 1080p | 1.1% | 1.1% | 1.1% | 1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 17.6% | 16.1% | 3.7% | 31.3% |
| YUV, text & graphics with motion,720p | 9.0% | 10.3% | 0.2% | 23.5% |
| YUV, mixed content, 1440p | 0.6% | 0.7% | 0.3% | 1.2% |
| YUV, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |  |
| Dec Time[%] | 123% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.1% | 7.5% | 2.3% | 17.7% |
| RGB, text & graphics with motion,720p | 1.4% | 5.5% | 0.1% | 13.9% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.1% | 9.7% | 2.2% | 23.7% |
| YUV, text & graphics with motion,720p | 1.3% | 6.7% | 0.1% | 20.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 106% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.4% | 5.0% | 1.3% | 11.3% |
| RGB, text & graphics with motion,720p | 0.8% | 3.4% | 0.1% | 8.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.1% | 6.5% | 1.9% | 15.8% |
| YUV, text & graphics with motion,720p | 0.7% | 4.0% | 0.0% | 11.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 107% |  |  |  |

Table 6. Anchor is SCM 1.0, IBC searches full frame, lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 8.3% | 8.5% | 3.5% | 13.5% |
| RGB, text & graphics with motion,720p | 8.7% | 6.8% | 0.2% | 19.4% |
| RGB, mixed content, 1440p | 0.5% | 0.5% | 0.4% | 0.6% |
| RGB, mixed content, 1080p | 0.5% | 0.5% | 0.5% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.2% | 10.2% | 2.7% | 17.8% |
| YUV, text & graphics with motion,720p | 7.3% | 6.6% | 0.1% | 15.8% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.6% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |  |
| Dec Time[%] | 124% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.2% | 4.4% | 1.8% | 9.4% |
| RGB, text & graphics with motion,720p | 1.2% | 3.3% | 0.1% | 7.5% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 5.5% | 1.5% | 12.6% |
| YUV, text & graphics with motion,720p | 1.0% | 3.7% | 0.0% | 8.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.8% | 2.9% | 1.0% | 5.9% |
| RGB, text & graphics with motion,720p | 0.7% | 2.3% | 0.1% | 5.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.5% | 3.6% | 1.3% | 8.2% |
| YUV, text & graphics with motion,720p | 0.6% | 2.5% | 0.0% | 6.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

Table 7. Anchor is SCCE3, IBC searches 2 left CTUs, lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.0% | 0.9% | 0.3% | 2.0% |
| RGB, text & graphics with motion,720p | 0.6% | 0.7% | 0.0% | 1.6% |
| RGB, mixed content, 1440p | 0.2% | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.1% | 0.3% | 2.3% |
| YUV, text & graphics with motion,720p | 0.7% | 0.8% | 0.0% | 1.7% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 108% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.2% | 1.5% |
| RGB, text & graphics with motion,720p | 0.1% | 0.4% | 0.0% | 1.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.7% | 0.2% | 1.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.5% | 0.0% | 1.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 1.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.2% | 0.9% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 8. Anchor is SCCE3, IBC searches full frame, lossless

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.7% | 0.7% | 0.2% | 1.6% |
| RGB, text & graphics with motion,720p | 0.5% | 0.6% | 0.0% | 1.2% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.2% | 1.8% |
| YUV, text & graphics with motion,720p | 0.6% | 0.7% | 0.0% | 1.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 109% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.4% | 0.1% | 1.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.8% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.5% | 0.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.4% | 0.0% | 1.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.3% | 0.1% | 0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.3% | 0.1% | 0.7% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusion

We propose a prediction algorithm to predict major color table by the nearest one. Compared with SCCE3 base software (SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047 [7]) under All intra, lossy, and 2 left CTUs IBC search, the BD rates are -1.2% and -1.1% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” respectively. For All intra, lossy, and full frame IBC search condition, BD rates of above two test sequence groups are all -0.8%. We recommend it for further studying.

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” 16th JCT-VC meeting, JCTVC-P0108, San Jose, US, Jan. 2014.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, F. Zou, “RCE4: Results of Test 2 on Palette Mode for Screen Content Coding,” 16th JCT-VC meeting, JCTVC-P0198, San Jose, US, Jan. 2014.

- G. Laroche, T. Poirier, C. Gisquet, and P. Onno C., “Non-RCE4: Palette Prediction for Palette mode”, 16th JCT-VC meeting, JCTVC-P0114, San Jose, US, Jan. 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “AHG10: Suggested Software for Palette Coding based on RExt6.0”, 17th JCT-VC meeting, JCTVC-Q0094, Valencia, SP, Mar. 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole, “AHG10: Simplification of Palette Based Coding”, 17th JCT-VC meeting, JCTVC-Q0047, Valencia, SP, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, 17th JCT-VC meeting, JCTVC-Q1015, Valencia, SP, Mar. 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” 17th JCT-VC meeting, JCTVC-Q1123, Valencia, SP, Mar. 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).