| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0268 |
| --- | --- |

| Title: | Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma, Wei Wang, Meng Xu, F. Duanmu, Haoping Yu 2330 Central Expy, Santa Clara CA 95050 United States | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Algorithm Description

This contribution reports the additional data to SCCE Test C.3 [2]. Technical difference is using larger search range for string match in this contribution, i.e., 3 left CTUs and upper 4 CTUs to the current CTU, while in SCCE Test C.3 only 1x4 CTUs are used for string search, as shown in Figure 1.

Figure 1 Illustration of extended search range of current contribution vs. JCTVC-R0144

Three tests are performed in this contribution, i.e.,

- Test A: SCCE Test C.3 using 2x4 Search Range + Full Frame Intra Block Copy (IBC)

- Test B: SCCE Test C.3 using 2x4 Search Range + IBC with 2x4 Search Range

- Test C: SCCE Test C.3 using 2x4 Search Range + IBC with 2x4 Search Range for Intra Frame while SCCE Test C.3 using 1x4 Search Range + IBC with 1x4 Search Range for Inter Frame

# Experimental results

Simulations are performed under the common test condition [1]. Full test results are provided in the attached Excel spreadsheets. Table 1 -Table 6 summarize the results with respect to SCM-1.0 anchor with 2CTU Intra Block Copy search ranges.

Table 1: lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test A

Table 2: lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test A

Table 3: lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test B

|  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) |  | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p | 32.0% |  | 30.5% | 23.0% | 42.8% |  |
| RGB, text & graphics with motion,720p | 17.1% |  | 17.0% | 1.8% | 28.4% |  |
| RGB, mixed content, 1440p | 6.7% |  | 6.9% | 6.2% | 7.6% |  |
| RGB, mixed content, 1080p | 7.6% |  | 7.6% | 7.6% | 7.6% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | 37.1% |  | 35.8% | 25.5% | 48.5% |  |
| YUV, text & graphics with motion,720p | 16.9% |  | 18.1% | 1.9% | 35.6% |  |
| YUV, mixed content, 1440p | 4.9% |  | 5.5% | 2.4% | 8.6% |  |
| YUV, mixed content, 1080p | 7.5% |  | 7.5% | 7.5% | 7.5% |  |
| YUV, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 203% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |  |
|  | Random Access |  |  |  |  |  |
|  | Bit-rate saving (Total) |  | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p | 18.9% |  | 20.1% | 7.6% | 33.0% |  |
| RGB, text & graphics with motion,720p | 3.1% |  | 9.2% | 0.8% | 23.5% |  |
| RGB, mixed content, 1440p | 1.4% |  | 1.4% | 0.7% | 2.2% |  |
| RGB, mixed content, 1080p | 1.4% |  | 1.4% | 1.4% | 1.4% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | 19.6% |  | 24.9% | 17.7% | 37.9% |  |
| YUV, text & graphics with motion,720p | 2.9% |  | 10.6% | 0.8% | 30.0% |  |
| YUV, mixed content, 1440p | 1.1% |  | 1.1% | 0.8% | 1.5% |  |
| YUV, mixed content, 1080p | 1.4% |  | 1.4% | 1.4% | 1.4% |  |
| YUV, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 114% |  |  |  |  |  |
| Dec Time[%] | 152% |  |  |  |  |  |
|  | Low Delay B |  |  |  |  |  |
|  | Bit-rate saving (Total) |  | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |  |
| RGB, text & graphics with motion, 1080p | 18.3% |  | 16.2% | 5.6% | 23.3% |  |
| RGB, text & graphics with motion,720p | 2.0% |  | 5.4% | 0.6% | 12.1% |  |
| RGB, mixed content, 1440p | 0.8% |  | 0.8% | 0.2% | 1.4% |  |
| RGB, mixed content, 1080p | 0.6% |  | 0.6% | 0.6% | 0.6% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, text & graphics with motion, 1080p | 18.2% |  | 20.3% | 15.2% | 27.3% |  |
| YUV, text & graphics with motion,720p | 1.7% |  | 6.1% | 0.5% | 16.1% |  |
| YUV, mixed content, 1440p | 0.8% |  | 0.8% | 0.2% | 1.5% |  |
| YUV, mixed content, 1080p | 0.6% |  | 0.6% | 0.6% | 0.6% |  |
| YUV, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% |  |
| Enc Time[%] | 112% |  |  |  |  |  |
| Dec Time[%] | 169% |  |  |  |  |  |

Table 4: Lossy coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test B.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -33.0% | -33.2% | -33.1% |
| RGB, text & graphics with motion,720p | -20.6% | -19.6% | -19.9% |
| RGB, mixed content, 1440p | -12.3% | -12.0% | -11.9% |
| RGB, mixed content, 1080p | -17.4% | -17.8% | -16.9% |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -33.4% | -35.4% | -34.3% |
| YUV, text & graphics with motion,720p | -18.7% | -21.6% | -23.8% |
| YUV, mixed content, 1440p | -13.2% | -16.4% | -16.3% |
| YUV, mixed content, 1080p | -18.0% | -21.5% | -21.7% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 190% |  |  |
| Dec Time[%] | 113% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -19.9% | -20.2% | -20.2% |
| RGB, text & graphics with motion,720p | -17.0% | -15.5% | -16.0% |
| RGB, mixed content, 1440p | -9.1% | -8.4% | -8.4% |
| RGB, mixed content, 1080p | -12.5% | -12.9% | -12.1% |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -19.7% | -22.8% | -22.0% |
| YUV, text & graphics with motion,720p | -14.9% | -18.1% | -20.4% |
| YUV, mixed content, 1440p | -9.4% | -14.4% | -13.9% |
| YUV, mixed content, 1080p | -13.5% | -17.8% | -18.6% |
| YUV, Animation, 720p | 0.1% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 179% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -13.8% | -14.2% | -14.3% |
| RGB, text & graphics with motion,720p | -10.5% | -9.6% | -9.9% |
| RGB, mixed content, 1440p | -6.2% | -5.6% | -5.5% |
| RGB, mixed content, 1080p | -6.5% | -6.7% | -5.9% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.1% | -15.2% | -14.6% |
| YUV, text & graphics with motion,720p | -8.0% | -10.6% | -12.7% |
| YUV, mixed content, 1440p | -6.0% | -11.2% | -11.0% |
| YUV, mixed content, 1080p | -6.2% | -9.6% | -10.7% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 207% |  |  |

Table 5: Lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test C

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 12.1% | 16.7% | 6.8% | 31.2% |
| RGB, text & graphics with motion,720p | 2.9% | 9.0% | 0.8% | 23.3% |
| RGB, mixed content, 1440p | 1.1% | 1.1% | 0.7% | 1.5% |
| RGB, mixed content, 1080p | 1.1% | 1.1% | 1.1% | 1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 13.2% | 21.6% | 11.6% | 36.2% |
| YUV, text & graphics with motion,720p | 2.7% | 10.4% | 0.8% | 29.9% |
| YUV, mixed content, 1440p | 0.8% | 0.8% | 0.8% | 0.8% |
| YUV, mixed content, 1080p | 1.2% | 1.2% | 1.2% | 1.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 156% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.0% | 11.9% | 4.8% | 19.5% |
| RGB, text & graphics with motion,720p | 1.7% | 5.1% | 0.5% | 11.5% |
| RGB, mixed content, 1440p | 0.4% | 0.5% | 0.2% | 0.7% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.4% | 16.3% | 10.7% | 23.8% |
| YUV, text & graphics with motion,720p | 1.4% | 5.8% | 0.5% | 15.4% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.2% | 0.6% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |  |
| Dec Time[%] | 171% |  |  |  |

Table 6. Lossy coding results with respect to SCM-1.0 for 2CTU Intra Block Copy of Test C

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -17.3% | -17.7% | -17.8% |
| RGB, text & graphics with motion,720p | -16.7% | -15.1% | -15.7% |
| RGB, mixed content, 1440p | -7.8% | -7.1% | -7.1% |
| RGB, mixed content, 1080p | -11.2% | -11.7% | -10.8% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -17.4% | -20.8% | -20.0% |
| YUV, text & graphics with motion,720p | -14.5% | -17.7% | -20.1% |
| YUV, mixed content, 1440p | -8.2% | -13.3% | -13.0% |
| YUV, mixed content, 1080p | -12.3% | -16.6% | -17.5% |
| YUV, Animation, 720p | 0.1% | -0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 179% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -10.6% | -11.2% | -11.2% |
| RGB, text & graphics with motion,720p | -9.8% | -8.8% | -9.2% |
| RGB, mixed content, 1440p | -5.0% | -4.4% | -4.2% |
| RGB, mixed content, 1080p | -5.6% | -6.0% | -5.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -9.2% | -12.5% | -11.9% |
| YUV, text & graphics with motion,720p | -7.3% | -9.9% | -12.0% |
| YUV, mixed content, 1440p | -5.0% | -10.4% | -10.3% |
| YUV, mixed content, 1080p | -5.1% | -9.2% | -10.0% |
| YUV, Animation, 720p | 0.1% | -0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.3% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 208% |  |  |

# Recommendation

It is recommended to extend the search range for string match at local area to further improve the performance.

# References

[1] JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

[2] Z. Ma, W. Wang, M. Xu, and H. Yu, “SCCE3 Test C.3: Combined Color Table and Index Map Coding of A.4 and B.8”, JCTVC-R0144, Sapporo, Japan, July 2014.

# Patent rights declaration(s)

Huawei Technologies (USA) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).