| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0272v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0062v2 |
| --- | --- |

| Title: | HEVCv1/MV-HEVC/SHVC HLS: Conformance cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Hendry Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com fhendry@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

V2 (revision 1) of this document provides the following:

- Syntax and semantics changes for signalling of the profile, tier, and level (PTL) syntax structure for the 0-th OLS at the beginning of the VPS extension, in response to the decision on the proposal item 1. These text changes are included in the attachment.

- An update to the proposal item 4, in order to be aligned with the decision on proposal item 1.

This document discusses layer sets, output layer sets (OLSs), and the use of the profile, tier, and level (PTL) syntax structure, the hrd_parameters( ) syntax structure, as well as the buffering period (BP), picture timing (PT), and decoding unit information (DUI) SEI messages for the conformance tests. The following items are proposed:

- To discuss the need of changing HEVC version 1 to allow signalling of tier and level for layer sets that are not corresponding to the entire bitstream in the base VPS, including for the 0-th layer set (i.e. the 0-th OLS that contains only the base layer), probably as a corrigendum.

- Disallowed the signalling of base-layer-output-only OLSs (where there is only one output layer that is the base layer) in the VPS extension.

- Clarify that directly nested BP, PT, and DUI SEI messages only apply to the operation points that correspond to base-layer-output-only OLSs. If item 4 is adopted, then this item is covered.

- Clarify the bitstream conformance test process to include three parts as described in subclause 1.7 of the contribution document.

# Discussions and proposals

## Layer sets signalled in the base VPS

All layer sets signalled in the base VPS are equivalent to OLSs for which there is only one output layer that is the base layer, referred to as base-layer-output-only OLSs. This is because decoders conforming to profiles defined in Annex A and their corresponding HRDs would only decode the base layer and ignore the enhancement layers, though all bits of the enhancement layer are counted in CPB operations when the OLS is decoded.

For the same reason as above, regardless of whether the entire bitstream corresponds to a layer set signalled in the base VPS, it is also equivalent to a base-layer-output-only OLS.

For convenience, an OLS for which there is at least one output layer that is not the base layer is referred to as a non-base-layer-output-only OLS.

## Profile

The profile is the same for all base-layer-output-only OLSs, including the one corresponding to the entire bitstream. This is true even when the base layer is externally provided, in which case the profile is not specified (the PTL syntax structure in the base VPS contains only all-zero bits).

When the base layer is in the bitstream, the profile for all the base-layer-output-only OLSs is signalled in the PTL syntax structure of the active SPS for the base layer and in the first PTL syntax structure of the active VPS (i.e. the PTL syntax structure in the base part of the active VPS).

## Tier and level

The tier and level signalled in the PTL syntax structure of the active SPS for the base layer and in the first PTL syntax structure of the active VPS are for the entire bitstream, which may or may not be correspond to a layer set or an OLS signalled in the VPS. However, as mentioned above, the entire bitstream corresponds to a base-layer-output-only OLS in any case.

Then where is the information of tier and level for other base-layer-output-only OLSs signalled, including the 0-th OLS that contains only the base layer itself in the layer set? Nowhere in the base VPS! Of course the tier and level of the entire bitstream would also apply, though not optimally.

Except the one for the 0-th OLS, the optimal tier and level for other base-layer-output-only OLSs may be signalled in the VPS part, but not required (because there is no requirement to have one base-layer-output-only OLS signalled in the VPS extension for each layer set signalled in the base VPS). JCTVC-R0227 / JCT3V-I0045 item 1 proposes to always signal the (optimal) and tier and level for the 0-th OLS.

From HEVC version 1 point of view, it seems a bit silly to do the bitstream conformance tests for all the operation points with their own HRD parameters, but checking the constraints against just the tier and level of the entire bitstream. This would allow the HRD parameters for all operation points other than the one corresponding to the entire bitstream to be much less optimal while still passing the conformance tests.

To avoid this issue, we should change HEVC version 1 to allow signalling of tier and level for layer sets that are not corresponding to the entire bitstream in the base VPS, including for the 0-th layer set (i.e. the 0-th OLS), probably as a corrigendum. If this is agreed, then the proposal item 1 in JCTVC-R0227 / JCT3V-I0045 should be ignored.

## HRD parameters in hrd_parameters( ) syntax structures

Inside the hrd_parameters( ) syntax structure, only three syntax elements are related to DPB operations, others are all for CPB operations only. These three DPB-related syntax elements are fixed_pic_rate_general_flag[ i ], fixed_pic_rate_within_cvs_flag[ i ], and elemental_duration_in_tc_minus1[ i ], one set for each value of TemporalId.

When pictures are cross-layer aligned (i.e. in each access unit there is a picture for each layer) and fixed picture rate is used, a hrd_parameters( ) syntax structure that applies to a layer set would apply to all OLSs that are associated with the layer set. However, in other situations, a hrd_parameters( ) syntax structure that applies to a layer set may be not applicable to all other OLSs associated with the layer set than the base-layer-output-only OLS.

In other words, we need to enable signalling of hrd_parameters( ) syntax structures in the VPS extension for OLSs specified in the VPS extension. Fortunately, this has been enabled in the AHG10 output text in JCTVC-R0010v3 / JCT3V-I0010v3.

## Questions related to above subsections

- Should it be required to signal a layer set in the base VPS to contain all layers of the bitstream? This is currently not required. Our preference is no (i.e. no technical change for this), as it can be derived, but some editorial clarifications would be needed, e.g. in the semantics of the hrd_parameters( ) syntax structure.

- Should it be allowed to signal layer sets containing the base layer also in the VPS extension? This is currently not allowed. Our preference is no (i.e. no change for this), as such layer sets can be signalled in the VPS base.

- Should it be disallowed to signal base-layer-output-only OLSs in the VPS extension, taking into account use cases with auxiliary picture layers that are not indicated as output layers? This is currently not disallowed. Our preference is yes (to disallow, i.e. make such a technical change), because these can anyway be covered by the base VPS part already.

## Applicability of non-nested and directly nested BP/PT/DUI SEI messages

Non-nested buffering period (BP), picture timing (PT), and decoding unit information (DUI) SEI messages apply to the entire bitstream (regardless of whether it corresponds to a layer set or OLS signalled in the VPS).

Directly nested BP, PT, and DUI SEI messages refer to BP, PT, and DUI SEI messages that are directly contained in scalable nesting SEI messages, instead of being indirectly nested (contained in bitstream partition nesting SEI messages that are in turn contained in the scalable nesting SEI messages). A non-nested SEI message is directly included in an SEI NAL unit, instead of being directly or indirectly nested.

According to HEVC version 1, the directly nested BP, PT, and DUI SEI messages certainly apply to the operation points that correspond to the layer sets signalled in the base VPS part, which are equivalent to base-layer-output-only OLSs. Can the directly nested BP, PT, and DUI SEI messages also apply to non-base-layer-output-only OLSs? Since these three SEI message contain DPB operations related parameters, often when they are directly nested, they would not apply to non-base-layer-output-only OLSs.

We therefore should clarify that directly nested BP, PT, and DUI SEI messages only apply to the operation points that correspond to base-layer-output-only OLSs.

This clarification can be used to resolve the following editor's note in subclause C.1 in the AHG10 output text in JCTVC-R0010v3 / JCT3V-I0010v3 (otherwise some inference rules for the default OLSs that a directly nested BP, PT, or DUI SEI would need to be specified – we considered that option but none of those straightforward inference rules we thought of seemed to be desirable):

[Ed. (MH): The selection between the scalable nesting SEI message and the bitstream partition nesting SEI message could be specified more clearly.]

Consequently, for all non-base-layer-output-only OLSs specified in the VPS extension, the applicable BP, PT, and DUI SEI messages are indirectly nested.

## Bitstream conformance tests

The bitstream conformance tests should consider the following three parts:

- Tests for the entire bitstream and its temporal subsets. In these tests only the base layer is the output layer (i.e. the entire bitstream is equivalent to a base-layer-output-only OLS). Use the PTL syntax structure and the hrd_parameters( ) syntax structure in the SPS active for the base layer for all these tests and the non-nested BP, PT, and DUI SEI messages. When there is a layer set corresponding to the entire bitstream, the applicable hrd_parameters( ) syntax structure in the active VPS and the applicable nested BP, PT, and DUI SEI messages may also be used alternatively.

- Tests for layer sets (that do not correspond to the entire bitstream) and their temporal subsets. These layer sets are equivalent to base-layer-output-only OLSs. The applicable hrd_parameters( ) syntax structure in the base part of the active VPS, and the applicable and directly nested BP, PT, and DUI SEI messages are used.

As part of version 2 (not version 1), when vps_max_layers_minus1 is greater than 0, a set of conformance tests is performed for the 0-th OLS and its temporal subsets, by using the 2nd PTL syntax structure in the VPS (i.e. the one at the beginning of the VPS extension), the applicable hrd_parameters( ) syntax structure in the base part of the active VPS, and the applicable and directly nested BP, PT, and DUI SEI messages.

- Tests for all OLSs specified in the VPS extension, excluding the base-layer-output-only OLSs that are covered above, and their temporal subsets, including those OLSs corresponding to the additional layer sets that do not contain the base layer.

For the base-layer-output-only OLSs among all OLSs specified in the VPS extension, if any, the applicable hrd_parameters( ) syntax structure either in the base or the extension part of the active VPS, and the applicable and directly nested BP, PT, and DUI SEI messages are used.

For non-base-layer-output-only OLSs among all OLSs specified in the VPS extension, the applicable hrd_parameters( ) syntax structure, either in the base or the extension part of the active VPS, and the applicable, indirectly nested BP, PT, and DUI SEI messages are used.

Note that if a non-base-layer-output-only OLS contains all layers in the entire bitstream, still only indirectly nested BP, PT, and DUI SEI messages may be used, not the non-nested ones, as the non-nested ones are for the base-layer-output-only OLS containing all layers in the entire bitstream.

The use of the PTL syntax structure is the same for both base-layer-output-only OLSs and non-base-layer-output-only OLSs among all OLSs specified in the VPS extension. The most appropriately applicable PTL syntax structure, either in the base VPS or in the VPS extension, is used.

Note that if a non-base-layer-output-only OLS contains all layers in the entire bitstream, still only a PTL syntax structure in the VPS may be used, not the one in the PTL syntax structure in the SPS active for the base layer, as that one is for the base-layer-output-only OLS containing all layers in the entire bitstream.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).