| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0279v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0091v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: More miscellaneous cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Adarsh K. Ramasubramonian Qualcomm Incorporated 5775 Morehouse Drive San Diego CA 92121 | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes the following changes:

- Clarification of the semantics of access unit delimiter RBSP to be applicable in multi-layer context (text in an attachment)

- Clarification of the specifications of order of NAL units and coded pictures and their association to access units in both subclauses 7.4.2.4.4 and F.7.4.2.4.4 (text in an attachment)

- To specify that for any independent non-base layer the used representation format is the one that is signalled in the active SPS for the layer

- To clarify the use of parameters in parameter sets for interpretation of semantics of the frame-field information SEI message, including the following two parts, and it is also suggested to make a systematic check for other SEI messages for the need of similar clarifications:

  - To require that a frame-field information SEI message that applies to a particular set of layers shall not be present when the value of field_seq_flag is not the same for all active SPSs for the particular set of layers

  - To require that a frame-field information SEI message that applies to a particular set of layers shall not be present when general_progressive_source_flag and general_interlaced_source_flag are not identical, respectively, for all the profile, tier and level syntax structures that apply to the bitstream partitions that contain the layers to which the frame-field information SEI message applies

- To clarify the semantics of the temporal sub-layer zero index SEI message for its use in multi-layer contexts, and to add a syntax element to indicate the number of previous consecutive temporal sub-layer zero pictures that have discardable_flag equal to 1 (text in an attachment)

- To change the recovery point SEI message to correct the POC derivation when random access is performed and to clarify/add the POC derivation when layer-switching is performed. To enable the use of recovery point SEI messages in bitstream with poc_msb_val signalled, a syntax element is added to indicate the POC MSB information of the pictures associated with the recovery point SEI message. (text in an attachment)

- In the version v2 of this document, the proposal of the additional syntax element in the recovery point SEI message is removed, and two options (Option 1 and Option 2) for a constraint on the recovery point SEI message are proposed. Option 1 is preferred by the proponents.

# Discussions and proposals

## On access unit delimiter

The semantics of the syntax element pic_type in the access unit delimiter (AUD) NAL unit is not clear when used in SHVC / MV-HEVC. An access unit in SHVC / MV-HEVC may contain one or more coded pictures, yet the semantics of pic_type says that it may be used to indicate the type of slices present in "a coded picture". As long as an AUD NAL unit can apply to two or more pictures, the current semantics is unclear. It is therefore proposed to update the semantics of AUD RBSP and pic_type syntax element to clarify this aspect.

The proposed text changes are provided in an attachment.

## On access unit boundary

Document JCTVC-R0236 / JCT3V-I0049 includes some nice improvements on the specification of order of NAL units and coded pictures and their association to access units in subclause 7.4.2.4.4. We identified some minor issues and tried to further improve the text. We also checked the text in subcluase F.7.4.2.4.4 of the latest MV-HEVC/SHVC specs and tried to improve the text there accordingly.

One changed aspect that might worth a written-down discussion is as follows. The HEVC version 1 specification has the following constraint in subclause 7.4.2.4.4:

When an access unit delimiter NAL unit is present (in an access unit), it shall be the first NAL unit (of the access unit). There shall be at most one access unit delimiter NAL unit in any access unit.

This constraint applies to the HEVC version 1 Specification. However, when a multi-layer bitstream contains AUD NAL units with nuh_layer_id greater than 0, such AUD NAL units should not be considered to be the beginning of a new access unit by a version 1 decoder. Failing to do so, the decoder would see empty access units as it would not recognize any NAL units that have nuh_layer_id greater than 0 and consider the bitstream as non-conforming. Therefore, it is proposed to change that constraint as follows:

When an access unit delimiter NAL unit with nuh_layer_id equal to 0 is present (in an access unit), it shall be the first NAL unit(of the access unit). There shall be at most one access unit delimiter NAL unit with nuh_layer_id equal to 0 in any access unit.

Text changes are provided in an attachment, in relative to the text provided in JCTVC-R0236v1 / JCT3V-I0049v1.

## On the representation format of independent non-base layers

A layer with nuh_layer_id greater than 0, regardless of whether it is an independent non-base layer (INBL), uses the representation format signalled in the VPS. If an INBL that refers to an SPS with V1CompatibleSPSFlag equal to 1 or nuh_layer_id equal to 0 is to be rewritten to be a base layer, e.g. as specified in the AHG10 output text in JCTVC-R0010v3 / JCT3V-I0010v3, when the used representation format from the VPS is different from the representation format (for any of the parameters) signalled in the SPS, then the rewriting process needs to change the SPS such that the representation format is the same as the used representation format from the VPS. This requirement is undesirable.

It is there proposed that for any INBL the used representation format is the one that is signalled in the active SPS for the layer.

No text is currently provided for this item.

## On SPS and PPS parameters for interpretation of SEI messages

An SEI message may apply to multiple layers or an (output) operation point associated with a (output) layer set containing multiple layers, e.g. when the SEI message is nested. When this happens, there can be multiple active SPSs and multiple active PPSs, e.g. one for each layer, involved when it comes to interpretation of such an SEI message. However, it seems that at least for some SEI messages, which one of the multiple active SPSs and multiple active PPSs should be used for the interpretation is unclear.

For example, general_progressive_source_flag and general_interlaced_source_flag are referred to in the semantics of ffinfo_source_scan_type in the frame-field information SEI message. These two flags are located in the Profile, Tier and Level (PTL) syntax structure, which can be in each of the active SPSs, and multiple of them can be in the VPS as well. Therefore it should be clarified which PTL syntax structure containing the two flags applies when the frame-field information SEI message applies to multiple layers.

Similarly for the SPS flag field_seq_flag that is used in the semantics of ffinfo_pic_struct and ffinfo_duplicate_flag in the frame-field information SEI message.

To solve the above problem, the followings are proposed:

- It is required that, the value of field_seq_flag shall be the same for all active SPSs for the layers to which the frame-field information SEI message applies. What is to be put to the text should be something like the following:

A frame-field information SEI message that applies to a particular set of layers shall not be present if the value of field_seq_flag is not the same for all active SPSs for the particular set of layers.

- Similarly for general_progressive_source_flag and general_interlaced_source_flag, they are required to be identical, respectively, for all the PTL syntax structures that apply to the bitstream partitions that contain the layers to which the frame-field information SEI message applies. The language in the spec should be something like the following:

A frame-field information SEI message that applies to a particular set of layers shall not be present when general_progressive_source_flag and general_interlaced_source_flag are not identical, respectively, for all the profile, tier and level syntax structures that apply to the bitstream partitions that contain the layers to which the frame-field information SEI message applies.

Similar restrictions apply for such syntax elements of other SEI messages as well. Basically, for any SEI message seiA that applies to multiple layers or multiple (output) layer sets, when multiple instances of any syntax element that is not part of seiA are involved in the semantics of any syntax element that is part of seiA, it is required that the value of the syntax element that is not part of seiA is the same for all the instances.

A systematic check for other SEI messages should be made.

No text is currently provided for this item beyond the above.

## On the temporal sub-layer zero index SEI message

The temporal sub-layer zero index (TSLZI) SEI message is used to identify pictures with TemporalId equal to 0 that are not RASL, RADL, or sub-layer non-reference (SLNR) pictures; for ease of description, such pictures are referred to as ReferencedTsl0Pic. When a ReferencedTsl0Pic is lost, and a subsequent picture in decoding order with the TSLZI SEI message is received, the decoder is able to identify from the temporal_sub_layer_zero_idx syntax element of the SEI message that the ReferencedTsl0Pic was lost and may perform error-recovery.

Currently, the semantics of the TSLZI SEI message has not been clarified to be (correctly) applicable in multi-layer contexts. Furthermore, there is a problem for the TSLZI SEI message to work together with discardable_flag, because the TSLZI SEI message does not consider pictures with discardable_flag equal to 1.

Discardable pictures are pictures that have discardable_flag equal to 1, and the decoding process of the rest of the pictures in the bitstream is not affected even if these pictures are not present in the bitstream. Hence, when a picture that is ReferencedTsl0Pic and discardable picture is lost, the decoder need not perform any error-recovery operations. However, with the current design, there is no way for the decoder to know whether a lost ReferencedTsl0Pic is a discardable picture or not. In other words, the SEI message is not working well together with discardable_flag.

Therefore, we propose text changes to clarify the semantics of the TSLZI SEI message to be (correctly) applicable in multi-layer contexts, and it is also proposed to extend the TSLZI SEI message by adding a syntax element to indicate the number of previous consecutive temporal sub-layer zero pictures that have discardable_flag equal to 1. This information would help decoder to understand whether a lost temporal sub-layer zero picture is actually a discardable picture and the decoder does not need to take any error recovery action for that loss.

Text changes are provided in an attachment.

## On the recovery point SEI message

When decoding starts at an access unit auA containing a recovery point SEI message and that recovery point SEI applies to a set of layers layerIdList, the MSB of the POC of the pictures in layers belonging to layerIdList are inferred to be equal to 0. In some situations, as shown in Figure 1 and Figure 2, this could result in incorrect referencing of POC of pictures. In the original bitstream (Figure 1), AU 256 contains a recovery point SEI message that applies to both the layers, and AU 512 is a POC resetting AU. Pictures in AU 512 refer to pictures in AU 511, with POC difference of −1; this value (−1) is signalled as delta POC for the reference pictures. The POC MSB values of the pictures in AU 256 and AU 511 are not signalled, while the MSB values of the pictures in AU 512 are signalled. When decoding starts at AU 256, the POC of pictures would be derived to be equal to 0 (because the signalled LSB would be equal 0). The POC of pictures in AU 511 would be derived to be equal to 255, and after POC is reset in AU 512, the updated POC of the pictures in AU 511 is equal to −257 (255 − 512). When pictures in AU 512 refer to pictures with POC difference of −1, no reference pictures would be found because the difference between POC of pictures in AU 512 and AU 511 now is −257. This could trigger the decoder to conclude that pictures may have been lost.

Figure 1: Bitstream with AU 256 containing a recovery point SEI message and MSB of POC pictures in AU 256 and AU 511 are not signalled. AU 512 is a POC resetting AU with only MSB reset, and MSB of pictures in AU 512 are signalled.

Figure 2: Decoding starts at AU 256. After reset at POC 512, POC of earlier pictures are decremented by 512 and pictures in AU 512 cannot find reference pictures with delta POC equal to −1.

To solve the above problem, it is proposed to extend the recovery point SEI message by adding a syntax element to indicate the POC MSB information of pictures associated with the recovery point SEI message. This information would be used to derive the POC value of the current picture (i.e., the picture associated with the recovery point SEI) when random access is performed from the access unit containing the SEI message.

Text changes are provided in an attachment.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).