| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0281 |
| --- | --- |

| Title: |  | Non-SCCE2: An improvement on Intra Line Copy of SCCE2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |
| Author(s) or Contact(s): | Liping Zhao, Xianyi Chen, Tao Lin 1239 Siping Ave, Shanghai 200092, China |  | Email: | lintao@tongji.edu.cn |  |
| Source: | Tongji University |  |  |  |  |

_____________________________

# Abstract

This contribution proposes an improvement on Intra Line Copy (Line-based Intra Copy) technique described in the Screen Content Core Experiment 2 (JCTVC-Q1122). The improvement is to allow a current line to copy a reference line inside the same CU as the current line or partially overlapped with the current line. Experiment results report -1.8%, -0.8%, and -0.4% BD-rate savings for YUV TGM 1080p sequences under AI, RA, and LB, respectively, using SCCE2 TestD as the anchor.

# Introduction

Intra Line Copy (ILC, a short name for Line-based Intra Copy) [1]-[3] combines the merits of IBC and Intra String Copy (SCCE4) to provide a good solution for SCC. The current ILC copies reference lines from outside of the current CU/PU, as shown in Figure 1.

# Description of Improvement

In the improvement, a reference line can come from inside of the same CU/PU as the current line (in-CU self-matching) and can also partially overlap with the current line (in-line self-matching), as shown in Figure 2.

In the overlapped case, the incomplete reference line is partially replicated or repeatedly replicated to make the reference line complete, as shown in Figure 3.

# Experimental Results

The following experiments were conducted using the HM-14.0+RExt-7.0+SCM-1.0 software [4] and the screen content common test conditions [5] to collect the BD-rate, encoding and decoding time of the proposed techniques in the All-Intra, Random Access, Low Delay B configurations. The SCCE2 TestD software is modified to implement the improvement. The following is lossy and lossless results. The anchor is SCCE2 TestD.

Table 1. BD-rate reduction in lossy coding.

Table 2. Bit-rate saving in lossless coding.

# Conclusion

In this contribution, ILC is improved by allowing reference lines from inside of the same CU/PU as the current line or partially overlapped with the current line. Experiment results show -1.8%, -0.8%, and -0.4% BD-rate savings for YUV TGM 1080p sequences under AI, RA, and LB, respectively. We would recommend the group to consider further studying this technique with other improvements of ILC in a CE.

# References

- C. C. Chen, X. Xu, and L. Zhang, “HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy,” JCTVC-Q1122, Valencia, Mar. 2014.

- C. C. Chen, T. S. Chang, R. L. Liao, W. H. Peng, H. M. Hang, C. L. Lin, and F. D. Jou, "AHG8: Line-based Intra Block Copy," JCTVC-O0205, Geneva, Oct. 2013.

- C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, Y.-J. Chang, C.-H. Hung, C.-C. Lin, J.-S. Tu, E.-C. Ke, J.-Y. Kao, C.-L. Lin, F.-D. Jou, and F.-C. Chen, "Description of screen content coding technology proposal by NCTU and ITRI International," JCTVC-Q0032, Valencia, Mar. 2014.

- JCT-VC, “HM-14.0+RExt-7.0+SCM-1.0,”  https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-14.0+RExt-7.0+SCM-1.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Conditions for Screen Content Coding Tests,” JCTVC-Q1015, Valencia, Mar. 2014.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).