| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0292 |
| --- | --- |

| Title: | Crosscheck of JCTVC-R0281: An improvement on Intra Line Copy of SCCE2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, Tsui-Shan Chang, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of JCTVC-R0281 which improves intra line copy of SCCE2. The software package provided by the proponent was tested under the screen content common test conditions specified in JCTVC-Q1015 [1]. Since the proponent did not correctly configure the level setting, the cross-check results have slightly rate difference at some rate points compared to the data provided by the proponent. Since the differences are smaller than 0.01%, the impact on BD-rate is negligible. Table 1 and Table 2 show the simulation results for lossy and lossless coding, respectively.

# Test results

Table 1. Simulation results of JCTVC-R0281 for lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.7% | -8.6% | -8.6% |
| RGB, text & graphics with motion,720p | -4.1% | -4.1% | -4.1% |
| RGB, mixed content, 1440p | -3.5% | -3.5% | -3.5% |
| RGB, mixed content, 1080p | -4.4% | -4.4% | -4.5% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -8.3% | -8.2% | -8.1% |
| YUV, text & graphics with motion,720p | -3.5% | -3.9% | -4.1% |
| YUV, mixed content, 1440p | -2.8% | -3.5% | -3.7% |
| YUV, mixed content, 1080p | -3.7% | -4.6% | -4.3% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.0% | -4.9% | -5.0% |
| RGB, text & graphics with motion,720p | -3.1% | -3.0% | -3.0% |
| RGB, mixed content, 1440p | -2.7% | -2.4% | -2.5% |
| RGB, mixed content, 1080p | -3.0% | -3.2% | -3.2% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.4% | -4.7% | -4.6% |
| YUV, text & graphics with motion,720p | -2.7% | -3.0% | -3.4% |
| YUV, mixed content, 1440p | -1.8% | -2.8% | -3.0% |
| YUV, mixed content, 1080p | -2.8% | -3.8% | -3.5% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.1% | -3.0% | -3.0% |
| RGB, text & graphics with motion,720p | -2.0% | -1.9% | -1.7% |
| RGB, mixed content, 1440p | -1.5% | -1.6% | -1.5% |
| RGB, mixed content, 1080p | -1.3% | -1.2% | -1.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.7% | -2.7% |
| YUV, text & graphics with motion,720p | -1.6% | -1.9% | -2.1% |
| YUV, mixed content, 1440p | -1.0% | -2.1% | -2.0% |
| YUV, mixed content, 1080p | -0.9% | -2.0% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.6% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Simulation results of JCTVC-R0281 for lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 8.5% | 8.5% | 5.5% | 12.9% |
| RGB, text & graphics with motion,720p | 2.1% | 2.4% | 0.7% | 4.4% |
| RGB, mixed content, 1440p | 1.1% | 1.3% | 0.7% | 1.9% |
| RGB, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 8.7% | 8.6% | 5.6% | 12.9% |
| YUV, text & graphics with motion,720p | 2.2% | 2.8% | 0.6% | 6.2% |
| YUV, mixed content, 1440p | 1.2% | 1.3% | 0.6% | 1.9% |
| YUV, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.8% | 5.2% | 2.3% | 9.4% |
| RGB, text & graphics with motion,720p | 0.5% | 1.3% | 0.4% | 3.7% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.2% | 0.4% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.8% | 5.3% | 2.6% | 9.6% |
| YUV, text & graphics with motion,720p | 0.5% | 1.7% | 0.4% | 5.2% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.2% | 0.4% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.5% | 4.0% | 1.8% | 6.5% |
| RGB, text & graphics with motion,720p | 0.4% | 0.9% | 0.3% | 2.5% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.4% | 4.1% | 2.1% | 6.8% |
| YUV, text & graphics with motion,720p | 0.4% | 1.1% | 0.3% | 3.3% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Mar. 2014.