| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0293 |
| --- | --- |

| Title: | Crosscheck of JCTVC-R0139 Modulation channel SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony corporation |  |  |

_____________________________

# Abstract

This is a crosscheck report of JCTVC-R0139.

The verification test is conducted using the provided software from the proponents. It is commented that two modes are implemented in the software package, Frame Packing mode and Auxiliary Picture mode. The Frame packing mode is cross-checked in the document.

Five HDR sequences are tested, which detail is reported in JCTVC-P0228. It is noted that the original files are OpenEXR format and converted to 16bit integer XYZ format, using the independent conversion tool provided by the proponents.

The modified encoder re-formats the input from 1920x1080 16bit to (vertically doubled, frame-packed) 1920x2168 10bit, before the encoding. The modified decoder post-processes the decoded result oppositely, and outputs 1920x1080 16bit format file. Afterward, PSNR is calculated by the Delta E 2000 metric, using the provided software.

It is reported that the simulation result by the cross-checker is identical to the result by the proponents

# Cross-check

## Source code

Source code is provided by the proponents as a package. Two modes are implemented in the package, Frame Packing mode and Auxiliary Picture mode.

The Frame packing mode is cross-checked in the document.

It is based on HM-13.0+RExt-6.0rc1. It is commented that the changes are basically to handle the proposed frame-packing method appropriately, and no cu-level change is implemented in terms of encoding/decoding process.

It is highlighted that the following macro setting are used.

#define HDR_FRAME_PACKING               1

#define HDR_SHM_ILLUM_MAP               0

Other macro settings are not modified from the original package.

## Test material

Five HDR sequences are tested. The detail information of the content is found in JCTVC-P0228 [3]. It is noted that the original files are OpenEXR format.

The original is converted to 16bit integer XYZ format, using the independent conversion tool provided by the proponent.

The conversion software 1) converts from RGB to XYZ in the floating point, 2) applies 709 gamma in the floating point, and finally 3) converts to 16bit integer with rounding.

Table1 shows md5sum of the converted results.

Table 1 md5sum of the input

| Filename | Md5sum |
| --- | --- |
| Balloon_1920x1080p_25_hf_709.xyz | ba00b5ab5453da71f7fa7e766b621348 |
| FireEater2_1920x1080p_25_hf_709.xyz | d7bf98439bdd4c39ccdc7441fbcd7135 |
| Market3_1920x1080p_50_hf_709.xyz | 6e725c33efb184c0318ec69db58d8d4e |
| Seine_1920x1080p_25_hf_709.xyz | 45ffb5c9d8529972ab88f31a74764272 |
| Tibul2_1920x1080p_30_hf_709.xyz | 7193e7be4ed5d42a7fad24524aad0faf |

## Encoder configuration setting

The test is conducted based on “encoder_randomaccess_main10.cfg.” The other CTC setting is not tested.

It is noted that InputChromaFormat is set to 444, in order to correctly handle the input. Except it, the CTC setting is not modified. Thus, basically no additional RExt coding tools from Ver1 are enabled in the test.

As for the frame packing, the following parameters are used. The parameter values are not changed from the original software. It is commented the detail behavior of the each parameter is not cross-checked.

#============ Dual Modulation ======================

DualModulation                      : 1

InverseDualMod                      : 1

DualModOnly                         : 0

DualModInputExtension               : exr

DualModStartPicture                 : 00000

DualModOutputLDR                    : orig_ldr

DualModExternalChromaFormat         : 444

DualModLDRSpace                     : LAB

InverseDualModConfig                : HM_2_HDR

InverseDualModReconFile             : rec_hdr.xyz

#============ Dual Modulation Parameters ==================

DualModLambda                       : 3200

DualModMu                           : 400

DualModRGBFactor                    : 40

DualModSLog                         : 2.5

DualModScalingFactor                : 60

DualModPSFXSpacing                  : 40

DualModPSFYSpacing                  : 40

DualModPSFWidth                     : 3

DualModPSFHeight                    : 3

DualModEnableTemporal               : 0

#============ SEI FramePacking ======================

SEIHdrFramePacking                      : 1

QPHdrFramePacking                               : 15

QPDeltaHdrFramePacking  :       0

The modified encoder re-formats the input from 1920x1080 16bit to (vertically doubled, frame-packed) 1920x2168 10bit, before the encoding. Figure 1 shows an example of the proposed frame packed image.

The modified decoder post-processes the decoded result oppositely, and outputs 1920x1080 16bit format file. Afterward, PSNR is calculated by the Delta E 2000 metric, using the provided software.

Figure 1 Example of Frame packed HDR image, 1920x2168 10bit

(Note: the color is not correctly expressed in the figure due to the capturing tool of the x-checker)

## Result

The result is shown in Table 2. The complete result is also attached in the contribution. It is identical to the result provided by the proponents.

It is noted that X, Y, Z psnr, DE 2000 and PSNR-DE2000 measurement is conducted by the proponent’s software. The input is 1920x1080 16bit file. The post-processed decode result is compared to the original as shown in Table 1.

Table 2 Summary result by the cross-checker

| Sequence | QPISlice | kbps | X psnr | Y psnr | Z psnr | DE2000 | PSNR-DE2000 | Enc T [s] | Dec T [s] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Balloon | 18 | 7338 | 47.08 | 46.87 | 47.35 | 2.08 | 44.98 | 18709.43 | 153.47 |
|  | 19 | 5397 | 46.86 | 46.66 | 47.17 | 2.12 | 44.91 | 17833.44 | 153.20 |
|  | 21 | 3164 | 46.39 | 46.22 | 46.79 | 2.18 | 44.77 | 16572.23 | 144.99 |
|  | 23 | 2061 | 45.85 | 45.73 | 46.36 | 2.25 | 44.63 | 15730.83 | 146.33 |
|  | 25 | 1432 | 45.23 | 45.16 | 45.86 | 2.34 | 44.48 | 15274.57 | 141.33 |
| FireEater2 | 20 | 7131 | 55.31 | 56.64 | 57.52 | 0.38 | 52.40 | 24736.43 | 159.84 |
|  | 21 | 5573 | 54.49 | 55.76 | 56.75 | 0.40 | 52.22 | 23405.31 | 156.86 |
|  | 24 | 2792 | 51.92 | 53.00 | 54.36 | 0.45 | 51.70 | 20316.82 | 150.50 |
|  | 25 | 2297 | 51.12 | 52.14 | 53.63 | 0.47 | 51.52 | 19524.49 | 148.10 |
|  | 27 | 1619 | 49.58 | 50.51 | 52.21 | 0.51 | 51.16 | 18376.53 | 144.46 |
| Seine | 25 | 9194 | 47.65 | 47.84 | 46.64 | 1.83 | 45.55 | 18972.99 | 154.45 |
|  | 28 | 4870 | 45.20 | 45.36 | 44.47 | 2.01 | 45.14 | 17156.48 | 145.52 |
|  | 30 | 3264 | 43.75 | 43.89 | 43.18 | 2.14 | 44.87 | 16329.88 | 146.55 |
|  | 33 | 1822 | 41.87 | 42.00 | 41.48 | 2.35 | 44.46 | 15515.91 | 142.86 |
|  | 34 | 1517 | 41.36 | 41.48 | 40.99 | 2.42 | 44.33 | 15296.62 | 142.91 |
| Market3 | 24 | 9781 | 41.86 | 41.64 | 40.84 | 3.05 | 43.33 | 38665.03 | 303.21 |
|  | 26 | 6992 | 40.63 | 40.41 | 39.62 | 3.33 | 42.93 | 37121.23 | 299.08 |
|  | 29 | 4367 | 38.79 | 38.58 | 37.83 | 3.81 | 42.36 | 35362.42 | 296.85 |
|  | 32 | 2750 | 36.94 | 36.73 | 36.05 | 4.36 | 41.77 | 34105.81 | 304.34 |
|  | 34 | 2024 | 35.75 | 35.54 | 34.89 | 4.77 | 41.38 | 33483.08 | 290.51 |
| Tibul2 | 21 | 10678 | 53.73 | 55.42 | 58.18 | 0.89 | 48.69 | 35281.74 | 196.25 |
|  | 23 | 7630 | 52.35 | 53.99 | 57.04 | 1.01 | 48.16 | 33160.45 | 192.93 |
|  | 25 | 5382 | 50.97 | 52.57 | 55.87 | 1.14 | 47.62 | 30985.83 | 189.91 |
|  | 29 | 2680 | 48.48 | 50.02 | 53.64 | 1.43 | 46.64 | 27131.42 | 184.34 |
|  | 31 | 1879 | 47.35 | 48.85 | 52.47 | 1.58 | 46.19 | 26192.00 | 182.51 |

# Reference

- F. Le Leannec, S. Lasserre and E. Francois, “Modulation channel SEI message,” JCTVC-R0139, 18th Meeting Sapporo, JP, June 2014

- F. Le Leannec, S. Lasserre and E. Francois, “Usage of modulation channel for high bit-depth signal encoding,” JCTVC-R0267, 18th Meeting Sapporo, JP, June 2014

- S. Lasserre, F. Le Leannec and E. François, “Description of HDR sequences proposed by Technicolor,” JCTVC-P0228, 16th Meeting San José, US, Jan. 2014

- C. Rosewarne, K. Sharman and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-P1006, 16th Meeting San José, US, Jan. 2014

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).