| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0300 |
| --- | --- |

| Title: | Non-SCE1: improved residual coefficient coding for 3D LUT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yuwen He 9710 Scranton Rd, Suite 250 San Diego, CA 92130 | Tel: Email: | +1-858-210-4803/4819 yan.ye@interdigital.com yuwen.he@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a small change to the residual coefficient coding method in JCTVC-R0151. Although the residual coefficient coding method in JCTVC-R0151 is effective in reducing 3D LUT coding bits, more frequently updated 3D LUT causes the overall 3D LUT signaling overhead in JCTVC-R0151 to increase by 14% compared to SHM6.0. It is reported that the proposed small change can reduce the overall signaling overhead of 3D LUT in JCTVC-R0151 by 8%. With the proposed change, the signaling overhead of 3D LUT using JCTVC-R0151 is reduced and becomes comparable to that of SHM6.0.

# Introduction

JCTVC-R0151 proposes the following changes to SHVC draft 6:

- U and V partition information is adaptively selected and signaled in the bitstream

- Within each octant, (A, B, C, D) parameters are used for cross component linear prediction. This replaced the vertex based parameters in the current design. Prediction of the (A, B, C, D) model parameters is simplified.

- The residual coefficients representing the prediction residuals of the (A, B, C, D) parameters are coded in 3 parts: the MSBs of the magnitude value are coded using Exp-Golomb coding, the 7 LSBs of the magnitude value are coded using fixed length coding, and the sign is coded as 1-bit flag if magnitude is non-zero.

JCTVC-R0151 reports that 1.2%, 0.9%, 1.1% and 0.4% luma BD-rate reduction is achieved on average for AI-1x, AI-2x, RA-1x, and RA-2x, respectively. Chroma BD-rate reduction is higher. However, signaling overhead of 3D LUT based on the results reported in JCTVC-R0151 also increases. Deeper analysis shows that adaptive U/V partition increases the effectiveness of the 3D LUT. Particularly, for the two “BirthdayFlash” sequences, this increased effectiveness causes more PPS updates with new 3D LUT being sent. More frequently sent 3D LUT also increases the overall signaling overhead compared to SHM6.0. As shown in Table 1, for all test cases, JCTVC-R0151 has 16% increase in 3D LUT signaling overhead compared to that of SHM6.0. The increased overhead is especially pronounced for the two “BirthdayFlash” sequences.

Table 1. 3D LUT signaling overhead of SHM6.0 and R0151

|  |  | Total # of 3D LUT bits |  | % increase |
| --- | --- | --- | --- | --- |
|  |  | SHM6.0 | R0151 |  |
| AI 1x | All sequences | 3772360 | 4438948 | 18% |
|  | Two “BirthdayFlash” sequences | 2931460 | 3694998 | 26% |
| AI 2x | All sequences | 3779510 | 4228036 | 12% |
|  | Two flash sequences | 2901170 | 3509570 | 21% |
| RA 1x | All sequences | 1548418 | 1768084 | 14% |
|  | Two “BirthdayFlash” sequences | 1069898 | 1316350 | 23% |
| RA 2x | All sequences | 1452478 | 1594332 | 10% |
|  | Two “BirthdayFlash” sequences | 990120 | 1141750 | 15% |
| All cases | All sequences | 10552766 | 12029400 | 14% |

# Proposed method

The entropy coding method in JCTVC-R0151 is effective in reducing the 3D LUT signaling overhead. However, using u(7) to code the 7 LSBs is not optimal. Let us examine the composition of a residual coefficient, denoted as CMResX (X being Y, U or V). The model parameters in JCTVC-R0151 are fixed to 12-bits. After prediction, the residual coefficient CMResX is quantized by removing the last cm_res_quant_bits number of bits. Therefore, if we denote the magnitude of CMResX to be M, then the number of bits needed to represent M is 12-cm_res_quant_bits. The value of M has 2 parts, an integer part represented with (12 – nMappingShift) bits, and a fractional part represented with (nMappingShift – cm_res_quant_bits) bits, where nMappingShift is the precision of fixed point operation used in color mapping process, nMappingShift = 10 + inputBitDepth – outputBitDepth. Such composition applies to 3 of the 4 model parameters (except D).

Based on this observation, this contribution proposes a small change to the entropy coding method in JCTVC-R0151 to reduce the 3D LUT signaling overhead. Instead of using u(7) to code the LSBs, it is proposed to use u(v), with v = (nMappingShift – cm_res_quant_bits – cm_delta_flc_bits), to code the LSBs. The value of cm_delta_flc_bits is selected by the encoder and signaled as part of the 3D LUT syntax, as shown below.

Table 2. Proposed syntax table for colour_mapping_table()

| colour_mapping_table( ) { | Descriptor |
| --- | --- |
| cm_octant_depth | u(2) |
| cm_y_part_num_log2 | u(2) |
| cm_input_luma_bit_depth_minus8 | u(3) |
| cm_input_chroma_bit_depth_delta | se(v) |
| cm_output_luma_bit_depth_minus8 | u(3) |
| cm_output_chroma_bit_depth_delta | se(v) |
| cm_res_quant_bits | u(2) |
| cm_delta_flc_bits_minus1 | u(2) |
| if( cm_octant_depth = = 1 ) { |  |
| cm_adapt_threshold_u_delta | se(v) |
| cm_adapt_threshold_v_delta | se(v) |
| } |  |
| colour_mapping_octants( 0, 0, 0, 0, 1 << cm_octant_depth ) |  |
| } |  |

cm_delta_flc_bits_minus1 specifies the delta value used to derive the number of bits used to code res_coeff_r. The variable CMResLSBits is set equal to max (0, (10 + CMInputBitDepthY − CMOutputBitDepthY – cm_res_quant_bits – ( cm_delta_flc_bits_minus1 + 1 ))).

Table 3. Proposed syntax table for colour_mapping_octants()

| colour_mapping_octants( depth, yIdx, uIdx, vIdx, length ) { [Ed. (MH): Camel casing suggested for all input parameters, i.e. depth and length should be changed.] | Descriptor |
| --- | --- |
| if ( depth < cm_octant_depth ) |  |
| split_octant_flag | u(1) |
| if ( split_octant_flag ) |  |
| for( k = 0; k < 2; k++ ) |  |
| for( m = 0; m < 2 ; m++ ) |  |
| for( n = 0; n < 2; n++ ) |  |
| colour_mapping_octants( depth + 1, yIdx + YPartNum * k * length / 2, uIdx + m * length / 2, vIdx + n * length / 2, length / 2) |  |
| Else |  |
| for( i = 0; i < YPartNum; i++ ) |  |
| for( j = 0; j < 4; j++ ) { |  |
| coded_res_flag[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ] | u(1) |
| if( coded_res_flag[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ] ) |  |
| for( c = 0; c < 3; c++ ) { |  |
| res_coeff_q[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ][ c ] ) | ue(v) |
| res_coeff_r[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ][ c ] ) | u(7) u(v) |
| if( res_coeff_q[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ][ c ] ¦ ¦ res_coeff_r[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ][ c ]) |  |
| res_coeff_s[ yIdx + (i << (cm_octant_depth-depth)) ][ uIdx ][ vIdx ][ j ][ c ] ) | u(1) |
| } |  |
| } |  |
| } |  |

res_coeff_q[ yIdx ][ uIdx ][ vIdx ][ i ][ c ] specifies the quotient of the residual for the colour mapping coefficient with index [ yIdx ][ uIdx ][ vIdx ][ i ][ c ]. When not present, the value of res_coeff_q is inferred to be equal to 0.

res_coeff_r[ yIdx ][ uIdx ][ vIdx ][ i ][ c ] specifies the remainder of the residual for the colour mapping coefficient with index [ yIdx ][ uIdx ][ vIdx ][ i ][ c ]. The number of bits used to code res_coeff_r is equal to CMResLSBits. If CMResLSBits is equal to 0, then res_coeff_r is not present. When not present, the value of res_coeff_r is inferred to be equal to 0.

res_coeff_s [ yIdx ][ uIdx ][ vIdx ][ pIdx ][ cIdx ] specifies the sign of the residual for the colour mapping coefficient with index [ yIdx ][ uIdx ][ vIdx ][ i ][ c ]. When not present, the value of res_coeff_s is inferred to be equal to 0.

The variables CMResY[ yIdx ][ uIdx ][ vIdx ][ i ], CMResU[ yIdx ][ uIdx ][ vIdx ][ i ] and CMResV[ yIdx ][ uIdx ][ vIdx ][ i ] are derived as follows:

CMResY[ yIdx ][ uIdx ][ vIdx ][ i ] = ( 1 − 2 * res_coeff_s[ yIdx ][ uIdx ][ vIdx ][ i ][ 0 ]) * ( ( res_coeff_q[ yIdx ][ uIdx ][ vIdx ][ i ][ 0 ] << 7 CMResLSBits) + res_coeff_r[ yIdx ][ uIdx ][ vIdx ][ i ][ 0 ])

CMResU[ yIdx ][ uIdx ][ vIdx ][ i ] = ( 1 − 2 * res_coeff_s[ yIdx ][ uIdx ][ vIdx ][ i ][ 1 ]) * ( ( res_coeff_q [ yIdx ][ uIdx ][ vIdx ][ i][ 1 ] << 7 CMResLSBits ) + res_coeff_r[ yIdx ][ uIdx ][ vIdx ][ i ][ 1 ])

CMResV[ yIdx ][ uIdx ][ vIdx ][ i ] = ( 1 − 2 * res_coeff_s[ yIdx ][ uIdx ][ vIdx ][ i ][ 2 ]) * ( ( res_coeff_q[ yIdx ][ uIdx ][ vIdx ][ i ][ 2 ] << 7 CMResLSBits) + res_coeff_r[ yIdx ][ uIdx ][ vIdx ][ i ][ 2 ])

# Performance evaluation

Table 4 shows the average BD rates and 3D LUT signaling overhead (CGS bits) of the proposed method compared to JCTVC-R0151. Although the overall BD performance is not changed, the number of bits used to code 3D LUT is reduced by 8-9% for all test cases. Table 5 gives detailed comparison of 3D LUT signaling bits using the proposed method and JCTVC-R0151.

Table 4. Performance of the proposed method compared to JCTVC-R0151 as anchors

Table 5. 3D LUT signaling overhead of the proposed method

|  | Total # of 3D LUT bits |  | % reduction |
| --- | --- | --- | --- |
|  | R0151 | proposed |  |
| AI 1x | 4438948 | 4048276 | -9% |
| AI 2x | 4228036 | 3905824 | -8% |
| RA 1x | 1768084 | 1610996 | -9% |
| RA 2x | 1594332 | 1466554 | -8% |
| All cases | 12029400 | 11031650 | -8% |

# Conclusions

This contribution proposes a small change to the entropy coding method in JCTVC-R0151 to reduce 3D LUT signaling overhead.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, “High efficiency video coding (HEVC) scalable extension Draft 6”, JCTVC-Q1008, Apr. 2014.

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 6 (SHM 6)”, JCTVC-Q1007, Apr. 2014.

- X. Li et al, “Non-SCE1: Improved CGS partitioning on top of SHM-6”, JCTVC-R0151, June 2014.