| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0306 |
| --- | --- |

| Title: | SCCE3 base Software Bugfix Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu1, Joel Sole1, Rajan Joshi1, Feng Zou1 Tzu-Der Chuang2, Yu-Chen Sun2, Yuwen He3, Xiaoyu Xiu3, Yan Ye3, Xingyu Zhang4, Robert Cohen4 1Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA 2MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 3 InterDigital Communications, LLC 9710 Scranton Rd, #250 San Diego, CA 92121 4 Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A | Tel: Email: | 1-858-651-7749 wpu@qti.qualcomm.com +886-3-5670766 ext. 24602 { peter.chuang, yc.sun} @mediatek.com |
| Source: | Qualcomm Incorporated, MediaTek Inc., InterDigital Communications, LLC, Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

In Valencia meeting, SCCE3 was established to study palette related coding tools. Palette mode was implemented on top of SCM1.0 as a basis for SCCE3. This proposal reports the reported bugs and fixes of this software.

# Technical Description

During software releases for SCCE3 experiments, some bugs in the base software were identified. They are specified as below.

A) (Qualcomm [3], MediaTek[4], Merl [5], InterDigital [6]): According to palette mode syntax description below, if a CU is palette mode, the following syntax elements such as part_mode should be bypassed. However, in the encoder function xEncIntraHeader, this logic was not correctly implemented.

| … |  |
| --- | --- |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| … |  |

BugFix: conditional return was inserted into xEncIntraHeader as below:

if (pcCU->getPLTModeFlag(0)) return;

B) (InterDitigal [6]) When counting the palette mode bits, the CABAC encoder was not properly reset.

BugFix: two lines of codes were inserted into PLTSearch function as below:

m_pcRDGoOnSbacCoder->load(m_pppcRDSbacCoder[uiDepth][CI_CURR_BEST]);

m_pcRDGoOnSbacCoder->store(m_pppcRDSbacCoder[uiDepth][CI_TEMP_BEST]);

C) (MERL [5]) According to the working draft below, palette_mode_flag is not signaled in intraBC mode. However, in xAddSymbolBitsInter function, palette_mode_flag was counted even if the current mode is intraBC.

| If( intra_block_copy_enabled_flag ) |  |
| --- | --- |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |

BugFix: removing the related code as below:

if (pcCU->isIntraBC(0)) // In the other case, encodePredMode does it

{

m_pcEntropyCoder->encodePLTModeInfo(pcCU, 0, true);

}

D) (Qualcomm, InterDigital [6]) According to the working draft below, the syntax element palette_num_signalled_entries is only signalled when the numPredPreviousPalette is less than the max_palette_size. However, this conditional check was not impelemented in the software.

| if( numPredPreviousPalette < max_palette_size) |  |
| --- | --- |
| palette_num_signalled_entries | ae(v) |

BugFix: In parsePLTModeSyntax and codePLTModeSyntax, one line of condition check is inserted, respectively, as follows:

if ( uiNumPLTPredicted < MAX_PLT_SIZE )

Bugfix A, B, and C are encoder only, under the macro of PLT_RDO_FIX in the attached software. Bugfix D is under the macro PLT_BUGFIX.

# Reference

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of screen content core experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “AHG10: Suggested software for palette coding based on RExt6.0,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0094, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- W. Pu, V. Seregin, R. Joshi, M. Karczewicz, and J. Sole, “SCCE3: Test C.4 – Combination of Tests A.3, A.7, A.8, B.12 and B.13,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0067, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

- T.-D. Chuang, Y.-C. Sun, P. Lai, S.-T. Hsiang, X. Zhang, Y.-W. Chen, Y.-W. Huang, and S. Lei, “SCCE3 Test C.1: Combination of SCCE3 Test A.2 and Test B.1,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0122, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

- X. Zhang and R. Cohen, “SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0200, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

- C.-M. Tsai, Y. He, X. Xiu, and Y. Ye, “SCCE3 Test B.10: Palette index value mapping,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0169, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).