| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0309 |
| --- | --- |

| Title: | Non-SCCE1: Combination of JCTVC-R0185 and JCTVC-R0203 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Ying Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Xiaozhong Xu, Shan Liu, Tzu-Der Chuang, Shawmin Lei Lihua Zhu, Jizheng Xu, Gary J. Sullivan | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com +886-3-5670766 ext. 25555 xiaozhong.xu@mediatek.com lizhu@microsoft.com jzxu@microsoft.com |
| Source: | Qualcomm Inc., MediaTek, Microsoft Corp. |  |  |

_____________________________

# Abstract

This proposal reports the combination results of JCTVC-R0185 and JCTVC-R0203 on block vector prediction method for Intra block copy. In the combination method, the spatial neighboring blocks as in JCTVC-R0185 and the default vectors as in JCTVC-R0203 are selected to form a block vector candidate list and one block vector is chosen as the predictor, similar as in HEVC version 1 AMVP. The performance is reported to be an average BD-rate of –9.8% and –10.8%, respectively for 1080p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Introduction

In current SCM, the last decoded block vector (BV) is used as BV predictor for one PU with Intra block copy (Intra BC) mode except that the PU is the first Intra BC PU in one CTU. For the first Intra BC PU in one CTU, its BV predictor is (-CU_Width, 0). Then, the resulting block vector difference (BVD) is coded by sharing the motion vector difference (MVD) coding method in HEVC. It is noted that only a single BV predictor is formed in order to create the MVD.

# Combination

Figure 1 Spatial neighboring blocks in HEVC AMVP scheme

This contribution proposes the combination of JCTVC-R0185 [1] and JCTVC-R0203 [2] on BV prediction method for Intra block copy. For JCTVC-R0203, the default vectors are chosen from the last 2 BVs in SCCE1 Test 3.2 and the constant BVs ((-2*CU_width, 0) and (-CU_width, 0)) in SCCE1 Test 3.4. In the proposed method, the BV predictor is derived similar to the AMVP scheme in HEVC. The predictor candidate list is constructed by first accessing in order the spatial neighboring blocks a1 and b1 as shown in Figure 1. If not both of the spatial neighbors contain block vectors, the last N coded BVs, which are initialized with (-2*CU_width, 0) and (-CU_width, 0), are used to fill into the block vector candidate list to make it contain two different entries. If N = 1, the first default vector will be the last coded BV, and the second default vector will be the preset constant ((-2*CU_width, 0) in this case). To avoid the line buffer, the above BV out of current CTU is considered unavailable.

# Results

In this section, the experimental results of the proposed method are presented, and BD-rates are provided for the lossy configuration and rate savings for the lossless configuration. The method is tested on top of SCM1.0. SCC common test conditions [3] are used. The following three tests are conducted with different encoder optimization techniques. The encoding/decoding time is not reliable.

- Test 1 : lossy results and lossless results with BV cost check at all possible search positions which has been reported by Canon in SCCE1 Test 3.3, N = 1

- Test 2 : lossy results and lossless results with BV cost check at all possible search positions and full RD check at BV predictor position which has been reported by Canon in SCCE1 Test 3.3, N = 1

- Test 3 : lossy results and lossless results with BV cost check at all possible search positions which has been reported by Canon in SCCE1 Test 3.3, N = 2

- Test 4 : lossy results and lossless results with BV cost check at all possible search positions and full RD check at BV predictor position which has been reported by Canon in SCCE1 Test 3.3, N = 2

Table 1 Lossy results for Test 1

Table 2 Lossy results for Test 2

Table 3 Lossy results for Test 3

Table 4 Lossy results for Test 4

# References

[1] C. Pang, Y. Chen, J. Sole, and M. Karczewicz, “SCCE1: Test 3.1 – Block vector prediction method for Intra block copy”, JCTVC-R0185, Sapporo, JP, June 2014.

[2] X. Xu, T.-D. Chuang, S. Liu, J. Ye, and S. Lei, “Non-SCCE1: Default vector selection for Intra block copy BV prediction”, JCTVC-R0185, Sapporo, JP, June 2014.

[3] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).