| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0311 |
| --- | --- |

| Title: | Non-SCCE3: Combination of palette tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Xiaoyu Xiu, Yan Ye Rajan Joshi, Vadim Seregin, Wei Pu, Marta Karczewicz, Joel Sole | Tel: Email: Tel. Email | +1-858-210-4819 yuwen.he@interdigital.com +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | InterDigital Communications Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

A combination of palette methods in SCCE3 is tested. Compared with SCM1.0, it is reported that the proposed method provides BD-rate improvements of (22.4%, 21.9%, 21.7%) and (22.5%, 25.3%, 24.5%) for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration with full frame IntraBC search, respectively, and the performance with respect to SCM1.0+SCCE3 software is BD-rate improvements of (13.1%, 12.7%, 12.5%) and (13.4%, 15.0%, 14.8%), respectively, for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration with full frame IntraBC search.

# Technical Description

SCCE3 has studied the palette mode and methods to improve it. It can be seen from the results of various tests in SCCE3 that the proposed methods have a high degree of interaction with each other and the encoder. It has been observed that some methods with good gains on top of the base SCCE3 software have no gain or even loss when tested on the top of combination packages (some C.x tests in SCCE3). As a consequence, the best combination of methods is not necessarily the addition of the best methods.

This proposal combines several methods in SCCE3 with the goal of achieving high performance with limited complexity. The included methods are the following:

- Identified bug-fixes in the SCCE3 base software (JCTVC-R0306)

- PPS signaling of the maximum palette predictor size

- Methods in SCCE3 test C.4 with escape coded as index (JCTVC-R0067)

- Encoder improvements in SCCE3 test C.4 (JCTVC-R0175)

- Palette table sharing from SCCE3 test A.5 (JCTVC-R0166)

- Palette table index coding by 1) reducing the palette index value by comparing to the last compared palette index and 2) assigning index value of 1 in the table to escape colors as in SCCE3 test B.10 (JCTVC-R0169) and redundancy removal

- Adaptive scans (horizontal, vertical and traverse) from SCCE3 test B.9 (JCTVC-R0168)

# Performance

Tables 1 and 2 provide BD-rate performance with respect to SCM1.0 and SCM1.0 + SCCE3, respectively, for all intra configuration using the SCC common test conditions (using full-frame intra block copying). Encoding and decoding times in the accompanying spreadsheets are not reliable since the simulations were run on a heterogeneous cluster.

Table 1. Performance of the proposal versus SCM1.0 (full frame IBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -22.4% | -21.9% | -21.7% |
| RGB, text & graphics with motion,720p | -16.4% | -14.4% | -14.8% |
| RGB, mixed content, 1440p | -7.3% | -6.0% | -6.0% |
| RGB, mixed content, 1080p | -8.2% | -7.8% | -7.5% |
| RGB, Animation, 720p | -0.3% | -0.9% | -1.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -22.5% | -25.3% | -24.5% |
| YUV, text & graphics with motion,720p | -12.0% | -16.8% | -20.9% |
| YUV, mixed content, 1440p | -5.8% | -12.1% | -12.3% |
| YUV, mixed content, 1080p | -7.9% | -13.6% | -13.6% |
| YUV, Animation, 720p | 0.2% | -2.0% | -1.8% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

Table 2. Performance of the proposal versus SCM1.0+SCCE3 palette (full frame IBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -13.1% | -12.7% | -12.5% |
| RGB, text & graphics with motion,720p | -11.3% | -9.8% | -10.0% |
| RGB, mixed content, 1440p | -6.2% | -4.9% | -5.1% |
| RGB, mixed content, 1080p | -6.1% | -5.6% | -5.5% |
| RGB, Animation, 720p | -0.4% | -0.8% | -1.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -13.5% | -15.0% | -14.8% |
| YUV, text & graphics with motion,720p | -7.8% | -10.9% | -13.5% |
| YUV, mixed content, 1440p | -4.9% | -9.9% | -10.3% |
| YUV, mixed content, 1080p | -5.7% | -10.5% | -10.6% |
| YUV, Animation, 720p | 0.1% | -2.0% | -1.8% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

# Conclusions

A high performance combination of palette methods is identified. This proposed combination is suitable for adoption in SCM to enhance the performance of SCC and facilitate further improvements of the palette mode.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).