| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0329 |
| --- | --- |

| Title: | Crosscheck of Non-SCCE3 on Combination of Palette Tools (JCTVC-R0311) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of Non-SCCE3 on combination of palette tools. The software package provided by the proponent was tested under the screen content common test conditions specified in JCTVC-Q1015 [1]. All BD-rate numbers produced by using All-Intra conditions match those provided by the proponents. Table 1 and Table 2 show the simulation results on top of SCM-1.0 and SCM-1.0+SCCE3 palette, respectively.

# Test results

Table 1. Simulation results on top of SCM-1.0 (full frame IntraBC).

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -22.4% | -21.9% | -21.7% |
| RGB, text & graphics with motion,720p | -16.4% | -14.4% | -14.8% |
| RGB, mixed content, 1440p | -7.3% | -6.0% | -6.0% |
| RGB, mixed content, 1080p | -8.2% | -7.8% | -7.5% |
| RGB, Animation, 720p | -0.3% | -0.9% | -1.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -22.5% | -25.3% | -24.5% |
| YUV, text & graphics with motion,720p | -12.0% | -16.8% | -20.9% |
| YUV, mixed content, 1440p | -5.8% | -12.1% | -12.3% |
| YUV, mixed content, 1080p | -7.9% | -13.6% | -13.6% |
| YUV, Animation, 720p | 0.2% | -2.0% | -1.8% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

Table 2. Simulation results on top of SCM-1.0+SCCE3 palette (full frame IntraBC).

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -13.1% | -12.7% | -12.5% |
| RGB, text & graphics with motion,720p | -11.3% | -9.8% | -10.0% |
| RGB, mixed content, 1440p | -6.2% | -4.9% | -5.1% |
| RGB, mixed content, 1080p | -6.1% | -5.6% | -5.5% |
| RGB, Animation, 720p | -0.4% | -0.8% | -1.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -13.5% | -15.0% | -14.8% |
| YUV, text & graphics with motion,720p | -7.8% | -10.9% | -13.5% |
| YUV, mixed content, 1440p | -4.9% | -9.9% | -10.3% |
| YUV, mixed content, 1080p | -5.7% | -10.5% | -10.6% |
| YUV, Animation, 720p | 0.1% | -2.0% | -1.8% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Mar. 2014.