| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014 | Document: JCTVC-R0331 M34401 |
| --- | --- |

| Title: | Non-SCCE3: Cross-verification of Improvements for SCCE3 Test C3 (JCTVC-R0304) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA 92127 USA | Email: | minhua@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This document reports cross-check results for JCTVC-R0304 which provides additional results on the top of SCCE3 Test C3 by combining other tools and simplifying the index coding. JCTVC-R0304 has combined the color table derivation from JCTVC-R0167, the escape pixel coding from JCTVC-Q0094 and transition copy for index coding, and simplified the index coding by restricting the distance of matched pairs to be 1 or CU width only and fixing the length of the matched pairs to be CU width.  It is asserted and the results match the ones reported by proponents.  The source code is also checked to confirm its consistency with the proposal description.

# Introduction

This document reports cross-check results for the JCTVC-R0304.  On the top of SCCE3 Test C3 reported in JCTVC-R0144, JCTVC-R0304 incorporated the following changes:

- Encoder only technique: the color table derivation uses the method presented in JCTVC-R0167

- Method proposed in JCTVC-Q0094 is used for escape pixel codling

- For index coding, the distance for matched pairs is restricted to be 1 or CU width, the length of matched pairs is fixed to be CU width, and for unmatched pairs transition copy proposed in JCTVC-R0122 is used for improving the index coding.

The following three settings are tested:

- Test A: Hybrid 1-D and 2-D string search for index map coding, with 2-D search limited to 3 left CTUs and the current CTU. Local IBC of 2CTUs is used.

- Test B: 1-D string search for index map coding within current CU, 2-D string search is disabled. Local IBC of 2CTUs is used.

- Test C: Hybrid 1-D and 2-D string search for index map coding, with 2-D search limited 4x2 CTU window.  Local IBC of 4x2 CTUs is used.

# Test Settings and Conditions

The simulations of this document have used software released by Huawei, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [2] are followed.

# Experimental results

The experimental results are summarized in Table 1 to Table 3 for lossy coding, and Table 4 to Table 6 for lossless coding, respectively. The anchor is SCM1 in which full frame IBC is disabled (i.e. use local IBC of 2CTUs). It is verified that those results match what proponents have reported. Please be advised that runtime reported here may not be accurate due to heterogeneous nature of the LSF farm.

Table 1. Experimental results on lossy coding for test A

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -33.8% | -34.2% | -33.7% |
| RGB, text & graphics with motion,720p | -23.3% | -21.8% | -22.0% |
| RGB, mixed content, 1440p | -9.0% | -7.8% | -7.9% |
| RGB, mixed content, 1080p | -12.0% | -12.0% | -11.8% |
| RGB, Animation, 720p | 0.6% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -31.9% | -36.1% | -35.1% |
| YUV, text & graphics with motion,720p | -17.4% | -23.2% | -26.2% |
| YUV, mixed content, 1440p | -7.8% | -16.0% | -15.6% |
| YUV, mixed content, 1080p | -12.2% | -21.1% | -20.8% |
| YUV, Animation, 720p | 0.2% | -0.8% | -0.7% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 139% |  |  |
| Dec Time[%] | 89% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -20.0% | -21.2% | -20.9% |
| RGB, text & graphics with motion,720p | -20.6% | -18.5% | -19.3% |
| RGB, mixed content, 1440p | -9.2% | -7.0% | -7.2% |
| RGB, mixed content, 1080p | -10.7% | -10.4% | -10.3% |
| RGB, Animation, 720p | -0.7% | -1.6% | -1.0% |
| RGB, camera captured, 1080p | 0.2% | 0.5% | 0.3% |
| YUV, text & graphics with motion, 1080p | -19.1% | -24.4% | -23.7% |
| YUV, text & graphics with motion,720p | -15.6% | -21.4% | -25.5% |
| YUV, mixed content, 1440p | -6.8% | -17.2% | -16.9% |
| YUV, mixed content, 1080p | -10.6% | -21.8% | -21.2% |
| YUV, Animation, 720p | 0.4% | -1.3% | -1.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -13.5% | -14.7% | -14.5% |
| RGB, text & graphics with motion,720p | -13.0% | -11.8% | -12.3% |
| RGB, mixed content, 1440p | -8.3% | -6.5% | -6.5% |
| RGB, mixed content, 1080p | -7.4% | -7.6% | -7.1% |
| RGB, Animation, 720p | -0.3% | -0.3% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -11.3% | -16.6% | -16.1% |
| YUV, text & graphics with motion,720p | -8.5% | -13.8% | -17.3% |
| YUV, mixed content, 1440p | -5.6% | -16.4% | -16.0% |
| YUV, mixed content, 1080p | -6.4% | -16.1% | -15.9% |
| YUV, Animation, 720p | 0.1% | -0.6% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 109% |  |  |

Table 2. Experimental results on lossy coding for test B

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -27.8% | -27.8% | -27.4% |
| RGB, text & graphics with motion,720p | -18.6% | -16.9% | -17.2% |
| RGB, mixed content, 1440p | -7.7% | -6.5% | -6.6% |
| RGB, mixed content, 1080p | -9.9% | -9.8% | -9.5% |
| RGB, Animation, 720p | 0.6% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -26.3% | -30.2% | -29.0% |
| YUV, text & graphics with motion,720p | -13.1% | -18.4% | -21.5% |
| YUV, mixed content, 1440p | -6.1% | -13.7% | -13.6% |
| YUV, mixed content, 1080p | -9.4% | -17.6% | -17.2% |
| YUV, Animation, 720p | 0.2% | -0.8% | -0.7% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -16.3% | -17.0% | -16.8% |
| RGB, text & graphics with motion,720p | -17.1% | -14.7% | -15.7% |
| RGB, mixed content, 1440p | -8.4% | -6.3% | -6.5% |
| RGB, mixed content, 1080p | -9.4% | -9.0% | -8.8% |
| RGB, Animation, 720p | -0.7% | -1.6% | -1.0% |
| RGB, camera captured, 1080p | 0.2% | 0.4% | 0.3% |
| YUV, text & graphics with motion, 1080p | -15.8% | -20.6% | -19.9% |
| YUV, text & graphics with motion,720p | -12.1% | -17.4% | -21.3% |
| YUV, mixed content, 1440p | -5.5% | -15.5% | -15.2% |
| YUV, mixed content, 1080p | -8.6% | -18.9% | -18.4% |
| YUV, Animation, 720p | 0.4% | -1.3% | -1.2% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -10.8% | -11.7% | -11.5% |
| RGB, text & graphics with motion,720p | -12.0% | -10.6% | -11.1% |
| RGB, mixed content, 1440p | -7.7% | -5.8% | -5.9% |
| RGB, mixed content, 1080p | -6.5% | -6.9% | -6.4% |
| RGB, Animation, 720p | -0.2% | -0.4% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -9.1% | -13.9% | -13.3% |
| YUV, text & graphics with motion,720p | -7.5% | -12.5% | -16.1% |
| YUV, mixed content, 1440p | -4.9% | -15.0% | -14.6% |
| YUV, mixed content, 1080p | -5.4% | -14.1% | -14.0% |
| YUV, Animation, 720p | 0.1% | -0.6% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 108% |  |  |

Table 3. Experimental results on lossy coding for test C

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -41.2% | -41.6% | -41.2% |
| RGB, text & graphics with motion,720p | -27.3% | -25.9% | -26.0% |
| RGB, mixed content, 1440p | -15.0% | -14.2% | -14.2% |
| RGB, mixed content, 1080p | -19.8% | -20.2% | -19.4% |
| RGB, Animation, 720p | 0.4% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -40.6% | -43.7% | -42.7% |
| YUV, text & graphics with motion,720p | -22.8% | -28.0% | -30.9% |
| YUV, mixed content, 1440p | -15.1% | -21.0% | -20.8% |
| YUV, mixed content, 1080p | -21.2% | -27.9% | -28.0% |
| YUV, Animation, 720p | 0.1% | -1.1% | -1.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 229% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -25.7% | -26.5% | -26.3% |
| RGB, text & graphics with motion,720p | -22.9% | -21.0% | -21.6% |
| RGB, mixed content, 1440p | -12.5% | -10.9% | -10.9% |
| RGB, mixed content, 1080p | -15.5% | -15.6% | -15.0% |
| RGB, Animation, 720p | -0.8% | -1.7% | -1.0% |
| RGB, camera captured, 1080p | 0.2% | 0.4% | 0.3% |
| YUV, text & graphics with motion, 1080p | -24.9% | -29.1% | -28.4% |
| YUV, text & graphics with motion,720p | -18.8% | -24.0% | -27.8% |
| YUV, mixed content, 1440p | -11.3% | -19.5% | -19.1% |
| YUV, mixed content, 1080p | -16.5% | -24.8% | -24.9% |
| YUV, Animation, 720p | 0.4% | -1.5% | -1.4% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 116% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -18.9% | -19.8% | -19.6% |
| RGB, text & graphics with motion,720p | -14.5% | -13.1% | -13.6% |
| RGB, mixed content, 1440p | -9.6% | -8.2% | -8.0% |
| RGB, mixed content, 1080p | -9.2% | -9.3% | -8.5% |
| RGB, Animation, 720p | -0.3% | -0.4% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -16.6% | -20.9% | -20.5% |
| YUV, text & graphics with motion,720p | -10.0% | -14.4% | -17.5% |
| YUV, mixed content, 1440p | -7.4% | -16.0% | -15.3% |
| YUV, mixed content, 1080p | -7.9% | -13.9% | -14.2% |
| YUV, Animation, 720p | 0.1% | -0.6% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 106% |  |  |

Table 4. Experimental results on lossless coding for test A

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 32.7% | 31.7% | 19.9% | 46.8% |
| RGB, text & graphics with motion,720p | 18.8% | 18.7% | 2.3% | 31.3% |
| RGB, mixed content, 1440p | 6.5% | 6.6% | 6.1% | 7.1% |
| RGB, mixed content, 1080p | 5.4% | 5.4% | 5.4% | 5.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 38.2% | 37.0% | 19.4% | 53.0% |
| YUV, text & graphics with motion,720p | 18.5% | 19.9% | 2.4% | 39.4% |
| YUV, mixed content, 1440p | 4.2% | 4.7% | 1.8% | 7.6% |
| YUV, mixed content, 1080p | 4.5% | 4.5% | 4.5% | 4.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 174% |  |  |  |
| Dec Time[%] | 87% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 13.3% | 19.4% | 10.3% | 35.0% |
| RGB, text & graphics with motion,720p | 4.2% | 10.3% | 1.2% | 25.7% |
| RGB, mixed content, 1440p | 1.1% | 1.1% | 0.9% | 1.3% |
| RGB, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 14.3% | 24.6% | 12.0% | 40.5% |
| YUV, text & graphics with motion,720p | 3.5% | 11.7% | 1.2% | 32.9% |
| YUV, mixed content, 1440p | 0.7% | 0.6% | 0.5% | 0.8% |
| YUV, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  |
| Dec Time[%] | 105% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.8% | 14.1% | 7.7% | 22.6% |
| RGB, text & graphics with motion,720p | 2.9% | 5.9% | 0.9% | 12.3% |
| RGB, mixed content, 1440p | 0.4% | 0.4% | 0.4% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.8% | 18.7% | 10.6% | 27.4% |
| YUV, text & graphics with motion,720p | 2.1% | 6.6% | 0.8% | 16.5% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |  |
| Dec Time[%] | 109% |  |  |  |

Table 5. Experimental results on lossless coding for test B

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 26.7% | 26.1% | 16.6% | 37.3% |
| RGB, text & graphics with motion,720p | 17.6% | 16.5% | 2.2% | 27.8% |
| RGB, mixed content, 1440p | 5.9% | 5.9% | 5.8% | 5.9% |
| RGB, mixed content, 1080p | 4.2% | 4.2% | 4.2% | 4.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 32.4% | 31.6% | 15.8% | 44.0% |
| YUV, text & graphics with motion,720p | 17.0% | 17.5% | 2.2% | 31.7% |
| YUV, mixed content, 1440p | 3.4% | 3.8% | 1.6% | 6.0% |
| YUV, mixed content, 1080p | 3.5% | 3.5% | 3.5% | 3.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 148% |  |  |  |
| Dec Time[%] | 85% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 10.7% | 15.4% | 8.5% | 27.5% |
| RGB, text & graphics with motion,720p | 4.0% | 8.8% | 1.1% | 20.3% |
| RGB, mixed content, 1440p | 1.0% | 1.0% | 0.8% | 1.3% |
| RGB, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.5% | 20.7% | 9.3% | 33.3% |
| YUV, text & graphics with motion,720p | 3.3% | 10.1% | 1.1% | 26.8% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.4% | 0.7% |
| YUV, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 109% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 9.3% | 11.1% | 6.3% | 17.5% |
| RGB, text & graphics with motion,720p | 2.9% | 5.6% | 0.9% | 11.2% |
| RGB, mixed content, 1440p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 9.3% | 15.7% | 7.9% | 22.3% |
| YUV, text & graphics with motion,720p | 2.1% | 6.2% | 0.8% | 15.1% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 108% |  |  |  |

Table 6. Experimental results on lossless coding for test C

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 38.4% | 37.1% | 28.8% | 51.5% |
| RGB, text & graphics with motion,720p | 20.0% | 20.2% | 2.7% | 34.3% |
| RGB, mixed content, 1440p | 8.0% | 8.3% | 7.3% | 9.3% |
| RGB, mixed content, 1080p | 8.5% | 8.5% | 8.5% | 8.5% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 43.6% | 42.2% | 28.5% | 57.3% |
| YUV, text & graphics with motion,720p | 19.8% | 21.5% | 2.8% | 42.6% |
| YUV, mixed content, 1440p | 6.0% | 6.6% | 3.2% | 10.0% |
| YUV, mixed content, 1080p | 8.4% | 8.4% | 8.4% | 8.4% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 282% |  |  |  |
| Dec Time[%] | 85% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 21.5% | 24.8% | 12.1% | 40.3% |
| RGB, text & graphics with motion,720p | 4.7% | 11.2% | 1.4% | 28.2% |
| RGB, mixed content, 1440p | 1.8% | 1.8% | 1.1% | 2.4% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 22.3% | 29.8% | 21.2% | 45.5% |
| YUV, text & graphics with motion,720p | 4.0% | 12.7% | 1.3% | 35.7% |
| YUV, mixed content, 1440p | 1.3% | 1.4% | 1.0% | 1.7% |
| YUV, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 124% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 20.6% | 20.0% | 9.4% | 29.0% |
| RGB, text & graphics with motion,720p | 3.3% | 6.5% | 1.0% | 13.8% |
| RGB, mixed content, 1440p | 1.0% | 1.0% | 0.5% | 1.6% |
| RGB, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 20.6% | 24.5% | 19.7% | 33.3% |
| YUV, text & graphics with motion,720p | 2.5% | 7.1% | 0.9% | 18.1% |
| YUV, mixed content, 1440p | 0.9% | 0.9% | 0.3% | 1.6% |
| YUV, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 116% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# Comments

Palette coding appears to be another efficient tool for screen content coding but adds significant implementation cost.  The simplification of color index coding proposed in JCTVC-R0304, especially fixing the length of matched pairs to be CU width, is desirable from the implementation point of view for the simplicity of memory access.

# References

[1]  H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”,  JCT-VC Document, JCTVC-Q1015, Valencia, ES, April 2014.

[2] Z. Ma, W.Wang, M. Xu, F. Duanmu, H. Yu, “Non-SCCE3: Improvements for SCCE3 Test C3”, JCT-VC Document, JCTVC-R0304, Sapporo, JP, 30 June – 9 July, 2014

# Patent rights declaration(s)

Broadcom Corporation does not have current or pending patent rights relating to the technology described in this contribution.