| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0332 |
| --- | --- |

| Title: | Non-SCCE3: Cross-check of JCTVC-R0232 on high throughput palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Rajan Joshi Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-845-1115 fzou@qti.qualcomm.com rajanj@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the cross checking results of JCTVC-R0232, non-SCCE3 high throughput palette coding. It is confirmed that the simulation results match those reported by the proponents.

# Background of JCTVC-R0232

In JCTVC-R0232, to reduce the context coded bins, the number of context coded bins for palette_run syntax is counted.When the number of context coded bins exceeds a predetermined threshold, the context coded bins are replaced by bypass bins. It is asserted that the context coded bins per pixel is reduced from 1.984375 bins/pixel to 1.5625 bins/pixel in the worst case.

# Analysis of worst-case JCTVC-R0232

In the HEVC design, during residue coding, for each 4×4 block of transform coefficients, 1.5625 bins/sample are context-coded. This number is calculated per component. In palette coding, the three components are processed together. The current SCCE3 palette method uses 1.984375 bins/pixel in the worst case, which is equivalent to 1.984375/3 = 0.661458 bins/sample (assuming 4:4:4 color format).

# Test Condition

SCCE3 test condition [1] is used. The software is provided by the proponent [2].

# Results

Table 1 shows the BD-rate results for all intra configuration with respect to the SCCE3 AHG anchor. The results matched those provided by the proponents.

Table 1. Cross checking results v.s. SCCE3 base SW as anchor (Full frame IBC, lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 109% |  |  |

For complete cross checking results, please refer to the attached spreadsheets.

# Reference

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of screen content core experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- S.-H. Kim, K. Misra, J. Zhao, and A. Segall, “None-SCCE3:High throughpit palette coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0232, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).