| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0336 |
| --- | --- |

| Title: | BoG on 1D dictionary |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ying Chen Jizheng Xu | Tel: Email: | cheny@qti.qualcomm.com jzxu@microsoft.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on SCCE proposals and in general dictionary coding methods.

# Introduction

A BoG was held on Monday July 1, 2014, 11:00 am to 1:00 pm to provide more detailed information of the input documents related to SCCE 4: string matching for sample coding. The agreed mandates of the BoG were:

- Establish preliminary methodology to evaluate the memory access bandwidth in the worst case scenario.

- Evaluate the additional decoder memory required for the CE proposals as well as the decoding processes to reorganize the memory buffer as well as other overheads, related to e.g., length/run mapping table, color table and CABAC throughput.

- Discuss potential benefits on constraining the search range of 1D dictionary.

- Establish a method and discuss software implementation of it to collect the average case memory access bandwidth.

- Identify key technical aspects that may be further studied in CE.

A second BoG was held on Monday July 1, 2014, 9:00 am to 11:00 am to further discuss the method of complexity analysis and discuss CE plan for SCCE4.

# Discussed topics

## Worst case memory access bandwidth

It sounds desirable for a dictionary coding method (if acceptable as part of SCC) to have a similar level of worst case memory access bandwidth as in current SCC.

It is likely that all-Intra (AI) and random access (RA) & Low delay B (LB) may have different memory bandwidth requirement in the current SCC design.

### Worst case memory bandwidth in SCC

For simplicity, the memory bandwidth for one color component is considered.

The general equation of the worst case memory bandwidth access is shown below:

Here, a block size is M×N, the interpolation filter length L and the memory block is m×n, and P is the average number of pixels needs to be accessed for each pixel of one component.

AI case:

Typical values of P are shown as below, for m×n equal to 1x1, 4x2, 8x2 and 4x4 respectively, assuming Intra BC applies for only uni-directional prediction.

| mxn | 1x1 | 4x2 | 8x2 | 4x4 |
| --- | --- | --- | --- | --- |
| P | 1 | 1.875 | 2.5 | 2.25 |

RA/LB case:

Typical values of P are shown as below, for m×n equal to 1x1, 4x2, 8x2 and 4x4 respectively, assuming bi-directional prediction 8x8 block is the worst case.

| mxn | 1x1 | 4x2 | 8x2 | 4x4 |
| --- | --- | --- | --- | --- |
| P | 4 | 10 | 12 | 12.5 |

Further study is encouraged to consider three components jointly. An excel document to be provided for worst case memory bandwideth dictionary coding: we need volunteer to take actions on these items and share the work with other CE participants. Open question hopefully can get answered by the end of this meeting.

### Worst case memory access bandwidth for dictionary coding

#### Assumptions

In order to compare the dictionary coding methods with the current design, we make the following assumptions. More assumptions can be added if agreed later.

- Each fetch of memory contains S pixel values in the reference memory and S is typically larger than 1.

- Fetching of S pixel has the same price as fetching mxn pixels if S (e.g., 16) is equal to mxn (e.g., 4x4).

It is commented that mxn fetching might be more expensive since it can include n rows, and each row might need to be accessed one by one. It is also noticed that by accessing S pixels in case of 2D matching may also include several rows.

- Even each fetch has a target of 1 pixel, the number of accessed pixels is equal to S.

Note that other parameters or conditions may be considered later for the calculation of the worst case memory bandwidth.

AI case:

Typical values of P are shown as below for dictionary coding methods, assuming the minimal number of runs is equal to T. Here MxN equal to 8x8 represents the block size that hits the worst case. Each memory fetch is contains S samples (similar to the mxn memory access pattern in case of motion compensation).

Note that this means typically P is equal to S/T.

| S, T | 1, 1 | 2, 1 | 4, 1 | 8, 1 | 16, 1 |
| --- | --- | --- | --- | --- | --- |
| P | 1 | 2 | 4 | 8 | 16 |

| S, T | 2, 2 | 4, 2 | 8, 2 | 16, 2 | n/a |
| --- | --- | --- | --- | --- | --- |
| P | 1 | 2 | 4 | 8 | n/a |

| S, T | 4, 4 | 8, 4 | 16, 4 | n/a | n/a |
| --- | --- | --- | --- | --- | --- |
| P | 1 | 2 | 4 | n/a | n/a |

It seems that 1D dictionary does not significantly increase the worst case memory bandwidth with certain constraints.

It was commented that the way of calculating the memory access might not cover the real worst case scenario.

- In terms of 2D matching, the assumption of 1D memory access might not be appropriate. Therefore, it might be good to still assume 2D memory access pattern, such as mxn memory access even for 1 or 2 pixels.

- In terms of 1D matching, as discussed in other proposals, either additional decoder memory or different memory accessing mechanism can be introduced, so that the above assumption (each fetch takes at least S pixels) can still apply.

In case of 2D-matching, assuming mxn memory patten, the following equation can be used for now.

Examples are as follows.

| mxn | 1x1 | 4x2 | 8x2 | 4x4 |
| --- | --- | --- | --- | --- |
| P | 1 | 8 | 16 | 16 |

Therefore, currently, the worst case memory bandwidth assess is max (m.n, S/T), which is typically mxn.

Besides constraining the minimum length of runs, the following constraints may be considered (as in JCTVC-R0060.

- Constrain the maximum number of runs per CU, assuming it is nR. The worst case memory bandwidth access becomes, equation to be provided. Ceil(nR.m.n/MxN). For example, if nR is 16 and mxn is 2x8, the average per pixel memory access is 4. When a run contains pixels that cannot be covered by a mxn block, this still needs refinement. However, larger blocks leads to less memory access. So the worst case number as suggested in the above equation might still be approximately correct. Note that this might force the encoder to stop 2D matching runs, but code escape pixels explicitly. So a coding efficiency trade-off is involved here.

## Additional decoder memory and decoding processes

If the dictionary coding is based on the pixels within the same CU, what

| Proposal | Additional decoder Buffer (pixels) | Additional computations for 1D matching | Mapping table | Frequent color table |
| --- | --- | --- | --- | --- |
| R0098/3.1 | 1M | No | N\A | N\A |
| R0140/3.2 | 0 | No | N\A | N\A |
| R0060/3.3 | 4K/3~4M/3 | No | One table has a size equal to the maximum value of length (12288) One table has a size equal to the maximum value of offset (4K to 4M) | N\A |
| R0145/3.4 | 0 | 1D matching is performed within the current CU. Depending on implementation, if no additional 1D buffer, potentially multiple accesses for one run might be needed. If the pixels are organized in the decoding order of the CUs, the additional addressing might not be needed. It seems more discussions are needed for this aspect. | N\A | 128 entries in the table, each contains three sample values. The table can be updated once an escape pixel is explicitly coded. |

| Proposal | CABAC throughput |
| --- | --- |
| R0098/3.1 | 2D MV is coded the same as MVD in Intra BC. 1D offset is coded by a prefix plus fix-length LSB: with 4 context coded bins. |
| R0140/3.2 | 2D MV is coded the same as MVD in Intra BC. |
| R0060/3.3 | 20 bins maximum for offset. About 18 bins maximum for run length. Each bin is coded with CABAC context |
| R0145/3.4 | Fixed length binarization, 12 bins maximum. Offset has 14 bins maximum. Each bin is coded with CABAC context. |
| R0137 | For 1D/2D length, up to 10 bins, context coded; The offset has 15 bins. Some of them share the context so up to 7 different contexts are used. For 1D offset, 15 context coded bins with 9 contexts. For 2D offset, 14 context coded bins with 10 contexts for MV. |

It is commented that if the number of context coded bins for a run/match is large, the total number of runs may become limited, therefore the total number of context coded bins for the whole CU might not be an extremely big value.

## Constraining the search range of dictionary coding

If the dictionary coding is based on the pixels within the same CU

| Proposal | Constraints of the search range |
| --- | --- |
| R0098/3.1 | No |
| R0140/3.2 | 2 CTU (the current and the left) |
| R0060/3.3 | Configurable |
| R0145/3.4 | 1D: same CU. 2D: 4 CTU (the current and three left ones) |

To avoid confusion, it is suggested to change the term “string matching” or “1D dictionary coding” to just “intra string copy”. Note that in “intra string copy”, we have “1D matching mode” and “2D matching mode”.

### Different level of memory accesses

Current worst case memory bandwidth is calculated by considering each match is from off-chip memory. However, closer regions might be easier to access.

If pixels of a region have been considered to be put on chip, what is the cost of the memory access? It is commented that memory access from such region may be lightweight. It is also commented that even it is on chip memory, the access is not free.

It is noted that in the context of SCCE1 (Intra BC), accessing such region is considered as inexpensive, however the external memory bandwidth within such region is not considered as zero. It is commented that in SCCE1, the read operation may still be considered as off-chip always in terms of memory bandwidth operation.

The constrained region is SCCE1 has the following configurations:

- 2 CTU as in range extension development;

- 4 CTU (with three left ones)

- 4 CTU plus four up rows (of the current CU).

In the context of this CE, it seems desirable to evaluate the method in cases of cases of both full frame and limited region. It sounds desirable to align the same constraint of the “intra string copy” as other methods, especially Intra BC. Examples are:

- Full frame Intra BC with full frame “intra string copy”

- 2 CTU Intra BC with 2 CTU “intra string copy”

## Establish a method and discuss software implementation of it to collect the average case memory access bandwidth.

It sounds the software used for Intra BC can be modified to serve the same purpose as in “intra string copy”. We need volunteers. Open question hopefully can get answer by the end of this meeting.

## Key technical aspects that may be further studied in CE

Information as copied from the CE report.

Table 1 Comparison of the key technical aspects of the CE proposals (1/2)

| Proposal | Scan | Min. run | Matching modes | Search Range | Res. Coding | Additional dec. Buffer | Lossy enc. Match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0098/3.1 | V&H | 1 pixel | 1D/2D | Full frame | No | 1M pixels | No |
| R0140/3.2 | H | 1 pixel | 2D | 2 CTU | No | None | No |
| R0060/3.3 | H | 1 sample | 1D | Full frame | No | 4K/3~4M/3 pixels | Yes |
| R0145/3.4 | H | 1 pixel | 1D/2D | 3 or 4?CTU | Yes | 3x64x64 pixels | No |

Table 2 Comparison of the key technical aspects of the CE proposals (2/2)

| Proposal | Enc. Hash Table Size (Bytes) | Eff. Highlight | Enc. Time AI, Other | Dec. time | Miscellaneous |
| --- | --- | --- | --- | --- | --- |
| R0098/3.1 | 160 M B (1080p) | -21.3% | 150%, 110% | 100% | 2D matching from pixels outside the current CU |
| R0140/3.2 | 64 K (2 CTUs) | -11.7% | 190%, 115% | 100% | 2D matching from any decoded pixels of the current CU |
| R0060/3.3 | 16K~ 16M (configurable) | -14.1% | 140%, 110% | 105% | Mapped table for offset/length coding |
| R0145/3.4 | 144 K (3 CTUs) | -11.4% | 105%, 105% | 120%~180% | Adaptive color table for escape pixel |

### Topics as in the proposals

#### Scan directions (Test 1)

It sounds majority of the proposals use horizontal scan, the benefit of adaptive vertical and horizontal scans can be tested (JCTVC-R0098).

#### Matching modes (Test 2)

It is agreed that 2D matching is less complex than 1D matching. The benefit of having 1D matching should be tested and the complexity analysis of 1D matching in terms of memory bandwidth access as well as decoder buffer requirement need to be provided (JCTVC-R0098, JCTVC-R0060).

#### Residue coding (Test 3)

The benefit of utilizing residue coding will be tested in CE and can be further tested together different constraints (JCTVC-R0145).

#### Mapping table for offset/length coding (Test 4)

It is found that the matching string length has unusual characteristic that many special and large values such as multiples of 3 and multiples of the current CU width occur much more frequently than many other small values. Based on the characteristics, a mapping of the string length values to two matching parameters (pos and len) is introduced to improve the coding efficiency for length values (JCTVC-R0060).

#### Frequent color table (Test 5)

A frequent color table is introduced to code escape pixels. There are 128 entries in the table, each contains three sample values. The table can be updated once an escape pixel is explicitly coded (JCTVC-R0145).

#### CABAC throughput (Test 6)

2D matching

Several proposals utilize the same method as in Intra BC. Even that method might have been changed, it is still benifitial to have the same method for 2D matching as in Intra BC.

However, there are methods further improves the efficiency of the CABAC coding.

- Fixed length binarization and individual context for each bin (JCTVC-R0145).

- Universal entropy coding, different context models (likely more complex) used but with binarization similar to HEVC MVD coding (JCTVC-R0137).

1D matching

This needs to be studied together with Test 2.

There are multiple methods for the entropy coding of offset in 1D matching.

- Fixed length binarization and individual context for each bin (JCTVC-R0145).

- Universal entropy coding, different context models (likely more complex) used but with binarization similar to HEVC MVD coding for one component (JCTVC-R0137).

- Unary coded prefix with fixed-length suffix coding (JCTVC-R0098).

Run coding

There are multiple methods for the run coding.

- Universal entropy coding, different context models (likely more complex) used but with binarization similar to HEVC MVD coding for one component (JCTVC-R0137).

- Unary coded prefix with fixed-length suffix coding, similar to offset coding (JCTVC-R0098).

- First bins indicating the value larger than 0,1 or 2 plus Golomb-Rice code (JCTVC-R0140)

- Prefix and suffix coding both use Exp-Golomb (JCTVC-R0060).

It was noticed that similar methods can be used for offset coding and run coding in multiple proposals. So it is encouraged to have combined results for the offset of 1D matching and run (for both 1D and 2D matches), if similar methods apply.

#### Constraints for run length and number of runs (Test 7)

- Minimum number of runs (JCTVC-R0140)

- Maximum number of runs (JCTVC-R0060)

#### Sample matching

It sounds that sample matching may make the design more complex. Given that, shall we still encourage this to be tested in the context of CE. Interesting parties can bring related proposals. No action item as for CE.

### Coordination of the CE on Intra String copy (ISC)

Test condition

- Per pixel matching

- 2D matching

- Full frame IBC with full frame ISC

- 4 CTU IBC with 4 CTU ISC

  - And aligned search ranges for IBC and ISC are encourage to be tested.

- Full frame IBC with 4 CTU ISC: not seem to be necessary at this stage (not required to be tested in CE).

- 4CTU IBC with full frame ISC: not really necessary (not required to not be tested in CE).

- Simple CABAC for the relevant syntax elements.

Test software

It sounds that three main software implementations are available.

- Software package #1 as in JCTVC-R0098: used in addition by JCTVC-R0140, JCTVC-R0255, JCTVC-R0137. The code sounds to be clean as confirmed by multiple companies. If that software can be chosen, some functionalities not in the test condition should be removed. It was also commented that this software package might not be efficient if only 2D matching is used due to the too strict constraint of search range at the encoder.

- Software package #2 as in JCTVC-R0140, JCTVC-R0255: not used in other proposals. Decoder method is the same as software package #1, but with improved 2D matching at the encoder so that pixels within the current CU can be used as reference in case of 2D matching.

- Software package #3 as in JCTVC-R0060: not used in other proposals. It is claimed to have more flexibility in terms of search range.

- Software package #4 as in JCTVC-R0145: not used in other proposals. It sounds this software package is more flexible in terms of changing the search range (for limited CTUs).

Several experts sound to prefer a common software to be exchanged within this CE. Jizheng Xu is responsible to take action to coordinate with others (mainly the proponent of software package #2 and software package #4) to provide a combined software package, with major aspects summarized as follows:

- Software package #1 with removing the 1D matching.

- More efficient encoder as in software package #2.

- 4 CTU constraint as in software package #4, this mechanism should be made more flexible, e.g., as controlled by a configuration item.

# Summary of the proposals

## Tools under investigation

### JCTVC-R0098 from Microsoft

The 1-D dictionary is enabled at the CU level with both horizontal scanning and vertical scans. Per-pixel matching is used for string matching. There are two types of 1-D dictionary mode, line based 1-D matching and shape based 2-D matching.

In the first 1D matching mode, the previous coded pixels using 1-D dictionary mode are kept in the dictionary (which is dynamically updated). In this mode, prediction mode and direct mode are allowed. If a matching is found, an offset (the offset relative to the position of the current pixel in the dictionary) and a matching length are signaled, otherwise the pixel value is signaled directly.  The above two cases are called prediction mode and escape mode respectively.

In the second 2D matching mode, all the previously reconstructed pixels can be used for prediction. Similar to 1D matching mode, 2D matching mode has prediction mode and escape mode. If a matching is found, a 2D MV and a matching length are signaled, otherwise, the pixel value is also signaled directly. Note that in 2D matching, when the current region starts a new row or column, the pixel used for prediction also starts a new row or column, as shown in the following figure. There is no additional memory requirement at the decoder side.

### JCTVC-R0140 from Qualcomm

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and left CTU. The proposed method utilizes horizontal scan only and per-pixel matching. The proposed method utilizes 2D matching mode thus can share the same reconstruction buffer as in intra BC, with up to 2CTU search range. It supports prediction mode and escape mode. The previously coded pixels within the current CU are allowed to be chosen as reference pixels.

### JCTVC-R0060 from Tongji/MediaTek

1D dictionary is enabled at the CU level, but based on sample matching: instead of matching three components of a full pixel, sample matching can take one sample of a pixel and then either stop or go to the next sample of the same pixel. The proposed method utilizes horizontal scan only and is based on 1D matching mode. It supports also prediction and escape mode. In the 1D matching mode, 1-D distance (pos) is used to represent 2-D displacement with the basic unit of sample instead of pixel. The maximum matching string length is extended from 273 to 12288. It is found that the matching string length has unusual characteristic that many special and large values such as multiples of 3 and multiples of the current CU width occur much more frequently than many other small values. Based on the characteristics, a mapping of the string length values to two matching parameters (pos and len) is introduced to improve the coding efficiency for length values. A histogram-based qp-dependent color quantization scheme is used as a pre-processing step of the input pixels to enable lossy match at the encoder side.

### JCTVC-R0145 from Huawei

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and three left CTUs. The proposed method utilizes horizontal scan and per-pixel matching. The 1D string matching can be based on either 1D matching mode or 2D matching mode. Compared to other CE proposals, different approaches for both prediction mode and escape mode are used. In the prediction mode, residual can be signaled with the normal HEVC residue coding. In the escape mode, if the color value can be found in an adaptive color table, the index to it rather than the color value itself is coded, otherwise, the color value is coded and the adaptive color table is updated based on the color value. The adaptive table is derived at the decoder unless if it contains only one entry. In that case, the adaptive table is signaled.

## Non-CE proposals

### JCTVC-R0137 from MediaTek

This contribution develops a method to code new syntax elements employed by string matching coding. The proposed method utilizes a universal entropy coding scheme representing a syntax value by sending its most significant bit (MSB) index and refinement bits. It is reported to achieve 4.8%, 2.9%, and 2.2% average Luma BD-rate savings compared with the results of the SCCE4 Test3.1 for YUV, text & graphics with motion, 1080p sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively, of the screen content coding (SCC) CTCs.

### JCTVC-R0255 from Qualcomm

This proposal presents preliminary results by constraining the current 1D dictionary (string match) methods investigated under the current SCCE4. It is reported that the current 1D dictionary methods may require much more frequent memory accesses from different locations of the reference area compared to e.g., Intra BC. Therefore, it could be beneficial to address more pixels for each string match. In this proposal, specifically, the matching length is constrained to be equal or larger than a predefined threshold T, e.g., 4. The simulation results show that the constraint as implemented based on SCCE4 proposals JCTVC-R0098 and JCTVC-R0140 lead to noticeable bitrate increase.