| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0339 |
| --- | --- |

| Title: | Cross-check of JCTVC-R0112 Non-SCCE3: Bi-color intra mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A | Tel: Email: | +1-617-621-7500 xzhang@merl.com cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document reports results from a crosscheck of JCTVC-R0112, Non-SCCE3: Bi-color intra mode for screen content coding by ITRI. The BD-rate performance is consistent with the results from the proposal. The encoded bit-streams can be correctly decoded by the decoder.

# Introduction

JCTVC-R0112 proposed a bi-color intra mode for screen content coding. A flag at slice level is signalled to enable/disable the proposed mode for every slice. Further a CU level flag is sent to indicate the selection of the bi-color mode for the current CU. The two colors are derived at both the encoder and the decoder side. The index map is coded like in palette mode.

# Experimental Results

The solution is built using GCC 4.4.7. The common test conditions from JCTVC-Q1123 were used for these tests. For lossy configurations, 3 sets of anchor methods and corresponding tested methods are run. For lossless configurations, 2 sets of anchor methods are used. Only all intra conditions are tested. In the software implementation of JCTVC-R0112, the proposed method is enabled by the following macro in the software:

- #define  ITRI_TWO_SAMPLE_MODE        1

## Lossy coding results

### SCM-1.0 common test condition with intraBC using full frame search

Decoding time is not valid.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.0% | -2.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.7% | -2.1% | -1.9% |
| YUV, text & graphics with motion,720p | -0.6% | -0.4% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.8% |
| YUV, mixed content, 1080p | -0.2% | -0.5% | -0.6% |
| YUV, Animation, 720p | 0.1% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 58% |  |  |

### SCM-1.0 common test condition with intraBC using 2CTU search

Decoding time is not valid.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.4% | -2.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.9% | -2.7% | -2.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.4% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.8% |
| YUV, mixed content, 1080p | -0.3% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.1% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 58% |  |  |

### SCM-1.0 common test condition with intraBC using full frame search + Palette

Decoding time is not valid.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.6% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.1% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 57% |  |  |

## Lossless coding results

### SCM-1.0 common test condition with intraBC using full frame search

Runtimes are not valid.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.4% | 0.5% | 0.1% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.1% | 1.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 67% |  |  |  |
| Dec Time[%] | 63% |  |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

Runtimes are not valid.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.1% | 0.2% | 2.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.2% | 0.2% | 2.3% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 66% |  |  |  |
| Dec Time[%] | 65% |  |  |  |

# Conclusions

It is confirmed that all the rate-distortion data generated for the cross-check tests match the results released by the proponents in R0112. The source code seems to match the description in JCTVC-R0112.