| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0340 |
| --- | --- |

| Title: | BoG on open issues in SHVC re-sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E.Alshina | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on open issues in SHVC re-sampling.

# Introduction

A BoG was held on Wednesday July 2, 2014, 17:00 pm to 20:00 pm to clarify open issues mentioned in AhG13 report, verify adequacy of integration to SHVC draft test all agreed during this meeting action related to re-sampling process.

# Discussed topics

## Re-sampling phases settings, range and signaling format

BoG has reviewed and verified derivations provided JCTVC-R0209.

Re-sampling phases used in SHVC spec recommended by AhG13 are

0≤ PhaseL < 32;

0≤PhaseC+8<24+8/S.

Here S is inverse scaling factor.

Re-sampling phases for Luma are not negative and can be signalled directly as ue(v) or as u(5).

Re-sampling phases for Chroma cannot be smaller than 8, so with offset +8 re-sampling phases for Chroma can be signalled as ue(v) (accurate for any S) or as u(6) (accurate for S≥1/5, which corresponds to scaling ratio 5).

Default values for re-sampling phases in JCTVC-R0209 were verified during BoG discussion.

Theoretical ranges, default values and possible variants of signalling are summarized in the Table1.

Table 1. Syntax elements related to re-sampling phases signaling.

| Syntax elements | Theoretical range | Default value | Variants for signaling |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| phase_hor_luma | 0…31 | 0 | ue(v) | u(5) | u(5) |
| phase_ver_luma | 0…31 | 0 | ue(v) | u(5) | u(5) |
| phase_hor_chroma_plus8 | 0…24+8/S | 8 | ue(v) | ue(v) | u(6) |
| phase_ver_chroma_plus8 | 0…24+8/S | int(4/S+4) | ue(v) | ue(v) | u(6) |

Recommendation:

- BoG recommends to use u(5) format for phase_hor_luma and phase_ver_luma and u(6) format for phase_hor_chroma_plus8 and phase_ver_ chroma_plus8.

## Non-4:2:0 format

Only Chroma re-sampling phases are different from Table 1 in reference and current layer use different color format. Recommendations for re-sampling phases ranges and signaling are suitable for non 4:2:0 cases as well.

Table 2. Expected ranges for Chroma re-sampling phases for non 4:2:0 cases.

| Syntax elements | Theoretical range | Default value |
| --- | --- | --- |
| Reference layer 4:2:0, current layer 4:4:4 |  |  |
| phase_hor_chroma_plus8 | 0…16+16/S | 8 |
| phase_ver_chroma_plus8 | 0…16+16/S | int(8/S) |
| Reference layer 4:2:0, current layer 4:2:2 |  |  |
| phase_hor_chroma_plus8 | 0…16+16/S | 8 |
| phase_ver_chroma_plus8 | 0…24+8/S | int(4/S+4) |
| Both reference layer and current layer 4:4:4 |  |  |
| phase_hor_chroma_plus8 | 8…24 | 8 |
| phase_ver_chroma_plus8 | 8…24 | 8 |

It was noticed during BoG discussion that extension of SHVC re-sampling process for non 4:2:0 color format support requires only ScaleFactorX and ScaleFactorY adjustment for Chroma. More changes are needed for extension of color mapping inter-layer processing part of SHVC spec to support of non 4:2:0 color format.

## Dynamic range in re-sampling process.

Re-sampling process requires follows per-sample calculation

xRef16 =(((xP –offsetX)*ScaleFactorX−addX+(1 << 11)) >> 12 ) + RefOffsetX,

yRef16 =(((yP –offsetY)*ScaleFactorY−addY+(1 << 11)) >> 12 ) + RefOffsetY

It is desirable to have no overflow of 32-bits register during this calculation.

It is desirable to have all multiplier within 16 bits.

Multipliers derived based on scaling ratio are ScaleFactorX, ScaleFactorY.

Only if S<1 (S is inverse scaling factor) ScaleFactorX(Y) is within 16 bits.

Recommendation:

- SHVC shall not support S>1 (current layer PicSize not smaller that reference layer PicSize).

- SHVC shall not use re-sampling process with S=1

  - this recommendation is opposite to the meeting notes (Decision: Allow a phase-only adjustment between the RL and EL (with spatial ratio equal to 1), but count this as a resampling operation.)

Multipliers derived based on syntax elements are offsetX and offsetY.

offsetX(Y) = 2*scaled_ref_layer_left(top)_offset shall be 16 bits number:

−(2^14) ≤ scaled_ref_layer_left(top)_offset ≤ (2^14)−1

Recommendation:

- scaled_ref_layer_left(top)_offset shall be in a range −214 to 214− 1, inclusive in order to keep multiplier within 16 bits. For the simplicity, BoG recommends the same range for scaled_ref_layer_right(bottom)_offset.

  - this recommendation is opposite to the meeting notes (Decision.. limit the se(v) range to −2^31 to 2^31−2 (actually based on the maximum length of the sc(v) codeword).)

Summand derived based on syntax elements are RefOffsetX and RefOffsetY

RefOffsetX(Y) = ((2*ref_layer_left(top)_offset)<<4) shall be within 32 bits:

−(2^26) ≤ ref_layer_left(top)_offset ≤ (2^26)−1

Recommendation:

- ref_layer_left(top)_offset shall be in a range −226 to 226− 1, inclusive in order to avoid 32 register overflow. For the simplicity, BoG recommends the range from −214 to 214− 1 (the same range is recommended for scaled_ref_layer_offset ) for all ref_layer_offset.

  - this recommendation is opposite to the meeting notes (Decision.. limit the se(v) range to −2^31 to 2^31−2 (actually based on the maximum length of the sc(v) codeword).)

## Agreed actions integration on top of AhG13 recommended text.

All agreed during Track A discussion actions (except mentioned below) are integrated on top of AhG13 recommended spec text. Modified spec text is attached to this report.

Remaining open issue (pure editorial) is unequalPictureSizeFlag derivation.