| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0342 |
| --- | --- |

| Title: | BoG on block vector binarization methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krishna Rapaka | Email: | krapaka@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This report summarizes the block and motion vector binarization methods. The binarization methods categorized into two section a) intraBC block vector difference coding b) inter motion vector difference coding. Performances of the proposed methods are evaluated based on the SCC common test conditions.

# Introduction

A BoG was held on Wednesday July 2, 2014, 7:00 pm to 9:30 pm with the following mandates:

- Analyze contributions JCTVC-R0123, JCTVC-R0181, JCTVC-R0088, ICTVC-R0074.

- Study performance and complexity of the methods in these proposals.

- Analyze specific encoding techniques applied to these methods.

# List of Block vector coding contributions

## SCCE1 Contributions

- JCTVC-R0186, SCCE1: Test 3.6 – Block vector coding for Intra block copy, C. Pang, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)

## Non-SCCE1 Contributions

- JCTVC-R0074, Non-SCCE1: Adaptive switching between differential and direct coding for intra block copy vectors, K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)

- JCTVC-R0088, Non-SCCE1: Vector residual coding for Inter and IBC modes, G. Laroche, T. Poirier, C. Gisquet, P. Onno

- JCTVC-R0123, Non-SCCE1: On block vector coding, C. Rosewarne, M. Maeda (Canon)

- JCTVC-R0181, Non-SCCE1: Block vector coding for Intra block copy, M. Karczewicz, K. Rapaka, C. Pang, J. Sole (Qualcomm)

# Summary of proposals

## IntraBC block vector binarization methods

- Test 2.1.2: BV binarization (JCTVC-R0186) - SCCE1 Method

First, a flag is signaled to indicate whether the difference is zero. When BV difference is not zero, Exp-Golomb code of order 3 is used.

- Test 2.1.3: Adaptive switching between differential and direct coding (JCTVC-R0074)

One flag is added to specify whether the block vector is predicted or not. If not predicted, BV is coded without prediction. The flag is signaled when the BV component is non-zero.

Method 1: BVD binarization is modified: conditionally adds a new Bypass flag to indicate with BV component is predicted or not.

First, a flag is signaled to indicate whether the BV component is zero. When BV component is non-zero, a second flag is signaled if BV component is predicted or not. A flag is coded to indicate if BV component is greater than 1. When BV component is greater than 1, Exp-Golomb code of order 3 is used.

Method 2: BVD binarization is modified: conditionally adds a new bypass flag to indicate with BV component is predicted or not. Conditionally adds a second bypass flag (based BVDx and BVDy values) to indicate the sign of BVx and BVy components. Further conditionally encodes sign flag based on the second flag introduced.

- Test 2.1.4: Vector residual coding for Inter and IBC modes (JCTVC-R0088)

A slice level flag is added to switch between the Golomb order (1 and 4). At encoder side, Golomb order is determined from the previous frame statistics.

- Test 2.1.5: BV binarization (JCTVC-R0123)

BVD binarization is modified: The coeff_abs_level_remaining coding is applied to the delta block vectors as follows:

- A separate Rice parameter initialization statistic is kept for each component of the delta block vector and updated on each usage of the corresponding component.

- A separate context is used to code the significance flag of each component. No ‘gr1’ flag is present.

- The sign of each component is bypass coded.

Additionally, following bug-fixes that were distributed to participants in SCCE1 were incorporated into the software:

#define BUGFIX_FULL_FRAME_SEARCH_PREDICTORS               1

#define BUGFIX_Q0175                                      1

#define BUGFIX_COMP_SIGNED_UNSIGNED                       1 // bug fix comp sign / const unsigned

These bug fixes were also used in the anchor for the presented results. Section 3 shows the performance improvement of the bug fixes.

- Test 2.1.6: BV binarization (JCTVC-R0181)

First a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than 16. If BVD is greater than 16, the remaining absolute level is coded using bypass Exp-Golomb codes with order 5. Otherwise, the remaining absolute level is coded using fixed length bypass code of length 4. One flag is coded to indicate sign of the BVD.

It may be noted that the binarization is same to Exp-Golomb codes with order equal to 4. Separate contexts for horizontal and vertical component are used for the above two prefix flags.

### Results for Proposals on Intra BV binarization

The proposed methods were implemented on top of SCM1.0 and simulated under the SCC common test conditions.

|  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2.1.2 |  | 2.1.3M1 | 2.1.3M2 | 2.1.4 | 2.1.5 | 2.1.6 |
| RGB, text & graphics with motion, 1080p | -2.3% |  | -2.5% | -2.9% | -2.4% | -2.7% | -2.9% |
| RGB, text & graphics with motion,720p | -1.3% |  | -1.2% | -1.4% | -1.5% | -1.7% | -1.7% |
| RGB, mixed content, 1440p | -0.8% |  | -0.9% | -1.1% | -0.7% | -0.8% | -0.9% |
| RGB, mixed content, 1080p | -1.0% |  | -1.1% | -1.4% | -0.9% | -1.1% | -1.2% |
| RGB, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% |  | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p |  | -2.9% | -3.0% | -3.5% | -3.1% | -3.2% | -3.7% |
| YUV, text & graphics with motion,720p | -1.6% |  | -1.5% | -1.7% | -1.7% | -2.1% | -2.3% |
| YUV, mixed content, 1440p | -1.0% |  | -1.2% | -1.4% | -0.9% | -1.1% | -1.2% |
| YUV, mixed content, 1080p | -1.2% |  | -1.4% | -1.6% | -1.1% | -1.3% | -1.4% |
| YUV, Animation, 720p | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  | 100% | 100% | 99% | 113% | 99% |
| Dec Time[%] | 103% |  | 99% | 98% | 94% | 99% | 97% |

|  | Random access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.1.2 | 2.1.3M1 | 2.1.3M2 | 2.1.4 | 2.1.5 | 2.1.6 |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.4% | -1.5% | NA | -1.3% | -1.7% |
| RGB, text & graphics with motion,720p | -1.0% | -0.8% | -1.0% | NA | -1.1% | -1.3% |
| RGB, mixed content, 1440p | -0.5% | -0.6% | -0.7% | NA | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.6% | -0.8% | -0.9% | NA | -0.7% | -0.8% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | NA | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% | NA | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.5% | -1.7% | NA | -1.5% | -1.9% |
| YUV, text & graphics with motion,720p | -1.2% | -1.0% | -1.1% | NA | -1.4% | -1.5% |
| YUV, mixed content, 1440p | -0.7% | -0.7% | -0.8% | NA | -0.6% | -0.8% |
| YUV, mixed content, 1080p | -1.0% | -1.0% | -1.1% | NA | -0.9% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | NA | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | NA | 0.0% | 0.0% |
| Enc Time[%] | 102% | 100% | 100% | 102% | 100% | 88% |
| Dec Time[%] | 103% | 100% | 99% | 95% | 100% | 88% |

|  | Low Delay |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.1.2 | 2.1.3M1 | 2.1.3M2 | 2.1.4 | 2.1.5 | 2.1.6 |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.0% | -1.1% | NA | -0.9% | -1.4% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% | NA | -0.5% | -0.8% |
| RGB, mixed content, 1440p | -0.6% | -0.4% | -0.4% | NA | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | -0.4% | NA | -0.5% | -0.6% |
| RGB, Animation, 720p | -0.2% | 0.0% | 0.1% | NA | 0.0% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | 0.0% | NA | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.0% | -1.2% | NA | -1.0% | -1.5% |
| YUV, text & graphics with motion,720p | -0.5% | -0.6% | -0.6% | NA | -0.9% | -0.8% |
| YUV, mixed content, 1440p | -0.3% | -0.4% | -0.5% | NA | -0.7% | -0.4% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.5% | NA | -0.7% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | NA | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | NA | 0.0% | 0.0% |
| Enc Time[%] | 99% | 100% | 100% | NA | 100% | 95% |
| Dec Time[%] | 102% | 100% | 100% | NA | 100% | 95% |

### Complexity analysis for proposals on Inter BV binarization

| Inter binarization methods | Complexity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCM | 2.1.2 (CE) | 2.1.3 | 2.1.4 | 2.1.5 | 2.1.6 |
| Binarization as in HEVC | Yes | No | No | No | No | No |
| # regular bins per BV | 4 | 2 | 4 | 4 | 2 + 2(rice/not coded) | 4 |
| # additional contexts | 0 | 0 | 0 | 0 | 2 (rice ctx) | 2 |
| Adaptation scheme | Fixed | Fixed | Fixed | Slice level | Block (PU) level | Fixed |
| Separated Ctx between component | No | Yes | No | No | Yes | Yes |
| Exp-Golomb Order | 1 | 3 | 3 | 1 or 4 (Adaptive) | No | 4 |
| Change to Gr1 flag | No | Yes (Removed) | No | No | Yes (removed) | Yes (Gr16) |
| Additional Bins | No | No | 1 or 2 Bypass bins | No | No | No |
| Rice | No | No | No | No | 0 to 7 | No |

- Test 2.2.1: Vector residual coding for Inter and IBC modes (JCTVC-R0088)

MVD binarization is modified A slice level flag is added to switch between the Golomb order (1 and 4) for Inter. At encoder side, Golomb order is determined from the previous frame statistics.

- Test 2.2.2: BV binarization (JCTVC-R0181)

MVD binarization is modified: First a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than 1. If MVD is greater than 1, the remaining absolute level is coded using bypass Exp-Golomb codes with order 3. Separate contexts for horizontal and vertical component are used for the above two prefix flags.

### Results for Proposals on Inter BV binarization (combined test BVD+ MVD)

The proposed methods were implemented on top of SCM1.0 and simulated under the SCC common test conditions.

|  | Random Access |  |
| --- | --- | --- |
|  | 2.2.1 | 2.2.2 |
| RGB, text & graphics with motion, 1080p | -3.2% | -3.5% |
| RGB, text & graphics with motion,720p | -1.0% | -1.7% |
| RGB, mixed content, 1440p | -0.3% | -0.9% |
| RGB, mixed content, 1080p | -0.3% | -0.8% |
| RGB, Animation, 720p | -0.1% | -0.3% |
| RGB, camera captured, 1080p | 0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | -3.6% | -4.1% |
| YUV, text & graphics with motion,720p | -1.0% | -1.9% |
| YUV, mixed content, 1440p | -0.2% | -0.9% |
| YUV, mixed content, 1080p | -0.5% | -1.0% |
| YUV, Animation, 720p | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.2% | 0.0% |
| Enc Time[%] | 93% | 101% |
| Dec Time[%] | 101% | 103% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | 2.2.1 | 2.2.3 |  |
| RGB, text & graphics with motion, 1080p | -3.9% | -4.0% |  |
| RGB, text & graphics with motion,720p | -1.1% | -1.7% |  |
| RGB, mixed content, 1440p | -0.3% | -0.6% |  |
| RGB, mixed content, 1080p | -0.4% | -0.9% |  |
| RGB, Animation, 720p | -0.2% | -0.1% |  |
| RGB, camera captured, 1080p | -0.2% | -0.3% |  |
| YUV, text & graphics with motion, 1080p | -4.5% | -4.5% |  |
| YUV, text & graphics with motion,720p | -1.0% | -1.8% |  |
| YUV, mixed content, 1440p | 0.0% | -0.5% |  |
| YUV, mixed content, 1080p | -0.3% | -0.4% |  |
| YUV, Animation, 720p | 0.0% | 0.0% |  |
| YUV, camera captured, 1080p | 0.1% | -0.1% |  |
| Enc Time[%] | 93% |  | 106% |
| Dec Time[%] | 101% |  | 106% |

## Performance Results for encoder bug fix

One of the proposals applies following encoder bug-fixes

#define BUGFIX_FULL_FRAME_SEARCH_PREDICTORS               1

#define BUGFIX_Q0175                                      1

#define BUGFIX_COMP_SIGNED_UNSIGNED                       1 // bug fix comp sign / const unsigned

The impact of the bug fix is shown below.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.3% |
| RGB, mixed content, 1440p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.6% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -1.5% | -1.5% | -1.5% |
| YUV, mixed content, 1440p | -0.8% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -0.8% | -0.7% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -1.0% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.3% |
| RGB, mixed content, 1080p | -0.5% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | -0.9% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.5% | -0.7% | -0.5% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.6% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -0.7% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.6% | -0.6% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.3% | -0.7% |
| YUV, mixed content, 1080p | -0.1% | -0.7% | 0.2% |
| YUV, Animation, 720p | -0.1% | -0.5% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

## Side-activity (BoG) suggestions

- All the proposals discussed above change the binarization method.

- All the proposals use Exp-Golomb codes, except one proposal that uses Rice.

- Higher gains are observed with higher order golomb codes. The adaptation schemes in proposal vary from fixed schemes to slice or PU level adaptive schemes.

- Continue the CE to study  interaction with BV predictor and other methods based on decisions in this meeting

- Suggestion to include one of the best performing methods in  the test model as starting point and study interactions on SCM 2.0 software. This would provide the good framework for further study.