| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0345 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0190 |
| --- | --- |

| Title: | BoG report on HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 3 July 2014.

The BoG recommends the following:

- JCTVC-R0226

  - Modification to semantics of slice_temporal_mvp_enabled_flag

  - Modification to semantics in temporal motion vector prediction constraints SEI message

- JCTVC-R0227

  - Signalling of bit rate and picture rate information for additional layer sets

  - Adding a restriction on update of representation format of the base layer

  - Bug fix and simplification for value ranges of num_negative_pics, num_positive_pics, and num_long_term_pics

  - Add semantic constraint of vps_vui_bsp_hrd_present_flag

  - Remove restriction on update of separate_color_plane_flag in SPS

- JCTVC-R0276

  - Generalize scalable nesting SEI message so that it can be applied to additional layer sets

  - Editorial cleanups to derivation of BitstreamToDecode in the HRD and semantics of scalable nesting SEI message and the sub-bitstream property SEI message to properly reference the subclauses of F.10.x

- JCTVC-R0154

  - Add inference value to alt_output_layer_flag[ i ]

  - Modify semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures

- JCTVC-R0155

  - Modify the semantics of slice_type for I slice_type with respect to IRAP pictures and sps_max_dec_pic_buffering_minus1[ TemporalId ] value to enable inter-layer prediction could be used for pictures with nuh_layer_id > 0

  - Constrain the allowed range for values of output_layer_set_idx_to_vps[ i ]

- JCTVC-R0157

  - Constrain DependencyId and ViewOrderIdx to increase with increasing values of nuh_layer_id, subject to offline review of revised text in v2 version

# Review of Contributions

JCTVC-R0226 MV-HEVC/SHVC HLS: On indication of the use of temporal motion vector prediction [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

This document proposes two changes. The first change is to update the semantics of slice_temporal_mvp_enabled_flag. It is asserted that with the change, the semantics of slice_temporal_mvp_enabled_flag would be consistent for all layers in a way that is aligned with HEVC version 1. The second change is to update the semantics of prev_pics_not_used_flag in the syntax of the temporal motion vector prediction constraints SEI message. It is asserted that without the change, the error resilience feature that was intended by setting prev_pics_not_used_flag equal to 1 is not achieved.

It was suggested to add a note indicating when the SEI message could be helpful.

Agreed to first proposal, change to semantics to slice_temporal_mvp_enabled_flag.  Refer to editors to consider adding a note.

Agreed to second proposal, change of semantics in temporal motion vector prediction constraints SEI message.

JCTVC-R0227 MV-HEVC/SHVC HLS: Miscellaneous cleanups [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm)]

This document proposes the following changes:

- Signalling of the profile, tier, and level (PTL) information for the 0-th output layer set (OLS).  Addressed elsewhere

- Signalling of bit rate and picture rate information for additional layer sets.  Agreed

- Adding a restriction on update of representation format of the base layer.  Agreed

- Not sending video signal information for externally provided base layer.

Suggestion to enable but not require base layer info to be signaled.  Revisit.

- Bug fix and simplification for value ranges of num_negative_pics, num_positive_pics, and num_long_term_pics

It was suggested that the bug could be fixed without doing the simplification.

Agreed.

- Conditional signalling or restriction of vps_vui_bsp_hrd_present_flag

Suggested to add a semantic constraint.  Agreed to add semantic constraint.

- No restriction on update of separate_color_plane_flag. Agreed.

JCTVC-R0276 MV-HEVC/SHVC HLS: On sub-bitstream extraction [M. M. Hannuksela (Nokia)] [late]

The following items are proposed in this contribution:

- The scalable nesting SEI message is generalized so that it can be applied to additional layer sets or output layer sets.

In order to avoid confusing version 1 decoders with a layer set index that is beyond the range of the layer sets in the base VPS, it is required that when a layer set index of an additional layer set (specified in the VPS extension) is used within a scalable nesting SEI message, the SEI NAL unit shall have nuh_layer_id greater than 0. Consequently, version 1 decoders would ignore the whole SEI NAL unit.

A suggestion was made to not include the output layer sets.  A suggestion was made to have the constraint apply for a lowest value of nuh_layer_id in the identified operation points, rather than just require nuh_layer_id > 0.  Agreed with the suggested changes.

- Subclauses F.10.x are consistently used when sub-bitstream extraction is referred to. Specifically:

- The derivation of BitstreamToDecode in the HRD (subclause C.1) is done similarly to the derivation of BitstreamToDecode in subclause 8.1.

- In the semantics of scalable nesting SEI message and the sub-bitstream property SEI message, subclauses F.10.1 or F.10.3 are invoked when a sub-bitstream with an external base layer is extracted or when an additional layer set is extracted, respectively.

Agreed.

- A requirement for bitstream conformance is added into subclauses F.10.1 and F.10.3 such that if zero or more such layers that are not reference layers of the remaining layers are removed from a layer set and an additional layer set, respectively, the output of the sub-bitstream extraction process is conforming.

Some discussion about the impact of HRD parameters and particularly CBR case. There could also be an issue with fixed frame rate if the layers had different frame rates.

Revisit.

- The semantics of the sub-bitstream property SEI message are modified to keep pictures of alternative output layers included in the sub-bitstream when they are to be output due to the absence of a picture with PicOutputFlag equal to 1 in the target output layer.

Revisit.

JCTVC-R0154 MV-HEVC/SHVC HLS: On Alternative Output Layer and Default Output Layer Indication [S. Deshpande (Sharp)]

In this document following is proposed:

- Proposal 1: A modification is proposed to the semantics of alt_output_layer_flag[ i ] to infer the value when it is not present. This is because alt_output_layer_flag[ i ] is only signaled for an output layer set when NumOutputLayersInOutputLayerSet[ i ] is equal to 1 and NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] is greater than 0, however the value of alt_output_layer_flag[ i ] is used in the specification for any output layer set.

Additionally a modification is proposed where derivation of the variables AltOptLayerFlag[ i ] is no longer necessary.

- Proposal  2: A modification is proposed to the semantics of default_output_layer_idc and output_layer_flag[ i ][ j ] to consistently handle auxiliary pictures.

Agreed to proposal 1, to add inference to alt_output_layer_flag[ i ], and editorial changes to use syntax element instead of variable.

Agreed to option 2 of proposal 2 of the r1 version, to change the semantics of default_output_layer_idc and output_layer_flag[ ] [ ] .

JCTVC-R0155 MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

Proposal 1 belongs to this agenda category.

Proposal 2 relates to the sub-bitstream property SEI message and hence relates to agenda category Error: Reference source not found.

In this document following is proposed:

- Proposal 1: It is proposed to modify the semantics of slice_type for I slice_type with respect to IRAP pictures and sps_max_dec_pic_buffering_minus1[ TemporalId ] value since inter-layer prediction could be used for pictures with nuh_layer_id > 0.

Agreed.  Editors to consider if this should be expressed in Annex F rather than subclause 7.  There already exists the following language in F.7.4.8.1:

–	"When nal_unit_type has a value in the range of 16 to 23, inclusive (IRAP picture), slice_type shall be equal to 2." is replaced by "When nal_unit_type has a value in the range of 16 to 23 and nuh_layer_id is equal to 0, inclusive (IRAP picture), slice_type shall be equal to 2."

The proposal makes additional semantics changes.

- Proposal 2: Allowed value range is defined for output_layer_set_idx_to_vps[ i ] to point to a valid output layer set.  Agreed.

JCTVC-R0157 Miscellaneous questions and clarifications for SHVC [J. Boyce (Vidyo)]

Chaired by M. Hannuksela

Proposal 3

- Constraint needed to restrict that DependencyId and ViewOrderIdx must increase with increasing values of nuh_layer_id

It was suggested to consider if this constraint should also apply when both AuxId’s are equal to each other, which would apply for the texture + depth coding case.

Agreed in spirit with the proposed suggestion, subject to offline review of the text.  A v2 version of the text with the suggested change will be uploaded.