| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0346_r5 |
| --- | --- |

| Title: | BoG on SCCE3 palette coding mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Robert Cohen Yu-Wen Huang | Email: | cohen@merl.com yuwen.huang@mediatek.com |
| Source: | SCCE3 BoG |  |  |

_____________________________

# Abstract

This document contains the notes from the BoG meeting on SCCE3 and SCCE3-related topics.

# Introduction

The mandates of this BoG are:

- Analyze the cat. C proposals in terms of complexity (memory bandwidth, CABAC throughput, number of operations per CU/sample etc., parsing dependencies

- Suggest and perform a similar analysis for new useful combinations of cat. A and B methods, in order to come up with a combination that provides a reasonable performance vs. complexity tradeoff

- Review remaining SCCE3-related documents

Cat. A is palette coding; B is color index map coding; C is SCCE3 combinations of palette table coding and color index map coding.

The BoG met on Thursday July 3, 2014 from 0920 – 2105, Friday, July 4 from 0900 – 1425, and Sunday July 6 from 0910-1215.

# Mandate 1: Category C analysis (SCCE3 combinations of palette table coding and color index map coding)

Please refer to attached analysis spreadsheet, which was filled out for existing proposals on the first day of the BoG, and for the proposed combined software on the second day of the BoG meeting.

On Friday morning, encoding runs for the combined software and individual-tool-disabling tests are still under way. Results will be reported as they become available. A new input contribution will be created to show and summarize these results (JCTVC-R0348).

JCTVC-R0348 Suggested combined software and text for run-based palette by the palette BoG [P. Onno (Canon), X. Xiu (InterDigital), Y.-W. Huang (MediaTek), R. Joshi (Qualcomm)] [late]

During the Sapporo meeting, the SCCE3 BoG provided a new run-based palette mode, which combines several SCCE3 run-based palette techniques asserted to have reasonable gain-complexity tradeoffs reported in the SCCE3 summary report. It is shown that the BD-rates savings achieved by the combined palette mode under full frame intra block copy (FF-IBC) and all intra conditions when compared against JCTVC-Q0094BF palette mode (and palette mode off) are as follows:

12.1%		(21.5%)	for RGB, text & graphics with motion, 1080p
10.7%		(15.8%)	for RGB, text & graphics with motion, 720p
5.8%		(6.9%)	for RGB, mixed content, 1440p
5.7%		(7.7%)	for RGB, mixed content, 1080p
12.3%		(21.4%)	for YUV, text & graphics with motion, 1080p
6.9%		(11.1%)	for YUV, text & graphics with motion, 720p
4.5%		(5.4%)	for YUV, mixed content, 1440p
5.1%		(7.3%)	for YUV, mixed content, 1080p

In comparison with SCCE3 category C palette modes and their non-CE variants, about 1-4% additional BD-rate savings are observed, while overall complexity is asserted to be reduced according to conducted analysis.

On top of the combined palette mode, a proposed simplification was further studied: horizontal scan and vertical scan are removed, and only horizontal traverse scan and vertical traverse scan are left. It is shown that the BD-rate increases are 0-0.1% with this method.

Also included in this proposal is a removal of the horizontal and vertical raster scans for the color index coding, reducing the scans to two (horizontal traverse and vertical traverse). Losses from disabling the horizontal and vertical raster scans are reported to be 0.0%- 0.1%.

In addition to the above results, more results were obtained by individually disabling tools individually. Losses in BD-Rate from disabling the tool, for RGB Text & Graphics 1080p G component are:

Disabling “palette sharing” versus combined palette mode: 0.5%

Disabling “palette stuffing and grouping reuse flags” versus combined palette mode: 6.7%

Disabling “grouping reuse flags” versus combined palette mode: 2.5%

Disabling “encoder-only palette generation” versus combined palette mode: 5.7%

Disabling “vertical scan and vertical traverse scan” versus combined palette mode: 0.8%

Disabling “redundant index removal” versus combined palette mode: 0.7%

Disabling “truncated binary codes for color indices” versus combined palette mode: 2.3%

Disabling “escape color values coded as bypass” versus combined palette mode: 0.1% (purpose of tool is to reduce complexity)

Disabling “traverse scans” versus combined palette mode: 0.7%

Disabling “palette reset per CTU row for WPP-on only” versus combined palette mode: 1.0%

Disabling “bugfixes of RDO and bugfix of palette_num_signalled_entries” versus combined palette mode: 0.1% (purpose of tool is bug fixes)

It was noted that the palette sharing removal only caused 0.5% loss, whereas in the CE it was producing more than 3% gain, so another tool may be targeting the same aspect.

There was discussion on whether the BoG should recommend adopting a palette mode tool into SCM. A proponent of JCTVC-R0348 reiterated that palette mode has been around for several meeting cycles and has been tested in a CE.

It was noted that lossless coding result are not yet available for the proposed combined software. A proponent noted that lossless results for individual tools are available from the SCCE3 results. It was also noted that this BoG had intended only to test lossy all-intra for in-meeting tests. It was noted that such results could become available during the short period after the meeting when the SCM software is being prepared for release. A non-proponent said that sufficient information to make a decision is available now, and that it would be good to know the lossless results, but with the information we have now, that’s sufficient for recommending that palette become part of the next SCM draft text and software.

Recommendation from BoG: Palette mode should become part of the next SCM draft text and software. The BoG also recommends that the palette mode described in JCTVC-R0348, including the removal of horizontal and vertical raster scans, be the version of palette mode that is incorporated as a result of this meeting. Also review the above individual-tool-removal tests listed above.

The following was clarified:

R0348 is a proposal of combined existing SCCE tools

R0351 takes R0348 as a basis, and switches on additional tools from both the CE and the non-CE category. Per the JCT-VC meeting notes, the non-CE tools are not being recommended by the BoG for immediate inclusion in the recommended palette mode text and software, but are being recommended for study in a CE. Additional tools from the CE category will be individually examined and recommended or not recommended for immediate inclusion. This list of CE-category tools is in JCTVC-R0351

JCTVC-R0351 Additional test results on top of suggested combined palette software in JCTVC-R0348 [R. Cohen, Y.-W. Huang (SCCE3 BoG)] [late] [miss]

- Long-term palette, Test A9 (JCTVC-R0047)

- Results not available now, recommend further study, not immediate inclusion.

- Transition copy, Test B2 (JCTVC-R0120)

- Partial results available, proponent asks this be studied in CE and not immediately included. Recommendation: further study in relevant CE

- Transition copy, Test B6 (JCTVC-R0084)

- All-intra RGB text & graphics 1080p: 0.6% G, 720p: 0.2% gains. Proponent asks for further study in CE and not immediate inclusion.

- Recommendation: further study in relevant CE

- Restricted run signaling, Tests B7 (R0085) and B14 (JCTVC-R0045)

- Gains 0.4% 1080p, 720p: 0.3% all-intra text & graphics. Loss of 0.1% for mixed content. Proponent asks that this version be included in recommended palette text and software. A participant asked whether this required index reconstruction, i.e. parsing dependencies. It was noted that at this stage there is no consensus that this kind of dependency is an issue. A participant noted that the gains of this tool are smaller than some of the non-CE tools that aren’t being considered for immediate inclusion. A participant asserted that this method modifies the encoder, and if the encoder is not modified, the gains are different. A proponent asserted that although these gains are small, some of the CE tools that are currently in JCTVC-R0348 have similar gains. Another participant (competing) said that these kinds of proposal should be studied in CE to study complexity issue to see whether these parsing dependencies are an issue. A cross-checker supported including this in the proposed combined software now.

- The proponents of test B7 and B14 ask for immediate inclusion; two non-proponents who have proposals using different methods to remove similar redundancies (which are not mutually-exclusive with this proposal) asked for this proposal not to be included immediately. One cross-checker supported immediate inclusion; the other cross-checker had no position at the moment. Some participants asked if the gains were worth the change. A proponent noted that similar already-included tools have similar gains, although another participant stated that gains of 0.4% and 0.7% aren’t necessarily similar.

- Recommendation: Decide on one of the following:

- If the parsing dependency is not an issue, then include this tool immediately in the combined proposed software, and conduct a CE on this and related tools, and as part of this CE, also test the disabling of this tool; otherwise

- Not include this tool immediately, and study this and related tools in a CE (and study whether parsing dependencies are an issue, if necessary).

- CU level escape color flag, Test B13 (JCTVC-R0066)

BD-Rate improvement 0.4% for text & graphics with motion, 0.1% for mixed content.

It was noted that there are no parsing dependencies with this proposal

This is related to the non-CE proposal JCTVC-R0051, which uses bypass coding for the CU-level escape color flag. It was noted in the BoG meeting report that JCTVC-R0051 would also be considered with JCTVC-R0066, if it produced no loss.

JCTVC-R0051 produced 0.4% & 0.3% for text & graphics with motion, 0.1% for mixed content.

Recommendation: Include this, with the bypass coding of the CU-level escape color flag (JCTVC-R0051 & JCTVC-R0066). Also study adding back the context coding, in a CE.

Existing non-SCCE tools being considered for study in a subsequent CE:

Note: Gains below are just listed for RGB, there was some brief discussion that common test conditions should include sufficient common reporting conditions.

JCTVC-R0051: Already discussed above, to be included in proposed combined software

JCTVC-R0113: Implementation still in progress, but existing results are of sufficient interest to be included in a CE.

JCTVC-R0135: 0.2% loss to 0.3% gain for RGB, perhaps include in index coding CE.

JCTVC-R0136: RGB T&G with motion 1080p: 1.7% gain – include in CE

JCTVC-R0161: 0.2% loss, do not include in CE given all the losses

JCTVC-R0174: 0.7%/0.3% gain, include both parts in CE

JCTVC-R0215, the part related to whether the maximum palette size should be signalled: include in CE for various maximum palette sizes (also perhaps include in an AHG discussion of what’s the best size for a palette)

JCTVC-R0228: 0.3% gain, palette reuse flag coding, include in CE

- Mandate 2: Suggest and analyze new combinations of cat. A and B methods

Several proponents discussed new combinations prior to this BoG meeting:

Category A palette coding:

- Palette sharing, Tests A2 (JCTVC-R0119), A4 (JCTVC-R0142), and A5 (JCTVC-R0166)

- Palette stuffing, Tests A3 (JCTVC-R0082), A4 (JCTVC-R0142), A5 (JCTVC-R0166), and A8 (JCTVC-R0063)

- Grouping reuse flags, Test A8 (JCTVC-R0063)

- Encoder-only palette generation, Tests C2 (JCTVC-R0086) and C4 supplemental (JCTVC-R0175)

=====================================================

Category B color index map coding:

- Vertical scan, Tests B4 (JCTVC-R0057), B8 (JCTVC-R0143), B9 (JCTVC-R0168), and B15 (JCTVC-R0048)

- Redundant index removal, Tests B5 (JCTVC-R0083), B10 (JCTVC-R0269), and B12 (JCTVC-R0065)

- Truncated binary codes for color indices, Tests B5 (JCTVC-R0083) and B12 (JCTVC-R0065)

- Escape color values coded as bypass, C4 (JCTVC-R0067)

- Traverse scans, B9 (JCTVC-R0168)

=====================================================

Others:

- Palette reset per CTU row for WPP-on only, C4 (JCTVC-R0067) (control with WPP flag to be added later)

- Bugfixes of RDO (JCTVC-R0306)

- Bugfix of palette_num_signalled_entries (JCTVC-R0215)

- Software memory reduction (JCTVC-R0307) (to be added later)

=====================================================

Implementation:

- Which software package should we start to add/remove things? Q0094BF, C2, or C4 supplemental? – discussed offline

=====================================================

In-meeting tests of the following tools on top of the combined proposal software will be performed:

- Long-term palette, Test A9 (JCTVC-R0047)

- Transition copy, Test B2 (JCTVC-R0120)

- Transition copy, Test B6 (JCTVC-R0084)

- Restricted run signaling, Tests B7 (R0085) and B14 (JCTVC-R0045)

- CU level escape color flag, Test B13 (JCTVC-R0066)

There was discussion on whether these tools should be tested in a “TE-12”-style test, where tools are individually disabled, i.e. revert back to JCTVC-Q0094 method for that particular tool. The proponents of this combined software will try to generate all-intra (lossy, full-frame IBC, 2-CTU optional) simulation results compared to a reference of JCTVC-Q0094BF palette mode (and maybe RA and LB if time) before the end of this JCT-VC meeting. Also, proponents will run disable individual tools in this proposed combination (and thus revert to JCTVC-Q0094BF for that tool) to see how each tool performs. The anchor for this latter set of experiments will be this new combination proposal with all tools enabled. Participants will suggest a set of tool-disabling tests if it turns out that it doesn’t make sense to individually disable every one of the new tools.

Results related to the proposed combined software, expected during this JCT-VC meeting:

- Combined software (the source code was released on the evening of Thursday July 3, with a minor update that doesn’t affect coding performance or bit-stream released after that). The document for this software and associated text and test results is JCTVC-R0348.

- Performance of combined software vs. JCTVC-Q0094BF (expected late Friday or early Saturday)

- Results from individual-tool disabling tests (expected Saturday)

- From Mandate 3, 6 non-SCCE proposals are being tested on the recently-released proposed combined software, and based on the results and software released to the SCCE3 email list (expected Saturday) they will be considered for inclusion in the combined software. The integration may be done during the 2-week period after the meeting if it is not feasible to do it during the meeting:

JCTVC-R0051, JCTVC-R0113, JCTVC-R0161, JCTVC-R0174, JCTVC-R0215, JCTVC-R0228

- There is also a proposed set of proposed memory-reduction fixes that are recommended in JCTVC-R0317. These should be added to SCM-2.0 if palette becomes part of SCM-2.0, or they should be added to the software used for common palette test conditions if palette does not become part of SCM-2.0.

- The following SCCE tools (also listed earlier) are also being tested on the proposed combined software, and based on the results and software released to the SCCE3 email list (expected Saturday) they will be considered for adding to the combined software:

Long-term palette, A9 (*Nokia)

Transition copy, B2 (*MediaTek)

Transition copy, B6 (*Canon)

Restricted run signaling, B7 (*Canon), B14 (Nokia)

CU level escape color flag, B13 (*Qualcomm)

Initial simulation results for the proposed combined software was available for all-intra full-frame IBC lossy conditions early Friday afternoon. The anchor was the SCCE3 anchor (with palette and full-frame IBC). For example, RGB text & graphics 1080p: BD-Rate improvements are 12.1% in the new software vs. 11.4% from the SCCE3 category C.4 supplemental tests. The latter results are available in JCTVC-R0175 and the SCCE3 summary report JCTVC-R0033. These results are being cross-checked now. Cross-check results and run-time ratios will be available some time Saturday July 5. These results will be included in JCTVC-R0348.

# Mandate 3: Review remaining SCCE3-related documents

General comments:

The recommendations for several of these proposals is to perform in-meeting tests on top of the proposed combined software that is planned to be released on Thursday, July 3. There was some discussion during the BoG that there may not be sufficient time to analyze and discuss the results of too many in-meeting tests, so it may be possible to combine several of these tests into CEs for the next meeting. Also, for the simplification proposals, if the in-meeting tests produce no loss, then those tools can be added to the proposed combined software now. It was noted that some cross-checking may be needed.

Another question is to whether “non-SCCE” proposals can be included in the proposed combined software given that they weren’t tested as much yet as compared to the SCCE tools.

### SCCE3 related (palette mode) (43)

JCTVC-R0051 Non-SCCE3: Adaptive escape scheme for palette mode [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

This document proposes an adaptive escape scheme for palette mode. The scheme can turn on/off the escape function of a palette CU by a CU-based escape flag. The contribution is implemented on top of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047. Experimental results show that, compared with SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, -0.7% and -0.4% BD-rate savings under AI-Lossy coding are, respectively, achieved (1080p) on 2-CTU and full-frame IBC conditions for YUV Text & Graphics with motion sequences.

Bypass mode is used to code the skip flag, in contrast to JCTVC-R0066, only difference is whether the flag is context coded. Results are similar.

Recommendation: Test this with the SCCE3 BoG’s combined software and add to software if there is no loss.

JCTVC-R0330 Crosscheck of Non-SCCE3: Adaptive escape scheme for palette mode [R. Joshi, W. Pu (Qualcomm)] [late] [miss]

Cross-checker stated that results matched, document will be uploaded.

JCTVC-R0075 Non-SCCE3: Encoder modification of palette mode for escape pixels [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

This document proposes an encoder modification of palette mode. If only few pixels are quantized to a major color, the method will modify the major color index as an escape index. The contribution is combined with JCTVC-R0051, and implemented on top of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047. Experimental results show that, compared with SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, –1.4% and –1.1% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively.

BD-rate savings obtaind by using a CU-level flag.

A similar, but not identical concept is used in the SCCE3 BoG combined method, from JCTVC-R0175 (subtest C.4 supplemental, with improved encoder, also subtest A.7).

Recommendation: Further study using the palette mode in SCM-2.0 (if present).

JCTVC-R0263 Cross-check of 'Non-SCCE3: Encoder modification of palette mode for escape pixels' (JCTVC-R0075) by ITRI [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0076 Non-SCCE3: Run-copy coding method for two-color index map of palette CU [Y.-J. Chang, C.-L. Lin, C.-H. Hung, C.-C. Lin, J.-S. Tu (ITRI)]

This document proposes a run-copy coding method for two-color index map of palette CU. An encoder modification is also implemented for the proposed coding method. The contribution is implemented on top of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047. Experimental results show that, compared with SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, -1.7% and -0.8% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively.

When palette CU has only two colors, the value of a major color index must be either 0 or 1. A two-color index map coding method is proposed by skipping the transmission of color indices.

A participant asked what happens if there’s an escape. The proponent said they restrict this from the encoder when there are only two colors in the palette table, i.e. there is no escape used. The decoder was modified as well not to expect an escape flag when there are only two colors in the palette. It was noted that this method would work well with JCTVC-R0051 (from the same proponent).

It was noted that this method is related to tests B.5, B.10, and B.12. It’s a special case of these redundant index removal tests.

Recommondation: If there is a CE on related palette mode improvements, this should be included.

JCTVC-R0068 Crosscheck Non-SCCE3: Run-copy coding method for two-color index map of palette CU [W. Pu (Qualcomm)] [late]

JCTVC-R0078 Non-SCCE3: Modified copy above mode for palette based coding [J. Zhu, Z. Xu, J. Ye (Fujitsu R&D Center)]

A simplification of the process for copy above mode on palette based coding is presented. It uses a pixel value directly rather than an index for the encoding and decoding process for copy mode. In addition,  copy above mode is extended from 1 direction to 3 directions. Reported test results show gain of 0.1% AI-lossless for 1 above direction and 0.1-0.3% AI-lossless on SCs for 3 above direction on top of the SCCE3 anchor (full frame IBC).

This proposal contains two methods:

Use pixel value rather than index for copy mode, so the index does not have to be derived.

Copy above mode extended from 1 to 3 directions above

Proponent only presented results for lossless coding

Recommendation: No action

JCTVC-R0079 Non-SCCE3: Skip palette size for palette based coding [J. Zhu, J. Ye (Fujitsu R&D Center)]

By deriving palette size from index information, the syntax element of palette size is skipped for palette based coding. Reported test results show gain of 0.2-1.5% on AI-lossless，0.1-0.8% on RA-lossless, 0.1-0.7% on LB-lossless,0.1-1.0% on AI-lossy, ,0.1-0.5% on RA-lossy and ,0.1-0.3% on AI-lossy on top of SCCE3 anchor(2CTU-IBC).

AI lossy RGB text & graphics 1080p BD-Rate change: -1.0% on G, 720p: -0.4 on G

A comment was that truncated Rice codes to remove redundancy when number of colors is not a power of two. This proposal interacts with those proposals (test B.5, B.10, and B.12), which may part of the SCCE3 BoG combined proposal. It was unclear whether this method would still yield gains over the combined proposal.

Recommendation: No action

JCTVC-R0080 Non-SCCE3: Modified Escaped pixel mode in palette based coding [J. Zhu, J. Ye (Fujitsu R&D Center)]

The escaped pixel mode is modified if the palette size is less than a predefined maximum palette size. The escape flag is used to indicate that the pixel value or index is coded. An escaped pixel run is introduced for case when the palette size is less than predefined maximum palette size. The predictor construction is also modified accordingly. Reported test results show gain of 0.3-1.7% AI, 0.1-1.1% RA, 0.1-1.0% LB for lossless coding and 0.2-1.0% AI, 0.1-0.4% RA, 0.1-0.5% LB for lossy coding on top of SCCE3 anchor (2CTU IBC).

This in effect looks isolated colors in the palette and makes them escape pixels. A run is used for the escaped value.

Recommendation: No action

JCTVC-R0112 Non-SCCE3: Bi-color intra mode for screen content coding [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)] [late]

This document proposes a bi-color intra mode for screen content coding. The proposed bi-color intra mode first transmits an index to specify which two colors are selected out of available spatial neighboring pixels of the current CU. Then an index map for the CU is established by the selected two colors. An index map coding is used to transmit the index map. Experimental results show that, compared with SCM-1.0, -2.9% and –2.7% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively.

RGB Text and Graphics with motion AI lossy BD-Rate change: -1.9%, -2.0%, -2.2% for GBR. With palette enabled: -0.3%, -0.3%, -0.3%.

This method selects 2 colors from previously-decoded boundary pixels, and then signals indices, and then it codes an index map using run-length coding similar to what they used in JCTVC-R0076.

A participant commented that the same method could be used within palette mode, i.e. enable palette, and if there are only two colors, apply JCTVC-R0076.

Proponent asks that this be furthered study in CE

Recommondation: Because both chairs of this BoG are proponents of related proposals JCTVC-R0058 (Test D.1), JCTVC-R0200 (Test D.2), JCTVC-R0198 (simplified Test D.1), we should complete this discussion during the BoG report in the main track.

JCTVC-R0113 Non-SCCE3: Improved Palette Index Coding with Contextualization [T. Laude (Univ. Hannover)]

This contribution presents a modified palette index coding method. In particular, it is reported that the most significant bin (MSB) of the palette_index syntax element is coded using contextualization rather than using the CACAC bypass mode. The modified palette index coding reportedly shows Y/G BD-rate changes of {-0.7%, -0.5%, 0,0%, -0.1%, 0.0%, 0.0%, -1.0%, -0.4%, 0.0%, -0.2%, 0.0%, 0.0%}, {-0.4%, -0.5%, -0.1%, -0.1%, 0.0%, 0.0%, -0.4%, -0.3%, -0.2%, -0.1%, 0.1%, 0.0%} and {-0.3%, -0.5%, -0.2%, 0.1%, 0.0%, 0.0%, -0.3%, -0.2%, -0.4%, -0.1%, 0.0%, 0.0%} for AI, RA and LD compared to the SCCE3 palette mode software under SCC common test conditions.

Currently, palette mode signalling is done with fixed-length coding using CABAC bypass mode. This proposal uses MSB contextualization.

RGB text and graphics with motion: -0.7%, -0.7%, -07% for GBR

A non-proponent expressed interest in this method and it may be a useful method to add to the BoG’s combined software platform.

Recommendation: Do in-meeting test of this on top of proposed BoG combined software, and include in software if there is no loss.

JCTVC-R0135 Non-SCCE3: Palette index coding using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

This contribution develops a new method for coding palette index values for improved compression efficiency. The proposed method utilizes a universal entropy coding scheme representing a syntax value by its most significant bit (MSB) index and refinement bits. It is reported to achieve average Luma BD-rate savings of 1.1%, 0.6%, and 0.4% for the AI, RA, and LB settings, respectively, using IBC with a full-frame search range on the full-length test sequences and 2.0%, 1.0%, and 1.0% for the AI, RA, and LB settings, respectively, using IBC with a 2-CTU search range on the short-length sequences in the YUV, text & graphics with motion, 1080p category compared with the related results of the SCCE3 palette base software under the SCC common test conditions.

RGB text & graphics with motion AI gains: -1.0%, -1.0%, -1.0% for GBR for full-frame IBC with palette; -1.8%, -1.9%, -1.8% for 2-CTU IBC with palette.

Recommendation: Do in-meeting test of this on top of proposed BoG common platform.

JCTVC-R0305 Cross-check of JCTVC-R0135 on Palette index coding using a universal entropy coding scheme (Non-SCCE3) [Y. He, Y. Yan (InterDigital)] [late]

JCTVC-R0136 Non-SCCE3: Run coding of the palette index map using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

This contribution develops a new method for coding syntax palette_run of palette coding for improved compression efficiency. The proposed method utilizes a universal entropy coding scheme representing a syntax value by its most significant bit (MSB) index and refinement bits. It is reported to achieve average Luma BD-rate savings of 0.9%, 0.4%, and 0.3% for the AI, RA, and LB settings, respectively, using IBC with a full-frame search range on the full-length test sequences and 1.4%, 0.6%, and 0.7% for the AI, RA, and LB settings, respectively, using IBC with a 2-CTU search range on the short-length sequences in the YUV, text & graphics with motion, 1080p category compared with the related results of the SCCE3 palette base software under the SCC common test conditions.

Similar to JCTVC-R0135, except that the method is used to code the palette run

Recommendation: Do in-meeting test of this on top of proposed BoG common platform.

JCTVC-R0323 Crosscheck of JCTVC-R0136 Non-SCCE3: Run coding of the palette index map using a universal entropy coding scheme [X. Zhang, R. Cohen (MERL)] [late] [miss]

JCTVC-R0161 Non-SCCE3: Simplified palette size coding [J. Zhao, S. K. Kim, K. Misra, A. Segall (Sharp)]

This contribution proposes to change the binarization of palette_num_signalled_entries to use a fixed length code, as compared to the existing Truncated Unary (TR) code. It reduces the worst case number of coded bins from 32 bins to 5 bins in the existing palette coding structure. It has been also observed that the proposed method provides coding performance benefit of 0.1% and 0.2% for lossy coding and lossless coding, respectively.

It was commented that the current JCTVC-Q0094BF software has a bug in which it uses unary instead of truncated unary coding.

It was also noted that these are all being coded using bypass mode.

The proponent noted that the purpose of this proposal is for simplification, not BD-Rate gain.

A participant inquired whether this was really a problem that needed to be addressed.

A chair commented that re-testing on the proposed common platform may not be necessary because this is primarily a change in signalling that doesn’t affect performance. A participant said that we would want to make sure there was no loss.

It was commented that the software defines the maximum palette size, and there is no signalling of the maximum number of possible entries, so this proposal’s target number of bins may be different than 5, depending on the palette size that is used.

Recommendation: Re-test on proposed common platform to verify that there is no loss, then consider for addition to proposed common platform.

JCTVC-R0322 Cross check of ‘Non-SCCE3: Simplified palette size coding’ (JCTVC-R0161) by Sharp [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-R0174 non-SCCE3: Contexts for coding index runs [R. Joshi, M. Karczewicz, W. Pu, V. Seregin, J. Sole (Qualcomm)]

Additional contexts for coding the runs of index values are proposed. The context is dependent on the index value. BD-rate results for two additional contexts are presented for all intra configuration with respect to the AHG palette base as well as SCCE3 subtest C.4 with improved encoder. The method reportedly improves the BD-rate performance by up to 0.3% and 0.5% with respect to AHG palette base with full-frame intra block copying and two CTU intra block copying, respectively. When compared against subtest C.4 with improved encoder, the BD-rate improvements are reported to be much higher (up to 0.6% and 0.9%, resepctively).

Additional results using exponential Golumb coding with parameter 2 to encoding remaining run values was presented. This information is in a later version of this document which is awaiting approval.

In the proposed combined software, sometimes the index has to be incremented by 1 on the decoder side, so it was suggested that we should also test what happens if we use the parsed index value (before increment), to avoid parsing dependencies.

Recommendation: Re-test on proposed common platform to verify that there is no loss, then consider for addition to proposed common platform.

JCTVC-R0316 Cross check of ‘non-SCCE3: Contexts for coding index runs’ (JCTVC-R0174) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-R0175 non-SCCE3: Additional results for subtest C.4 [R. Joshi, V. Seregin, W. Pu, M. Karczewicz, J. Sole (Qualcomm)]

In subtest C.4 of screen content coding experiment 3 (SCCE3), a combination of proposed tools from subtests A.7, A.8, B.12 and B.13 was tested. This document reports changes to the encoder for subtest C.4. It is asserted that there are no normative changes with respect to subtest C.4. The changes in the encoder reportedly achieve BD-rate improvements up to 11.5% and 13.7% with respect to the SCCE3 palette base as anchor for all intra configuration for full-frame intra block copying and two CTU intra block copying, respectively. Simulation results for a subset of C.4 tools are also presented.

The presentation also included one of the informational tables from the SCCE3 Summary Report (JCTVC-R0033 presentation, page 39) showing that the gains from combined tools exceed the added gains of individual tools.

The proponent supports the proposed common BoG software, so at this stage, this document is more of an informative document.

JCTVC-R0308 Cross-check of JCTVC-R0175: non-SCCE3: Additional results for subtest C.4 [S. H. Kim, A. Segall (Sharp)] [late]

JCTVC-R0198 Non-SCCE3 Test D.1 Modified single color mode [P. Lai, S. Liu, Y.-W. Chen, S. Lei (MediaTek)]

This contribution presents modified methods related to SCCE3 Test D.1 Single Color Mode (JCTVC-R0058). In the original proposal, to obtain the final two single color candidates, the candidate list reconstruction process is modified by pruning five samples to only two samples. For this proposal, the syntax remains the same as the original proposal, and only the candidate list construction process is changed. Simulation results of this Method 1 report very similar coding efficiency as the original SCCE3 Test D.1 proposal.

Instead of reducing 5 color candidates two 2, this proposal directly selects two candidates. The gains in performance for AI conditions is 0.0%-0.1%.

A participant asked if it’s worth adding a new mode for small gains, if changing palette mode can achieve most of these gains. However, palette has more overhead than this method.

Refer to the notes for JCTVC-R0112 regarding discussing this in the JCTVC-plenary.

JCTVC-R0222 Cross-check of JCTVC-R0198 Non-SCCE3 Test D.1 Single Color Mode, Method 1 [R. Cohen, X. Zhang (MERL)]

JCTVC-R0202 Non-SCCE3: Copy from previous row mode for palette coding [F. Zou, R. Joshi, J. Sole, M. Karczewicz, W. Pu, V. Seregin (Qualcomm)]

This proposal presents a “copy from previous rows” mode for the palette coding. The proposed method enables to copy pixels from previously coded rows beyond the row above in the current CU. This is defined as a palette mode besides the existing “Copy from left”, “Copy above” and “Escape” modes. The reference row index is coded in the bitstream to indicate which row is copied from. The simulation results report that the proposed method achieves 0.9% and 1.0% BD-rate savings for 1080p text and graphics RGB and YUV respectively against the SCCE3 anchor in the full frame intra BC test condition.

This proposal adds a “copy from a previous row” mode and associated index. It is enabled from the third row onward, and is only used (via encoder) if the copy above mode is invalid.

Copying from escaped pixels is not allowed.

Recommendation: Consider for inclusion in palette-mode CE, if conducted.

JCTVC-R0213 Non-SCCE3: Palette and deblocking [J. Sole, C. Pang, W. Pu, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]

The deblocking filter is applied for palette coded blocks. However, generally, some kinds of filtering and smoothing processes are avoided for screen content coding. It is proposed to not deblock CUs coded with palette mode in the same way that PCM CUs and lossless CUs (i.e., cu_transquant_bypass_flag enabled) are not deblocked. It is asserted that this saves some complexity, avoids unwanted filtering and achieves BD-rate reductions (in the range of 0.0% to -0.1%). Additionally, a flag at SPS level could be included to enable/disable deblocking of palette coded CUs.

The proponent has bitstreams at the meeting for those interested in viewing the decoded video

It was suggested tht if palette is in SCM-2.0, some subjective testing related to deblocking should be done.

If intra block is next to PCM or lossless block, a strong filter would be applied to intra side of edge, and none on PCM/lossless, via adjusting boundary strength.

It was asked that if palette is adopted into SCM-2.0, what kind of deblocking should be done.

It was commented that deblocking with respect to transform skip hasn’t been looked at.

JCTVC-R0126 is on a related topic.

Recommendation: Study in AHG or CE with JCTVC-R0126

JCTVC-R0319 Cross check of ‘Non-SCCE3: Palette and deblocking’ (JCTVC-R0213) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0215 Non-SCCE3: WD text fixes for palette [J. Sole, R. Joshi, V. Seregin, W. Pu, M. Karczewicz (Qualcomm)]

It is asserted that there are some inconsistencies in the SCCE3 palette design. These inconsistencies have been detected and fixes are proposed. The issues include the SPS flag for palette mode, the signalling of the maximum palette size, and the signalling of the number of new entries.

(Question: How can WD text be fixed for something that is not included?)  (this comment is from meeting notes before BoG)

The Section 3 change for palette_num_signalled_entries was discussed earlier and will be included in the proposed combined software.

For Section 2, signalling the maximum palette size, it was asked if there’s benefit for signalling the max palette size vs. having it pre-defined.

Recommendation: For the Section 1 SPS flag to enable palette, consider this when drafting the text, and add this to the proposed combined software. Section 2 signalling max palette size: further study. Section 3 change for palette_num_signalled_entries: Include in proposed combined software. Also specify the maximum palette size in the proposed text for palette (currently 31).

JCTVC-R0318 Crosscheck of JCTVC-R0215 on WD text fixes for palette [Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek)] [late]

JCTVC-R0228 Non-SCCE3: Run-length coding for palette predictor [V. Seregin, M. Karczewicz, W. Pu, R. Joshi, J. Sole (Qualcomm)]

This contribution is related to the palette method studied in SCCE3 and method of signaling the palette entries predicted from the palette predictor formed from the palette entries of the previous coded blocks. It is proposed to use run-length coding to code the palette predictor reuse flags. Reportedly, it provides up to 3.6% luma BD-rate gain on top of the SCCE3 software base, and up to 11.8% luma BD-rate gain with combination of SCCE3 C4 test. Additionally, the palette predictor size is increased from 64 to 128. Increased palette predictor provides additionally up to 0.6% luma BD-rate gain.

SCCE3 Test A.8 (JCTVC-R0063) proposes a different way to code the reuse flags, via grouping. The method proposed here outperforms Test A.8 by approximately 0.1%-0.2%, with additional gains on top of the Test C.4 supplemental tests.

JCTVC-R0063 and this proposal are from the same proponents.

Recommendation: Perform in-meeting test on combined platform, and if performance improvement is still produced, include in the proposed combined software (assuming “non-SCCE” proposals are allowed to be in that software).

JCTVC-R0270 Cross-check of JCTVC-R0228 on Run-length coding for palette predictor (Non-SCCE3) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0229 Non-SCCE3: Palette predictor resetting [V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)]

This contribution is related to the palette method studied in SCCE3. In this method, the palette entries of the current CU can be predicted using the palette predictor consisting of the palette entries of the previous coded CUs. In current SCCE3 software, palette predictor is reset at each CTB row. This contribution proposes to perform the palette predictor reset in the beginning of each slice and tile, and in the beginning of each CTB row if wavefront is used. Otherwise, no palette predictor resetting is done. Removal of palette predictor resetting provides up to 1.2% luma BD-rate improvement.

The proposed combined software enables CTB row palette resetting if wavefront is enabled.

JCTVC-R0233 is related to this, so discussion of this will continue with that proposal.

JCTVC-R0271 Cross-check of JCTVC-R0229 on Palette predictor resetting (non-SCCE3) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0176 Non-SCCE3: Combination of subtest C.4 with new palette tools [R. Joshi, W. Pu, V. Seregin, M. Karczewicz, J. Sole (Qualcomm)]

In subtest C.4 of screen content coding experiment 3 (SCCE3), a combination of proposed tools from subtests A.7, A.8, B.12 and B.13 was tested. This document reports the combination of subtest C.4 with other new palette tools related to contexts for coding index runs and run signalling for binary palette prediction vector and encoder bugfixes. It is reported that BD-rate improvements up to 12.5% and 14.9% is achieved with respect to SCCE3 palette base under full-frame intra block copying and two CTU intra block copying, respectively. When palette predictor size of 128 is used, BD-rate improvements up to 12.9% and 15.3%, respectively, is reported.

This can be considered informational since we now have a proposed combined software for palette.

JCTVC-R0232 Non-SCCE3: high throughput palette coding [S. H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

This contribution proposes a high throughput palette coding method by reducing the number of context coded bins for palette coding. Specifically, for each CU, the number of context coded bins for palette_run is counted and context bins are replaced by bypass bins when the number of context coded bins exceeds a predetermined threshold. The proposed method doesn’t change binarization method for palette_run reducing the worst case context coded bins per pixel from 1.984375 bins/pixel to 1.5625 bins/pixel.  The results show that the average coding performance is 0.0% and BD-rate variation has been observed between 0.2% and -0.2%.

There were some comments from several participants that this may not be reducing the worst case conditions for the whole codec by just changing the process for palette coding.

The proponent said that the issue with palette coding is that all three components are being coded at once, which increases the worst case number of context-coded bins as compared to non-palette coding where the components are being coded separately.

A participant commented that decoder complexity is increased, and there may be other ways to reduce the number of context-coded bins.

Recommendation: Further study

JCTVC-R0332 Non-SCCE3: Cross-check of JCTVC-R0232 on high throughput palette coding [F. Zou, R. Joshi (Qualcomm)] [late] [miss]

JCTVC-R0233 Modifications to palette coding for tiles/slices/dependent slices/wavefronts [K. Misra, S.-H. Kim, J. Zhao, A. Segall (Sharp)]

This contribution proposes modifications to palette coding for harmonizing operation with tiles/slices/dependent slices/wavefronts:

1.	Prevent palette table prediction across tile/slice boundary

2.	For dependent slice segments, define a storage and synchronization process for the palette table predictor

3.	For wavefronts, define storage and synchronization process for palette table predictor

Item 1 is the same as in JCTVC-R0229

Item 2 (not in JCTVC-R0229). It was noted that the JCTVC-Q0094BF text doesn’t clearly define the synchronization or prediction across dependent slice segments. Considering adding this to the WD text for palette if palette is adopted.

Item 3 The current software and text always reset per CTB row. The BOG’s proposed combined software will reset per CTB row only when wavefront is used. A participant said that Item 3 is a more complex solution than what’s proposed in the BoG combined software, and we don’t know yet what the BD-Rate performance of this item is.

Recommendations for both JCTVC-R0229 and JCTVC-R0233:

- Prevent palette prediction across slice or tile boundaries by resetting the palette predictor size to be zero at the start of slice and tile boundaries: Include in proposed palette text and software if palette is adopted. (JCTVC-R0229 and JCTVC-R0233)

- Allow palette prediction across (dependent?) slice segments of the same slice: Consider adding this to the text and software if palette is adopted. (JCTVC-R0233)

- Reset palette predictor at the start of CTB row if wavefront is enabled (JCTVC-R0229): This is currently planned to be added to the proposed combined software. Whether this is controlled by the wavefront flag or the method proposed in JCTVC-R0229 can be studied in an AHG for palette mode, if established.

- For item 3 of JCTVC-R0233, further study perhaps in AHG is needed.

JCTVC-R0234 Palette coding and scaling lists [K. Misra, S.-H. Kim, J. Zhao, A. Segall (Sharp)]

This contribution proposes modification to palette coding asserted to harmonize operation with scaling lists. The ESCAPE pixel value in palette coding uses the existing HEVC quantization process. However, use of the scaling list is part of this process. It is proposed that when palette coding mode is used then the scaling lists are set to be flat.

It was noted that flat scaling lists are used for transform skip, too.

There’s no scaling list defined for 64x64 CUs, so this may affect this proposed method, too.

No implementation or simulation results were provided.

No action

JCTVC-R0237 Non-SCCE3: Delta QP signalling in palette mode [J. Sole, R. Joshi, W. Pu, V. Seregin, M. Karczewicz (Qualcomm)]

In the previous JCT-VC meeting, SCCE3 was established to study palette related coding tools. The palette modes in SCCE3 code escape values using the coefficient quantization parameter, QP. However, there is no mechanism to vary this QP in CUs coded with the palette mode, so an encoder has to work at constant QP as long as CUs are coded in palette mode. SCCE3 test B.13 proposes a CU-level escape flag that indicates whether escape pixels exist in the current CU. This document proposes to use CU esacape flag as entry point for the syntax elements that adjust the QP (like the cbf flags in residue coding). That is, when CU-level escape flag is on, then the delta QP value can be signalled. In this way, encoders have the flexibility to use the palette mode and adjust the QP.

The current palette software uses the slice-level QP.

It was asked whether there is benefit from this method. The proponent said that this method could be used, for example, to get better quality over text, and it can be used to achieve lossless coding.

The proponent noted that there’s a need for a method to control the quality of these escape pixel values. Currently there is no way to do that.

Recommondation: Discuss briefly in plenary to get other opinions, to see if it is of sufficient interest to be added to the combined software now or if it should first be implemented and cross-checked.

JCTVC-R0266 Non-SCCE3: Palette Table Prediction by the Nearest Palette Table [C.-H. Hung, C.-L. Lin, Y.-J. Chang, C.-C. Lin, J.-S. Tu (ITRI)] [late]

Palette coding analyzes the major colors of a CU. Then it creates a palette table and represents each pixel by corresponding color index. Palette tables from neighboring CUs may have many consistent major colors. This document proposes a prediction algorithm to predict the major color table by the nearest ones. Compared with SCCE3 reference software (SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047) under All intra, lossy, and 2 left CTUs IBC search, the BD rates are -1.2% and -1.1% for “RGB, text & graphics with motion, 1080p” and “YUV, text & graphics with motion, 1080p” respectively. For All intra, lossy, and full frame IBC search condition, BD rates of above two test sequence groups are all -0.8%.

This method was also combined with palette stuffing, and it is asserted to yield an additional 0.5% improvement in BD-Rate.

Recommendation of BoG: Further study with possibly related proposals

JCTVC-R0286 Cross-check of JCTVC-R0266: Non-SCCE3: Palette Table Prediction by the Nearest Palette Table [S. H. Kim, A. Segall (Sharp)] [late]

JCTVC-R0268 Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer [Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei R&D USA)] [late]

This contribution reports the additional data to SCCE Test C.3. The technical difference is that it uses a larger search range for string matching in this contribution, i.e., 3 left CTUs and upper 4 CTUs to the current CTU, while in SCCE Test C.3 only 1x4 CTUs are used for string search.

Four tests were performed, with various search ranges for the string matching and for IBC.

This method applies to the string-matching method for coding color indices.

Average losses in performance as compared to Test C.3 with full-search IBC (averaged over the set of RGB text & graphics and mixed content sequences) range from 2.1% to 2.3% for lossy all-intra conditions, and 1% to 2.5% for lossless all-intra conditions.

Recommendation: Further study with other Test C-related tools and IBC search range variation studies.

JCTVC-R0277 Non-SCCE3: Cross-check of JCTVC-R0268 SCCE3 with support of 2x4 CTUs as reference buffer [P. Lai, X. Xu (MediaTek)] [late]

JCTVC-R0304 Non-SCCE3: Improvements for SCCE3 Test C3 [Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei R&D USA)] [late]

This contribution reports the results of a modified SCCE3 Test C.3 where a new color mode is further introduced and the SCCE3 A.6 method is borrowed to select the colors using Rate-Distortion optimization.

The modifications are to the color table derivation and addition of a color mode for index coding.

For color table derivation, the color table encoding uses the same method as in the JCT-VC R0144 (same proponents), and escape pixels are applied without HEVC residual coding in contrast to SCCE3 C.3.

This method applies to the string-matching method for coding color indices.

Compared to Test C.3, this method produces approx. 5% - 8% improvement in BD-Rate for all-intra lossy test conditions.

Recommendation: Further study with other Test C-related tools and IBC search range variation studies.

JCTVC-R0331 Non-SCCE3: Cross-verification of Improvements for SCCE3 Test C3 (JCTVC-R0304) [M. Zhou (Broadcom)] [late] [miss]

JCTVC-R0306 SCCE3 base Software Bugfix Report [W. Pu, J. Sole, R. Joshi, F. Zou (Qualcomm), T.-D. Chuang, Y.-C. Sun (MediaTek), Y. He, X. Xiu, Y. Ye (InterDigital), X. Zhang, R. Cohen (MERL)] [late]

In the previous JCT-VC meeting, SCCE3 was established to study palette related coding tools. Palette mode was implemented on top of SCM1.0 as a basis for SCCE3 in JCTVC-Q0094. This proposal reports the reported bugs and fixes on top of this software.

3 of the 4 bugfixes are encoder-only.

- According to palette mode syntax description below, if a CU is palette mode, the following syntax elements such as part_mode should be bypassed. However, in the encoder function xEncIntraHeader, this logic was not correctly implemented.

- When counting the palette mode bits, the CABAC encoder was not properly reset.

- According to the working draft below, palette_mode_flag is not signaled in intraBC mode. However, in xAddSymbolBitsInter function, palette_mode_flag was counted even if the current mode is intraBC.

- According to the working draft below, the syntax element palette_num_signalled_entries is only signalled when the numPredPreviousPalette is less than the max_palette_size. However, this conditional check was not impelemented in the software.

The change in compression performance is 0% - 0.1% (mostly on the order of a few hundredths of a percent).

JCTVC-R0311 Non-SCCE3: Combination of palette tools [Y. He, X. Xiu, Y. Ye (InterDigital), R. Joshi, V. Seregin, W. Pu, M. Karczewicz, J. Sole (Qualcomm)] [late]

A combination of palette methods in SCCE3 is tested. Compared with SCM1.0, it is reported that the proposed method provides BD-rate improvements of (22.4%, 21.9%, 21.7%) and (22.5%, 25.3%, 24.5%) for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration with full frame IntraBC search, respectively, and the performance with respect to SCM1.0+SCCE3 software is BD-rate improvements of (13.1%, 12.7%, 12.5%) and (13.4%, 15.0%, 14.8%), respectively, for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration with full frame IntraBC search.

This is a combination of 7 methods, related to palette mode.

This can be considered informational since we now have a proposed combined software for palette.

JCTVC-R0329 Crosscheck of Non-SCCE3 on Combination of Palette Tools (JCTVC-R0311) [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late] [miss]

JCTVC-R0317 Non-SCCE3: memory reduction for palette mode software [C. Gisquet, G. Laroche, P. Onno (Canon), T.-D. Chuang (MediaTek)]

In the scope of SCCE3, concerns were raised over the palette mode software memory allocations. This contribution reports the investigations on reducing these allocations. It is further reported that the suggested changes result in no change to the generated bitstreams. It is therefore asserted that simple changes can be applied to reduce these memory allocations, and furthermore, that the core issue is actually not within the palette mode algorithm, but it is due to the particular software implementation and the fact that anything added to the CU class gets saved and copied many times in the software.

The changes in palette-related software include reducing the declared types of some arrays (e.g. from 2 bytes to 1 bytes); removed or merged some redundant or unused arrays; changed allocation of m_piPLT and related variables to be on 8x8 CU basis instead of 4x4; reused existing buffers.

Prior to these changes, the memory increase was 78% for RA conditions in SCCE3 palette software vs SCM 1.0. With these changes, the memory increase is 10% over SCM 1.0. For example, prior to these changes, for 720p sequences the memory usage is up to about 1 GB with palette, 560 MB without palette; with these changes the memory usages is up to 625 MB and 560 MB without palette.

These changes did not alter the bitstreams.

A participant commented that in the original HM, there are 2 lossless coding schemes. The cu-transquant-bypass mode may use a buffer for another purpose. A proponent commented that the changes here don’t affect that.

Recommendation: Include these changes in the proposed combined palette software.