| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0348 |
| --- | --- |

| Title: | Suggested combined software and text for run-based palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrice Onno Xiaoyu Xiu Yu-Wen Huang Rajan Joshi | Emails: | patrice.onno@crf.canon.fr xiaoyu.xiu@interdigital.com yuwen.huang@mediatek.com rajanj@qti.qualcomm.com |
| Source: | Canon Research Centre France, InterDigital Communications, Inc., MediaTek Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

During the Sapporo meeting, the SCCE3 BoG provided a new run-based palette mode, which combines several SCCE3 run-based palette techniques with reasonable gain-complexity tradeoffs reported in the SCCE3 summary report. It is shown that the BD-rates savings achieved by the combined palette mode under full frame intra block copy (FF-IBC) and all intra conditions when compared against JCTVC-Q0094BF palette mode (and palette mode off) are as follows:
12.1%		(21.5%)	for RGB, text & graphics with motion, 1080p
10.7%		(15.8%)	for RGB, text & graphics with motion, 720p
5.8%		(6.9%)	for RGB, mixed content, 1440p
5.7%		(7.7%)	for RGB, mixed content, 1080p
12.3%		(21.4%)	for YUV, text & graphics with motion, 1080p
6.9%		(11.1%)	for YUV, text & graphics with motion, 720p
4.5%		(5.4%)	for YUV, mixed content, 1440p
5.1%		(7.3%)	for YUV, mixed content, 1080p
In comparison with SCCE3 category C palette modes and their non-CE variants, about 1-4% additional BD-rate savings are observed, while overall complexity is reduced according to conducted analysis.

On top of the combined palette mode, a simplification was further studied: horizontal scan and vertical scan are removed, and only horizontal traverse scan and vertical traverse scan are left. It is shown that the BD-rate increases are only 0-0.1% with this simplification.

Based on the above results, it is proposed to adopt this simplified combined palette mode into SCM-2.0.

# Introduction

The HEVC Screen Content Core Experiment 3 (SCCE3) [1] was established at the 17th JCT-VC meeting to study palette mode techniques according to the screen content test conditions [2]. At the 18th JCT-VC meeting, a break out group (BoG) on SCCE3 palette coding mode [3] was formed. The first two mandates of the BoG were 1) analyze the category C proposals in terms of complexity; 2) suggest and perform a similar analysis for new useful combinations of category A and B methods, in order to come up with a combination that provides a reasonable performance vs. complexity tradeoff. This document will describe the suggested combination and provide the analysis results in the following sections.

# Combined palette mode

On top of the JCTVC-Q0094BF palette mode described in the SCCE3 summary report, JCTVC-R0033, the following items were agreed to be integrated to produce the combined palette mode.

Category A, palette table coding techniques:

- Palette sharing, Test A2/A4/A5 (JCTVC-R0119/R0142/R0166)

- Palette stuffing, Test A3/A4/A5/A8 (JCTVC-R0082/R0142/R0166/R0063)

- Grouping reuse flags, Test A8 (JCTVC-R0063)

- Encoder-only palette generation, Test C2/C4 supplemental (JCTVC-R0086/R0175)

Category B, color index map coding techniques:

- Vertical scan, Test B4/B8/B9/B15 (JCTVC-R0057/R0143/R0168/R0048)

- Redundant index removal, Test B5/B10/B12 (JCTVC-R0083/R0169/R0065)

- Truncated binary codes for color indices, Test B5/B12 (JCTVC-R0083/R0065)

- Escape color values coded as bypass, Test C4 (JCTVC-R0067)

- Traverse scans, Test B9 (JCTVC-R0168)

Other techniques:

- Palette reset per CTU row for WPP-on only, Test C4 (JCTVC-R0067)

- Bugfixes of RDO (JCTVC-R0306)

- Bugfix of palette_num_signalled_entries (JCTVC-R0215)

- Software memory reduction (JCTVC-R0307)

The combined software was released and uploaded. The item of software memory reduction, which does not change any bitstream, was excluded from the combined software in order to distribute the software early. This item should be added later.

# Simulation results of tests planned by the BoG

Since palette mode is an intra coding tool, FF-IBC and all intra conditions were agreed to be sufficient for making related decisions. Test results under FF-IBC and all intra conditions are as follows.

## JCTVC-Q0094BF palette mode versus palette mode off

## Combined palette mode versus palette mode off

## Combined palette mode versus JCTVC-Q0094BF palette mode

## Disabling “palette sharing” versus combined palette mode

Note: Run time percentages are inaccurate.

## Disabling “palette stuffing and grouping reuse flags” versus combined palette mode

One of the original BoG test plans was to study disabling “palette stuffing” versus the combined palette mode. However, “grouping reuse flags” will be also removed if “palette stuffing” is removed due to the current implementation of the combined palette mode. Therefore, disabling “palette stuffing and grouping reuse flags” was tested instead.

Note: Run time percentages are inaccurate.

## Disabling “ grouping reuse flags” versus combined palette mode

## Disabling “encoder-only palette generation” versus combined palette mode

## Disabling “vertical scan and vertical traverse scan” versus combined palette mode

One of the original BoG test plans was to study disabling “vertical scan” versus the combined palette mode. However, “vertical traverse scan” will be also removed if “vertical scan” is removed due to the current implementation of the combined palette mode. Therefore, disabling “vertical scan and vertical traverse scan” was tested instead.

Note: The combined software contains horizontal, vertical, horizontal traverse, and vertical traverse scans, while the tested software contains only horizontal and horizontal traverse scans.

## Disabling “redundant index removal” versus combined palette mode

## Disabling “truncated binary codes for color indices” versus combined palette mode

## Disabling “escape color values coded as bypass” versus combined palette mode

## Disabling “traverse scans” versus combined palette mode

Note: Run time percentages are inaccurate.

Note: The combined software contains horizontal, vertical, horizontal traverse, and vertical traverse scans, while the tested software contains only horizontal and vertical scans.

## Disabling “palette reset per CTU row for WPP-on only” versus combined palette mode

## Disabling “bugfixes of RDO and bugfix of palette_num_signalled_entries” versus combined palette mode

Two of the original BoG test plans were to study disabling “bugfixes of RDO” versus the combined palette mode and to study disabling “bugfix of palette_num_signalled_entries” versus the combined palette mode. Due to insufficient resource during the Sapporo meeting period, disabling “bugfixes of RDO and bugfix of palette_num_signalled_entires” versus combined palette mode was tested instead.

Note: Run time percentages are inaccurate.

# Complexity analysis of palette modes

Please see JCTVC-R0348_PaletteModeComplexityAnalysis.xlsx. The combined palette mode does not require any off-chip memory bandwidth or additional on-chip memory. No CABAC parsing throughput issue or parsing dependency issue is found.

# Proposed simplification on top of the combined palette mode

In the combined palette mode, the color index map coding can use horizontal, vertical, horizontal traverse, and vertical traverse scans. It is proposed to remove the horizontal and vertical scans and support only horizontal traverse and vertical traverse scans.

# Simulation results of simplified combined palette mode

The test results of the simplified combined palette mode in comparison against the combined palette mode under FF-IBC and all intra conditions are as follows.

Note: Run time percentages are inaccurate.

Note: The combined software contains horizontal, vertical, horizontal traverse, and vertical traverse scans, while the simplified software contains only horizontal traverse and vertical traverse scans.

# Conclusions

In comparison with SCCE3 category C palette modes and their non-CE variants, about 1-4% additional BD-rate savings are observed for the newly combined palette mode of the BoG, while overall complexity is reduced. Then, a simplified combined palette mode is proposed to remove horizontal and vertical scans and keep only horizontal traverse and vertical traverse scans with 0-0.1% BD-rate increases. It is proposed to adopt the simplified combined palette mode into SCM-2.0.

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “SCCE3: Summary report of CE on palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0033, 18th Meeting: Sapporo, JP, June-July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1015, 17th Meeting: Valencia, ES, March-April 2014.

- R. Cohen, Y.-W. Huang, “BoG on SCCE3 palette coding mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0346, 18th Meeting: Sapporo, JP, June-July 2014.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).