| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0352 |
| --- | --- |

| Title: | Proposed Profile, Tier, Level Conformance for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Ye-Kui Wang Miska Hannuksela | Email: | jill@vidyo.com |
| Source: | Vidyo, Qualcomm, Nokia |  |  |

_____________________________

# Abstract

A high level description of a proposed scheme for Profile, Tier, Level Conformance for SHVC is provided.  Modifications are proposed to Scalable Main and Scalable Main 10 profiles and to the generic level limits and profile-specific level limits.  A new Scalable High 10 profile is also proposed.

# Proposal

The proposal can be summarized as follows.

- Scalable Main and Scalable Main 10 profiles

  - Bitstream conformance point (profile, tier, level - PTL) constraints required to be signaled per layer

    - Additional PTL bitstream constraints may be signaled for partitions in one or more partitionings

    - Decoder conformance based upon per layer PTL signaling

- Scalable High profile

  - Bitstream conformance point PTL constraints signalled for each partition of one or more partitionings

  - Decoder conformance based upon conformance point per partition PTL signalling constraints of any conforming partitioning

The motivations behind the design are as follows.

- Scalable Main profile

  - Enables simple re-purposing of single layer cores

    - Each core only required to decode a single layer

  - Simple definition of conformance

  - Allows any number of layers, selectable by decoder

- Scalable High profile

  - Advantageous when each layer does not utilize full capabilities of a level

  - Enables re-purposing of a single layer core to decode multiple layers when total requirements (bitrate, sample rate, pictures size) fit within a level

  - Particularly beneficial with many layers

The partitionings concept is more fully described in JCTVC-R0043.

More description of the proposal can be found in the attached slide deck.

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).