| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0353 |
| --- | --- |

| Title: | Crosscheck for Modulation channel SEI message (JCTVC-R0139), Frame packing mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Koohyar Minoo 6450 Sequence Dr. San Diego, CA, 92122 U.S.A. | Tel: Email: | +1-858-404-2521 Koohyar.minoo@arrisi.com |
| Source: | Arris Group Inc. |  |  |

_____________________________

# Abstract

This document provides a crosscheck for the input contribution JCTVC-R0139. Specifically this cross-check is concerned with the frame packing approach of the aforementioned proposal. The results of this cross check, confirms the number provided by proponents.

# Introduction

JCTVC-R0139 proposes to compress high-bit depth HDR content with existing low-bit-depth profiles (8 and 10) of HEVC and its extensions. This is achieved by means of de-composing the HDR to two low-bit-depth signals. One signal is a modulation signal and the other one is a low-bit-depth representation of content.

The specific method for encoding the two signals, which is examined in this crosscheck, is by means of frame-packing and a SEI message.

# Processing flow

## Pre-processing

The original sequences, in the OpenEXR (half float) format, were in RGB (BT.709) domain. Using a software tool provided by the proponents, corresponding 16bits sequences were generated in the xyz domain.

## Encoding

Using a modified HM-13.0+RExt-6.0rc1, the xyz sequences were encoded as follows:

- Each picture of the xyz 16bits sequence was converted to two pictures, each 10bits. A modulation picture, and a low-bit-depth. The low-bit depth picture for this crosscheck was in Lab space.

- The two pictures were tiled in a top/bottom format and passed to the encoder RExt encoder.

- Encoder embedded the frame-packing SEI message for synthesis of coded pictures.

## Decoding

Using a modified version of HM-13.0+RExt-6.0rc1, the coded bit stream was converted to a reconstructed xyz (16bits) sequence, using the information in the frame packing SEI message.

## Measuring performance

Performance was measured by comparing the reconstructed xyz at the output of the decoder with the xyz sequence fed to the input of the encoder.

## Results

The attached datasheet contains the results which match the one provided by in JCTVC-R0139 contribution, reported for the same Windows platform.

# Patent rights declaration(s)

Arris Group does not have any current or pending patent rights relating to the technology described in this contribution.