| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0355_r1 |
| --- | --- |

| Title: | AHG11: Suggested editorial improvement on TMCTS-SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami, Sally Hattori, Teruhiko Suzuki Sony City Osaki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com shinobu.hattori@jp.sony.com teruhikos@jp.sony.com +1-408-352-4086 ali.tabatabai@am.sony.com |
| Source: | Sony corp. |  |  |

_____________________________

# Abstract

This contribution suggests the editorial improvement regarding the level/tier aspects in Temporal Motion-constrained tile sets SEI on top of JCTVC-Q1008_v5.

In the revision1, the review result on 6 July 2014 is reflected with green highlight. The items are listed below.

  - Decision: In the discussion it was suggested, and agreed, to constrain num_tile_rects_in_set_minus1 to 0 when the tier and level information is present.

  - Add constraint that "Require the NAL units of the tiles in the MCTS to not include any tiles that are not in the MCTS when this tier/level information is indicated."

  - Update the equation related to "Sample of variable derivation process" according the decision that " num_sets_in_message_minus1 is equal to 0".

  - The counting for the NumBytesInNalUnit is changed to “the sum” counting.

  - Further cleanup on the derivation process of the interim variables.

# Remark

It is remarked that adding the blue highlighted encoder constraint was agreed in the 16th San Jose meeting on adopting JCTVC-P0051_v2.

mcts_tier_level_idc_present_flag[ i ] equal to 1 specifies that the mcts_tier_flag[ i ] and mcts_level_idc[ i ] syntax elements are present. mcts_tier_level_idc_present_flag equal to 0 specifies that the mcts_tier_flag[ i ] and mcts_level_idc[ i ] syntax elements are not present. When mcts_tier_level_idc_present_flag[ i ] is equal to 1, the i-th motion-constrained tile set shall form a rectangular shape.

The related discussion in JCTVC-P_Notes_d9.doc is cited. The corresponding sentences are blue highlighted.

JCTVC-P0051 HLS: Extensions to Temporal Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami (Sony)]

Discussed 01-12 p.m. (GJS & JRO).

This contribution proposes an extension to a temporal motion-constrained tile sets SEI message to indicate the level information for a decoder to decode each defined motion-constrained tile set. The proposal provides flexibility for HEVC tile structure to be applied for various applications such as in interactive UHDTV application, dynamic high-quality zoom-in application and interactive on-demand e-learning etc. The idea was proposed in JCTVC-N0117 and JCTVC-O0063. The functionality of indicating the level information for each motion-constrained tile set was agreed to be useful with proposal in JCTVC-N0117 and JCTVC-O0063. This contribution further clarifies the specification text on the definitions of bit rate level constraints for motion-constrained tile sets.

The SEI message is intended for single-layer bitstreams.

For an associated level indicator, the proposal constrains the size of the bounding rectangle that includes the entire MCTS and counts the bits only for the tiles in the MCTS (which may not be rectangular).

A suggestion in the discussion was to instead require that any MCTS that has an indicated level value must contain only one tile rectangle.

As proposed, the proposal would require, when a level_idc is indicated for some MCTS, it would need to be indicated for all MCTSs in the SEI message (and, as a consequence, all MCTSs in the SEI message would need to be rectangular). An alternative would be to send a flag for each MCTS to indicate whether it has an associated level or not, or to define a particular value of level_idc (e.g., 0) as an indication that a level is not identified.

It was discussed whether we would need to specify this in terms of a bitstream rewriting process. A complete specification for this might not be necessary.

It was remarked that it may be desirable to have a way to deal with the lack of a high tier for some levels. Adding a tier flag was suggested.

In regard to bit rate, the constraint for the level indicator would be for the VCL NAL units of the MCTS.

VBR would need to be assumed for the HRD, with the maximum allowed bit rate for that level and the maximum allowed CPB capacity for the indicated level. Initial CPB removal delay the same as in the overall bitstream. Other aspects (DPB removal delays, CPB removal delays, etc.) as in the containing bitstream.

It was remarked that a constraint would be needed to establish that the set of slices that contain the tiles in the MCTS cannot contain any other tiles (or CTUs), so that the MCTS is constructed from whole NAL units (when a level indication is provided).

Simplifications:

- have a presence flag for each level_idc

- have a tier flag for each level_idc

- only one tile rectangle for each level_idc

Decision: Adopted.

(Some further editorial work may be needed to clarify the conformance to the indicated level.)

# Suggested editorial improvement

The editorial improvement is suggested as follows

The editor’s question and comment are highlighted in the yellow.

The suggested change is highlighted in the blue.

.

D.2.25	Temporal motion-constrained tile sets SEI message syntax

| temporal_motion_constrained_tile_sets( payloadSize ) { | Descriptor |
| --- | --- |
| mc_all_tiles_exact_sample_value_match_flag | u(1) |
| each_tile_one_tile_set_flag | u(1) |
| if( !each_tile_one_tile_set_flag ) { |  |
| limited_tile_set_display_flag [Ed. Sony thing.] | u(1) |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| if( limited_tile_set_display_flag ) |  |
| display_tile_set_flag[ i ] | u(1) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| if( !mc_all_tiles_exact_sample_value_match_flag ) |  |
| mc_exact_sample_value_match_flag[ i ] | u(1) |
| mcts_tier_level_idc_present_flag[ i ] [Ed. Sony thing] | u(1) |
| if( mcts_tier_level_idc_present_flag[ i ] ) { |  |
| mcts_tier_flag[ i ] | u(1) |
| mcts_level_idc[ i ] | u(8) |
| } |  |
| } |  |
| } else { |  |
| max_mcts_tier_level_idc_present_flag | u(1) |
| if( max_mcts_tier_level_idc_present_flag ) { |  |
| max_mcts_tier_flag | u(1) |
| max_mcts_level_idc | u(8) |
| } |  |
| } |  |
| } |  |

D.3.25	Temporal motion-constrained tile sets SEI message semantics

The temporal motion-constrained tile sets SEI message indicates that the inter prediction process is constrained such that no sample value outside each identified tile set, and no sample value at a fractional sample position that is derived using one or more sample values outside the identified tile set, is used for inter prediction of any sample within the identified tile set.

Let a set of pictures associatedPicSet be the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message, inclusive, up to but not including the first of any of the following in decoding order:

–	The next access unit, in decoding order, that contains a temporal motion-constrained tile sets SEI message applicable to targetLayerId.

–	The next IDR or BLA picture, in decoding order, with nuh_layer_id equal to targetLayerId.

–	The next IRAP access unit, in decoding order, with NoClrasOutputFlag equal to 1.

The scope of the temporal motion-constrained tile sets SEI message is the set of pictures associatedPicSet.

When a temporal motion-constrained tile sets SEI message is present for any picture in associatedPicSet, a temporal motion-constrained tile sets SEI message applicable to targetLayerId shall be present for the first picture of associatedPicSet in decoding order and may also be present for other pictures of associatedPicSet.

The temporal motion-constrained tile sets SEI message applicable to targetLayerId shall not be present for any picture in associatedPicSet when tiles_enabled_flag is equal to 0 for any PPS that is active for any picture in associatedPicSet.

The temporal motion-constrained tile sets SEI message applicable to targetLayerId shall not be present for any picture in associatedPicSet unless every PPS that is active for any picture in associatedPicSet has the same values of the syntax elements num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_flag, column_width_minus1[ i ], and row_height_minus1[ i ].

NOTE 1 – This constraint is similar to the constraint associated with tiles_fixed_structure_flag equal to 1, and it may be desirable for tiles_fixed_structure_flag to be equal to 1 when the temporal motion-constrained tile sets SEI message is present (although this is not required).

NOTE 2 – When loop filtering is applied across tile boundaries, inter prediction of any samples within a temporal motion-constrained tile set that refers to samples within four samples from a temporal motion-constrained tile set boundary that is not also a picture boundary may result in propagation of mismatch error. An encoder can avoid such potential error propagation by avoiding the use of motion vectors that cause such references.

When more than one temporal motion-constrained tile sets SEI message applicable to targetLayerId is present for the pictures of AssociatedPicSet, they shall contain identical content.

The number of temporal motion-constrained tile sets SEI messages applicable to the same nuh_layer_id value in each access unit shall not exceed 5.

mc_all_tiles_exact_sample_value_match_flag equal to 0 indicates that when the coding tree units that are outside any identified tile are not decoded and the boundaries of the identified tile are treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile may not be exactly the same as the value of the same sample when all the coding tree units of the picture are decoded. mc_all_tiles_exact_sample_value_match_flag equal to 1 indicates that when the coding tree units that do not belong to any tile in the tile set are not decoded and the boundaries of the identified tile are treated as picture boundaries for purposes of the decoding process, the value of each sample in the tile would be exactly the same as the value of the sample that would be obtained when all the coding tree units of all pictures in associatedPicSet are decoded. [Ed. (GJS): What is an "identified tile"? Change to "tile in the MCTS" or define "identified tile" as meaning that]

each_tile_one_tile_set_flag equal to 1 indicates that each tile is a separate temporal motion-constrained tile set.

limited_tile_set_display_flag equal to 1 specifies that the display_tile_set_flag[ i ] syntax element is present and indicates that the tiles not included within any tile set with display_tile_set_flag[ i ] equal to 1 are not intended for display. limited_tile_set_display_flag equal to 0 specifies that the display_tile_set_flag[ i ] syntax element is not present.

num_sets_in_message_minus1 plus 1 specifies the number of temporal motion-constrained tile sets identified in the SEI message. The value of num_sets_in_message_minus1 shall be in the range of 0 to 255, inclusive.

mcts_id[ i ] contains an identifying number that may be used to identify the purpose of the i-th identified tile set (for example, to identify an area to be extracted from associatedPicSet for a particular purpose). The value of mcts_id[ i ] shall be in the range of 0 to 232 − 2, inclusive.

Values of mcts_id[ i ] from 0 to 255, inclusive, and from 512 to 231 − 1 inclusive, may be used as determined by the application. Values of mcts_id[ i ] from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of mcts_id[ i ] in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore (remove from the bitstream and discard) it.

display_tile_set_flag[ i ] equal to 1 indicates that the i-th tile set is intended for display. display_tile_set_flag[ i ] equal to 0 indicates that the i-th tile set is not intended for display. When not present, the value of display_tile_set_flag[ i ] is inferred to be equal to 1.

num_tile_rects_in_set_minus1[ i ] plus 1 specifies the number of rectangular regions of tiles in the i-th identified temporal motion-constrained tile set. The value of num_tile_rects_in_set_minus1[ i ] shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive.

top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified temporal motion-constrained tile set, respectively, in tile raster scan order.

mc_exact_sample_value_match_flag[ i ] equal to 0 indicates that when the coding tree units that are outside of the i-th identified temporal motion-constrained tile set are not decoded and the boundaries of the temporal motion-constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the identified tile set may not be exactly the same as the value of the same sample when all the coding tree units of the picture are decoded. mc_exact_sample_value_match_flag[ i ] equal to 1 indicates that when the coding tree units that do not belong to the temporal motion-constrained tile set are not decoded and the boundaries of the temporal motion-constrained tile set are treated as picture boundaries for purposes of the decoding process, the value of each sample in the temporal motion-constrained tile set would be exactly the same as the value of the sample that would be obtained when all the coding tree units of all pictures in associatedPicSet are decoded.

NOTE 3 – It should be feasible to use mc_exact_sample_value_match_flag equal to 1 when using certain combinations of loop_filter_across_tiles_enabled_flag, pps_loop_filter_across_slices_enabled_flag, pps_deblocking_filter_disabled_flag, slice_loop_filter_across_slices_enabled_flag, slice_deblocking_filter_disabled_flag, sample_adaptive_offset_enabled_flag, slice_sao_luma_flag, and slice_sao_chroma_flag.

mcts_tier_level_idc_present_flag[ i ] equal to 1 specifies that the mcts_tier_flag[ i ] and mcts_level_idc[ i ] syntax elements are present. mcts_tier_level_idc_present_flag[ i ] equal to 0 specifies that the mcts_tier_flag[ i ] and mcts_level_idc[ i ] syntax elements are not present.

When mcts_tier_level_idc_present_flag[ i ] is equal to 1, num_tile_rects_in_set_minus1[ i ] shall be equal to 0 and  a slice segment containing one or more coding tree units within the i-th motion-constrained tile set shall not include any coding tree unit outside the i-th motion-constrained tile set.[Ed. (JC), rephrasing this constraint, need verification]

mcts_tier_flag[ i ] specifies the tier context for the interpretation of mcts_level_idc[ i ] corresponding to the i-th motion-constrained tile set. mcts_tier_flag[ i ] equal to 0 indicates conformance to the Main tier, and mcts_tier_flag[ i ] equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A-1. mcts_tier_flag[ i ] shall be equal to 0 -when mcts_level_idc[ i ] is less than 120 [Ed. (ON): 120 / 30 indicates level 4]. When not present, the value of mcts_tier_flag[ i ] is inferred to be equal to general_tier_flag.

mcts_level_idc[ i ] indicates a level to which the i-th tile set region, corresponding to the i-th motion-constrained tile set, conforms. The value of mcts_level_idc[ i ] shall be less than or equal to the value of general_level_idc in the active SPS RBSP. When not present, the value of mcts_level_idc[ i ] is inferred to be equal to general_level_idc.

max_mcts_tier_level_idc_present_flag equal to 1 specifies that the max_mcts_tier_flag and max_mcts_level_idc syntax elements are present. max_mcts_level_idc_present_flag equal to 0 specifies that the max_mcts_tier_flag and max_mcts_level_idc syntax elements are not present.

When max_mcts_level_idc_present_flag is equal to 1, a slice segment containing one or more coding tree units within a tile shall not include any coding tree unit outside the tile.[Ed. (JC), rephrasing this constraint, need verification]

max_mcts_tier_flag specifies the tier context for the interpretation of max_mcts_level_idc to which all motion-constrained tile sets conform. max_mcts_tier_flag equal to 0 indicates conformance to the Main tier, and max_mcts_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A-1. max_mcts_tier_flag shall be equal to 0 when max_mcts_level_idc is less than 120 [Ed. (ON) 120 / 30 indicates level 4] - [Ed. (JC) please confirm the modification]. When not present, the value of max_mcts_tier_flag[ i ] is inferred to be equal to general_tier_flag.

max_mcts_level_idc indicates a level to which all motion-constrained tile sets conform. The value of max_mcts_level_idc shall be less than or equal to the value of general_level_idc in the active SPS RBSP. When not present, the value of max_mcts_level_idc is inferred to be equal to general_level_idc.

The following describes the tier and level restrictions on the bitstreams of each motion-constrained tile set:

–	If mcts_tier_level_idc_present_flag[ i ] and max_mcts_tier_level_idc_present_flag are both equal to 0, the mctsLevelIdc of all motion-constrained tile sets are inferred to be equal to general_level_idc and the mctsTierFlag of all motion-constrained tile sets are inferred to be equal to general_tier_flag and the specifications of Annex A apply to all motion-constrained tile sets.

–	Otherwise (mcts_tier_level_idc_present_flag[ i ] or max_mcts_tier_level_idc_present_flag is equal to 1), the following applies:

–	The variables mctsLevelIdc[ i ], mctsTierFlag[ i ], mctsWidthInSamplesY[ i ], mctsHeightInSamplesY[ i ], NumTileColumnsInMCTS[ i ] and NumTileRowsInMCTS[ i ], which indicate the level and tier constraint of the i-th motion-constrained tile set, are derived as follows:

–	If each_tile_one_tile_set_flag is equal to 0, for each tile set with index i in the range of 0 to num_sets_in_message_minus1, inclusive, the following applies:

–	mctsLevelIdc[ i ] is set equal to mcts_level_idc[ i ].

–	mctsTierFlag[ i ] is set equal to mcts_tier_flag[ i ].

–	mctsWidthInSamplesY[ i ] is set equal to the sum of the ColumnWidthInLumaSamples[ k ], for all k in the range of ( top_left_tile_index[ i ][ 0 ] % ( num_tile_rows_minus1 + 1 ) ) to ( bottom_right_tile_index[ i ][ 0 ] % ( num_tile_rows_minus1 + 1 ) ), inclusive.

–	mctsHeightInSamplesY[ i ] is set equal to the sum of the RowHeightInLumaSamples [ k ], for all k in the range of ( top_left_tile_index[ i ][ 0 ] / ( num_tile_rows_minus1 + 1 ) ) to ( bottom_right_tile_index[ i ][ 0 ] / (num_tile_rows_minus1 + 1 ) ), inclusive.

-

–	NumTileColumnsInMCTS[ i ] is set equal to ( bottom_right_tile_index[ i ][ 0 ] % ( num_tile_rows_minus1 + 1 ) ) − ( top_left_tile_index[ i ][ 0 ] % ( num_tile_rows_minus1 + 1 ) ).

–	NumTileRowsInMCTS[ i ] is set equal to ( bottom_right_tile_index[ i ][ 0 ] / (num_tile_rows_minus1 + 1 ) ) − ( top_left_tile_index[ i ][ 0 ] / ( num_tile_rows_minus1 + 1 ) ).

–	Otherwise (each_tile_one_tile_set_flag is equal to 1), for each tile with TileId i, with i in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive, the following applies:

–	mctsLevelIdc[ i ] is set equal to max_mcts_level_idc.

–	mctsTierFlag[ i ] is set equal to max_mcts_tier_flag.

–	mctsWidthInSamplesY[ i ] is set equal to ColumnWidthInLumaSamples[ i % ( num_tile_rows_minus1 + 1 ) ].

–	mctsHeightInSamplesY[ i ] is set equal to RowHeightInLumaSamples[ i / ( num_tile_rows_minus1 + 1 ) ].

-

–	NumTileColumnsInMCTS[ i ] is set equal to 1.

–	NumTileRowsInMCTS[ i ] is set equal to 1.

–	The variables mctsSizeInSamplesY[ i ] and NumSliceSegmentsInMCTS[ i ] are derived as follows:

–	mctsSizeInSamplesY[ i ] is derived as follows:

mctsSizeInSamplesY[ i ] = mctsWidthInSamplesY[ i ] * mctsHeightInSamplesY[ i ]	(D-1)

–	NumSliceSegmentsInMCTS[ i ] is set equal to the number of slice segments in the i-th motion-constrained tile set.

–	The variables mctsMaxLumaPs[ i ], mctsMaxCPB[ i ] mctsMaxSliceSegments[ i ], mctsMaxTileRows[ i ], mctsMaxTileCols[ i ], mctsMaxBR[ i ] and mctsMinCr[ i ] are derived as follows. [Ed. The upper bounds variables should be derived from the table in Annex A]

–	mctsMaxLumaPs[ i ] is set equal to MaxLumaPs specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMaxCPB[ i ] is set equal to MaxCPB specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMaxSliceSegments[ i ] is set equal to MaxSliceSegmentsPerPicture specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMaxTileRows[ i ] is set equal to MaxTileRows specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMaxTileCols[ i ] is set equal to MaxTileCols specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMaxBR[ i ] is set equal to MaxBR specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].

–	mctsMinCr[ i ] is set equal to MinCr specified in Table A-2 for the level indicated by mctsLevelIdc[ i ] with the tier indicated by mctsTierFlag[ i ].--

[Ed. Which bits are counted? Do we count non-VCL bits? Intent is Yes.

Which layers does it include? – as indicated for scope of SEI.

If there are multiple tiles in a slice and some are in the MCTS and some are not.

Are we counting NAL units or bits inside of NAL units.

Agreed to count all bits of all NAL units that include the identified tiles.

Agreed: Require the NAL units of the tiles in the MCTS to not include any tiles that are not in the MCTS when this tier/level information is indicated.]

–

mctsLevelIdc[ i ] and mctsTierFlag[ i ] indicate the level and tier to which the i-th motion-constrained tile set conforms, as specified in Annex A with the following modifications and additions:

–	The variable PicSizeInSamplesY is replaced with the variable mctsSizeInSamplesY[ i ].

–	The value of ( num_tile_ columns _minus1 + 1 ) is replaced with the variable NumTileColumnsInMCTS[ i ].

–	The value of ( num_tile_rows_minus1 + 1 ) is replaced with the variable NumTileRowsInMCTS[ i ].

–	The number of slice segments for the i-th motion-constrained tile set is set to NumSliceSegmentsInMCTS[ i ].

–	The variable MaxTileCols is replaced with the variable mctsMaxTileCols [ i ].

–	The variable MaxTileRows is replaced with the variable mctsMaxTileRows [ i ].

–	The variable MaxSliceSegmentsPerPicture is replaced with the variable mctsMaxSliceSegments[ i ].

–	The variable MaxLumaPs is replaced with the variable mctsMaxLumaPs[ i ].

–	The variable MaxBR is replaced with the variable mctsMaxBR[ i ].

–	The variable MinCr is replaced with the variable mctsMinCr[ i ].

–	The nominal removal time of motion-constrained tile set from the CPB shall be the same as removal time of corresponding access unit.

–	The difference between consecutive output times of motion-constrained tile sets from the DPB shall be same as removal time of corresponding access unit.

–	The value of NumBytesInNalUnit is replaced with the sum of the size of the all NAL units belonging to the i-th motion-constrained tile set in bytes.

- Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).