| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0356 |
| --- | --- |

| Title: | PU level IntraBC on/off and signalling |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu | Tel: Email: |  | +1 408 526 1899 xiaozhong.xu@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek |  |  |  |

_____________________________

# Abstract

In this contribution, PU level intra block copy (IntraBC) on/off and signalling are tested. Results report average 0.0%, 0.5%, 0.7% B-D rate savings for AI, RA and LD configurations, compared with CU level IntraBC on/off (SCM1.0) in 1080p text and graphics RGB category, without encoding time increase. It is asserted that the current encoder implementation enables PU level IntraBC only on 8x4 and 4x8 blocks; some encoder improvements are desired.

# Introduction

In SCM1.0, the intra block copy (IntraBC) mode is operated at CU level, where all the PU(s) within a CU are coded with IntraBC mode when enabled. In this contribution, the IntraBC on/off is moved from CU to PU. Two signalling methods, i.e. using intra_bc_flag or ref_idx, are tested.

# Algorithm description

In current SCM1.0, IntraBC mode is switched on and off at CU level, as follows [1].

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| ...... |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { |  |
| ...... |  |
| } |  |

## intra_bc_flag approach

In this approach, the IntraBC related syntax is moved from CU to PU. The text changes are highlighted in yellow as follows.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ¦¦ intra_block_copy_enabled_flag) |  |
| pred_mode_flag | ae(v) |
| …… |  |

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| if(slice_type != I && intra_block_copy_enabled_flag) |  |
| intra_bc_flag | ae(v) |
| if( intra_bc_flag ) { |  |
| mvd_coding( 0, 0, 2 ) |  |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| ... |  |
| } |  |
| } |  |

## ref_idx approach

In this approach, the ref_idx of L0 is used to signal whether the current PU is IntraBC coded. The CU syntax change is the same as in section 2.1; the PU syntax re-uses that of HEVC.

# Results

In this section, two sets of simulations are performed.

In test A, the intra_bc_flag method is integrated on HM-14.0+RExt-7.0+SCM1.0 [2]. The simulations are performed under the updated CTC [3]. Test A1is shown in table 1, where the IntraBC mode is with intra DST (for 4x4) and intra deblocking; Test A2 is shown in table 2, where the IntraBC mode is with inter DCT (for 4x4) and inter deblocking.

In test B, the simulations for signaling methods of using either intra_bc_flag or ref_idx are performed. The software platform is SCCE1 Test 2.1 (JCTVC-R0190) [4]. The simulations are performed under the updated CTC [3]. All the results in this test are evaluated against HM-14.0+RExt-7.0+SCM1.0 [2]. Test B1 is shown in table 3, where the results of SCCE1 test 2.1 (using ref_idx to signal IntraBC) are shown; Test B2 is shown in table 4, where the intra_bc_flag is used instead; Test B3 is shown in table 5, where the encoder decision for SCCE1 test 2.1 is turned off (in this case, the all the PUs will choose the same mode, while each PU still has IntraBC signalling). Note that the runtime in both tests is not accurate.

Table 1: Results of test A1 for lossy configuration

Table 2: Results of test A2 for lossy configuration

Table 3: Results of test B1 for lossy configuration

Table 4: Results of test B2 for lossy configuration

Table 5: Results of test B3 for lossy configuration

# Conclusion

The PU level IntraBC on/off and signalling are tested. Results are reported in this contribution. It is recommended to further study PU level IntraBC, its interaction with other IntraBC coding tools and signalling in Core Experiment. .

# Reference

[1] D. Flynn, et al, “HEVC Range Extensions Draft 6”, JCTVC-P1005, Jan. 2014

[2] HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-14.0%2BRExt-7.0%2BSCM-1.0

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Apr. 2014

[4] X. Xu, S. Liu and S. Lei, “SCCE1 Test 2.1: IntraBC coded as Inter PU”, JCTVC-R0190, June 2014

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).