| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0357 |
| --- | --- |

| Title: | Updated text of the Chroma resampling filter hint SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Barroux, Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku, Kawasaki, 211-8588 Japan Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-754-2639 guillaume.b@jp.fujitsu.com kazui.kimihiko@jp.fujitsu.com +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | FUJITSU LABORATORIES LTD., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution proposes an update of the Chroma resampling filter hint SEI message text specification over the Valencia output document JCTVC-Q1008. The suggested changes notably take into account the input contribution JCTVC-R0217 from MovieLabs.

# Described changes

## Changes relative to version 2

The proposed changes can be found attached to this contribution under the name “JCTVC-R0357_proposed_text”.

This contribution proposes significant editorial improvement regarding the semantic description of the syntax elements and the description of the processes involved in the Chroma resampling filter hint SEI message.

Among the editorial improvements, several tables were added for better readability and the already existing tables were changed to reduce the amount of information they contained and improve their clarity.

The following variables were added to improve the overall clarity of the process: upSamplingFlag, verTapLength, verFilterCoeff, horTapLength and horFilterCoeff.

The following normative changes where done:

- minimum_degradation_flag was removed. It was agreed that the usefulness of the flag was too limited.

- The additional constraint stating that both hor_chroma_filter_idc and ver_chroma_filter_idc could not both be set to 0 was added. This notably prevents the encoder to send a Chroma resampling filter hint SEI message with no actual filter signalled in it.

- The scope of SEI message was changed from CVS to CLVS.

- In the definition of ver_filtering_field_processing_flag, the additional constraint that “when ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1, pic_struct shall be within the range of 9 to 12 inclusive” was added. It provides the assurance that the decoder will have the necessary pieces of information enabling it to interleave top fields and bottom fields as intended by the encoding side.

Some spelling mistakes, typos and formatting problems have also been corrected.

## Changes relative to version 3

In the semantic description of ver_filtering_field_processing_flag, the constraint over the values pic_struct can take when ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1 has been transferred onto the value ver_filtering_field_processing_flag may take.

Instead of having two specific constraints for ver_chroma_filter_idc and hor_chroma_filter_idc to prevent them from being both equal to 0, a general constraint was added outside of the two flag semantic description.

A presentation of the variables lenDv, lenUv, lenDh and lenUh was added as these variables were used without being presented before.

The way p0 and p1 arrays are introduced for the filtering processes descriptions have been changed.

Various smaller editorial improvements were made.

It was remarked by some editors that in table D-9 and D-10, the collumn related to the range of value of j could be removed as it can be easily guessed that the length of the filters defined in the most right collumn defines the range of j for each line. No action was taken because it was believed other editors may have different points of view on the matter.

# Changes regarding MovieLab’s request

While trying to improve the overall quality of the SEI message description, a normative change was made that altered the filtering process. The change made could be described as:

When ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1, the filtering process shall be performed on fields independently and using the set of filters defined for frame filtering.

The contribution JCTVC-R0217 from MovieLabs then requested for some changes to accommodate the Progressive segmented Field case and it was taken in account. To address the request, the following semantic change has been made for the ver_filtering_field_processing_flag:

When ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1, pic_struct shall be within the range of 9 to 12 inclusive. Additionally, the filtering process should be performed on a frame basis. The input of the filtering process should be frames resulting from the interleaving of fields in accordance with the information conveyed by the pic_struct values.

The authors would like to point out that regarding the filtering process to be conducted when ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1, the final version of the proposed text does not differ from the original version. However, as described previously, the additional constraint that “when ver_filtering_field_processing_flag is equal to 0 and field_seq_flag is equal to 1, pic_struct shall be within the range of 9 to 12 inclusive” was added.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).