| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0360v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0200v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Miscellaneous cleanups re editors' notes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes the following changes, for addressing some of the editors' notes in the latest MV-HEVC/SHVC draft specifications, and some other issues discussed during the meeting:

- Change the definition of "picture unit" as follows, and remove the empty subclause F.7.4.2.4.6 that is a placeholder for adding a specification of order of NAL units and association to picture unit:

picture unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain allthe VCL NAL units of a coded picture and their associated non-VCL NAL units.

It is claimed to be clear because the existence of the following definitions in version 1 text:

associated non-VCL NAL unit: A non-VCL NAL unit (when present) for a VCL NAL unit where the VCL NAL unit is the associated VCL NAL unit of the non-VCL NAL unit.

associated VCL NAL unit: The preceding VCL NAL unit in decoding order for a non-VCL NAL unit with nal_unit_type equal to EOS_NUT, EOB_NUT, FD_NUT, or SUFFIX_SEI_NUT, or in the ranges of RSV_NVCL45..RSV_NVCL47 or UNSPEC56..UNSPEC63; or otherwise the next VCL NAL unit in decoding order.

- Add the following constraint to subclause 7.4.2.4.4:

A VCL NAL unit with a particular value of nuh_layer_id shall precede another VCL NAL unit that is within the same access unit and has a greater value of nuh_layer_id.

This item should be included into the version 1 defect report.

It was noted that in subclause F.7.4.2.4.1, the following restriction exists (however, still the above change should be included into subclause 7.4.2.4.4):

A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede, in decoding order, all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

- Add the following note to the end of subclause C.3.4:

NOTE – Decoders can start to store decoded parts of the current picture into the DPB before the decoding of the current picture is completed.

- Make the following changes to the semantics of the progressive refinement segment start SEI message, such that the picture for which the condition is true is not included in refinementPicSet:

The progressive refinement segment start SEI message specifies the beginning of a set of consecutive coded pictures in the current layer in decoding order that consists of the current picture and a sequence of one or more subsequent pictures in the current layer that refine the quality of the current picture, rather than a representation of a continually moving scene.

Let picA be the current picture. The tagged set of consecutive coded pictures refinementPicSet in the current layer starts from the next picture in the current layer after the current picture in decoding order and continues, in decoding order, until one of the following conditions is true: [Ed. (MH): It is suggested to change the phrasing to indicate that the picture for which the condition is true is not included in refinementPicSet.]

–	A new CVS begins. The picture in the current layer and in the access unit that starts the new CVS is not included in refinementPicSet.

–	The bitstream ends.

–	pic_order_cnt_delta is greater than 0 and the PicOrderCntVal of the next slice, which belongs to the picture picB in the current layer, to be decoded, i.e. PicOrderCnt( picB ), is greater than PicOrderCnt( picA ) plus pic_order_cnt_delta, where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB. The picture picB is not included in refinementPicSet.

–	A progressive refinement segment end SEI message that has the same progressive_refinement_id as the one in this SEI message and also applies to the current layer is decoded. The picture in the current layer and in the access unit containing the progressive refinement segment end SEI message is not included in refinementPicSet.

The decoding order of pictures within refinementPicSet should be the same as their output order.

- Make the following changes to the semantics of nuh_layer_id in subclause F.7.4.2.2, as the sentence is not present in HEVC version 1 spec and addition of it seems introducing some backward compatibility issue:

When nal_unit_type is equal to EOB_NUT, the value of nuh_layer_id shall be equal to 0. Decoders shall ignore (i.e. remove from the bitstream and discard) all NAL units with nal_unit type equal to EOB_NUT and nuh_layer_id greater than 0.

- Make the following changes (this item was agreed by the BoG on editors' notes when the BoG met):

To the semantics of sps_rep_format_idx:

sps_rep_format_idx specifies the index, into the list of rep_format( ) syntax structures in the VPS, of the rep_format( ) syntax structure that applies to the layers that refer to this SPS. When not present, the value of sps_rep_format_idx is inferred to be equal to 0. The value of sps_rep_format_idx shall be in the range of 0 to vps_num_rep_formats_minus1, inclusive. [Ed. (GT): Inferences to 0 seems not to be necessary. We might consider to infer it to vps_rep_format_idx[ LayerIdxInVps[ layerIdCurr ] ], when not present. ]

To the semantics of sps_scaling_list_ref_layer_id:

It is a requirement of bitstream conformance that, when an SPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and sps_infer_scaling_list_flag in the SPS is equal to 1, the layer with nuh_layer_id equal to sps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

To the semantics of pps_scaling_list_ref_layer_id:

It is a requirement of bitstream conformance that, when a PPS with nuh_layer_id equal to nuhLayerIdA is active for a layer with nuh_layer_id equal to nuhLayerIdB and pps_infer_scaling_list_flag in the PPS is equal to 1, the layer with nuh_layer_id equal to pps_scaling_list_ref_layer_id shall be a direct or indirect reference layer of the layer with nuh_layer_id equal to nuhLayerIdB.

- Make the following changes in subclause F.8.1:

If the access unit has at least one picture with nuh_layer_id greater than 0, the TemporalId and PicOrderCntVal of the decoded picture with nuh_layer_id equal to 0 are set equal to the TemporalId and PicOrderCntVal, respectively, of any picture with nuh_layer_id greater than 0 in the access unit.

Otherwise, the decoded picture with nuh_layer_id equal to 0 is discarded and the sub-DPB for the layer with nuh_layer_id equal to 0 is set to be empty. [Ed. (YK): Make sure that there is a restriction, indirect or direct, that requires the TemporalId to be equal to 0 when BlIrapPicFlag is equal to 1.]

NOTE 1 – The constraint on the value of TemporalId being required to be equal to 0 applies to pictures with nuh_layer_id equal to 0 when vps_base_layer_internal_flag is equal to 0.

# Patent rights declaration(s)

The authors of this contribution are not aware of current or pending patent rights regarding the specific modifications proposed in this contribution. However, Qualcomm Incorporated may have current or pending patent rights relating to other related aspects of the technology described in this contribution and, conditioned on reciprocity, are each prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).