| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R1101 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Vector entropy coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Chao Pang Xiaozhong Xu | Email: | cpang@qti.qualcomm.com xiaozhong.xu@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the Core Experiment 1 on vector entropy coding.

# Introduction

This document defines the Core Experiment 1 (CE1) on vector entropy coding to be performed for the upcoming 19th JCT-VC meeting.

The CE is studying two aspects: block vector difference (BVD) entropy coding for intra block copy (IBC) mode, and motion vector difference (MVD) entropy coding for Inter mode.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| K. Miyazawa A. Minezawa | Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp Minezawa.Akira@ds.MitsubishiElectric.co.jp | Mitsubishi | X | X |
| X. Xu P. Lai S. Liu S.-T. Hsiang T.-D. Chuang Y.-W. Huang | xiaozhong.xu@mediatek.com polin.lai@mediatek.com Shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com | MediaTek | X | X |
| G. Laroche T. Poirier C. Gisquet P. Onno | guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr | Canon (France) | X | X |
| C. Rosewarne M. Maeda | chris.rosewarne@cisra.canon.com.au maeda.mitsuru@canon.co.jp | Canon (Australia) | X | X |
| K. Rapaka C. Pang J. Sole M. Karczewicz | krapaka@qti.qualcomm.com cpang@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com | Qualcomm | X | X |
| C.-L. Lin C.-C. Lin | Chunlung@itri.org.tw JackLin@itri.org.tw | ITRI |  | X |
| W.-H. Peng C.-C. Chen | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw | NCTU |  | X |

# Tools to be tested

## BVD entropy coding for IBC mode

### Test 1.1: BVD coding for IBC (JCTVC-R0181, Qualcomm)

In this test, first a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than N. If BVD is greater than N, the remaining absolute level is coded using bypass Exp-Golomb codes. One flag is coded to indicate sign of the BVD. Additionally, other Exp-Golomb codes would be tested.

### Test 1.2: On block vector coding for IBC (JCTVC-R0123, Canon)

In this test the block vector (BV) coding is modified to use coeff_abs_level_remaining syntax (without the gr1 flag), with Rice parameter adaptation applied separately for the horizontal and vertical components of the BV and updated per PU.

### Test 1.3: Adaptive switching between differential and direct coding for intra block copy vectors (JCTVC-R0074, Mitsubishi)

In this test, one flag is added to specify whether the BV is predicted or not. If not predicted, BV is coded without prediction. The flag is signalled only when the BV component is non-zero. An additional modification of the coding process of sign flags of vectors to reduce redundancy is also tested.

### Test 1.4: BVD coding for IBC using a universal entropy coding scheme (JCTVC-R0133, MediaTek)

In this test, the absolute value of each BVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

## MVD entropy coding for Inter mode

### Test 2.1: MVD coding for Inter (JCTVC-R0181, Qualcomm)

In this test, first a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than N. If MVD is greater than N, the remaining absolute level is coded using bypass Exp-Golomb codes. One flag is coded to indicate sign of the MVD. Additionally, other Exp-Golomb codes would be tested

### Test 2.2: MVD coding using a universal entropy coding scheme (JCTVC-R0134, MediaTek)

In this test, the absolute value of each MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

## MVD and BVD entropy coding for Inter and IBC modes

### Test 3.1: BVD coding for IBC and MVD coding for Inter (JCTVC-R0181, Qualcomm)

In this test, combinations of Test 1.1 and Test 2.1 applied to BVD and MVD are tested.

### Test 3.2: MVD and BVD entropy coding for Inter and IBC modes (JCTVC-R0088, Canon)

In this test, a flag at slice level is used to determine the residual coding method for both Inter and IBC modes. The current SCM2.0 BVD and MVD coding should be considered with this method or with one of the proposed BVD methods in section 3.1.

### Test 3.3: Unified BVD & MVD coding using a universal entropy coding scheme (JCTVC-R0134, MediaTek)

In this test, the absolute value of each BVD and MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

## Combination tests

### Test 4.1: Combination of Test 1.1 + Test 1.3 (Mitsubishi)

In this test, the combination of Test 1.1 and Test 1.3 is investigated.

### Test 4.2: Combination of Test 1.2 + Test 1.3 (Mitsubishi)

In this test, the combination of Test 1.2 and Test 1.3 is investigated.

### Test 4.3: Combination of Test 1.3 + Test 1.4 (Mitsubishi)

In this test, the combination of Test 1.3 and Test 1.4 is investigated.

# Cross-checks

|  | Proponent | Cross-checker |
| --- | --- | --- |
| Test 1.1 | Qualcomm | MediaTek |
| Test 1.2 | Canon (Australia) | Qualcomm |
| Test 1.3 | Mitsubishi | ITRI |
| Test 1.4 | MediaTek | Mitsubishi |
| Test 2.1 | Qualcomm | MediaTek |
| Test 2.2 | MediaTek | Canon (Australia) |
| Test 3.1 | Qualcomm | Mitsubishi |
| Test 3.2 | Canon (France) | Qualcomm |
| Test 3.3 | MediaTek | Canon (France) |
| Test 4.1 | Mitsubishi | MediaTek |
| Test 4.2 | Mitsubishi | NCTU |
| Test 4.3 | Mitsubishi | Qualcomm |

# Test conditions

All the tests will be performed on top of SCM-2.0. The common conditions for screen content coding tests from JCTVC-R1015 shall be used for these experiments.

# Timeline

T1: Release of SCM-2.0: 01 August, 2014

T2: Release of individual test (test 1.x, test 2.x, and test 3.x) software and crosscheck begins, T2 = T1 + 3 weeks: 22 August, 2014

T3: Release of combined test (test 4.x) software and crosscheck begins, T3 = T2 + 3 weeks: 12 September, 2014

T4: Document upload deadline, 07 October, 2014

# References

- JCTVC-R1015, Common conditions for screen content coding tests [H. Yu, R. Cohen, K. Rapaka, J. Xu]