| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R1104 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Intra Line Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Xiaozhong Xu Li Zhang Tao Lin | Email: | cheerchen.cs98g@g2.nctu.edu.tw xiaozhong.xu@mediatek.com lizhang@qti.qualcomm.com lintao@tongji.edu.cn |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document provides the description of the HEVC Screen Content Coding Core Experiment 4 on intra line copy.

# Introduction

This document defines the HEVC Screen Content Coding Core Experiment 4 (CE4) on intra line copy (formerly known as line-based intra copy [1]) to be performed for the upcoming 19th JCT-VC meeting. Two related documents proposed in the 18th JCT-VC meeting were decided to be further studied in this CE. The techniques to be evaluated in this CE include:

- search range of intra line copy (JCTVC-R0132) [2];

- intra line copy with constrained BVs (JCTVC-R0132) [2];

- self-matching intra line copy (JCTVC-R0281) [3].

The first one tests various settings to the search range of intra line copy to investigate the benefit in comparison of the restricted and full-frame intra block copy. The second test imposes an additional constraint, which requires that all the reference samples used for intra line copy should be limited to an area inside the valid search range. The third test is a padding scheme for padding the samples of reference lines overlapped with the current CU, so the usage of closer lines from the current CU is allowed.

This CE addresses the aforementioned topics and evaluates the coding gain, complexity in terms of encoding/decoding time, memory access bandwidth, and CABAC throughput of the above techniques.

# List of Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| Wen-Hsiao Peng Chun-Chi Chen | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw | NCTU/ITRI | X | X |
| Tao Lin | lintao@tongji.edu.cn | Tongji Univ. | X |  |
| Xiaozhong Xu Shan Liu | xiaozhong.xu@mediatek.com shan.liu@mediatek.com | MediaTek |  |  |
| Joel Sole Krishna Rapaka Li Zhang | joels@qti.qualcomm.com krapaka@qti.qualcomm.com lizhang@qti.qualcomm.com | Qualcomm |  | X |
| Chun-Lung Lin Ching-Chieh Lin | chunlung@itri.org.tw JackLin@itri.org.tw, JackLin.itri@gmail.com | ITRI |  |  |
| Zhan Ma Wei Wang Meng Xu Fanyi Duanmu Haoping Yu | Zhan.Ma@huawei.com, zhan.ma@outlook.com Wei.Wang.sc@huawei.com, rick.wang.101@gmail.com m.xu@huawei.com fanyi.duanmu@huawei.com haoping.yu@huawei.com | Huawei |  | X |
| Ankur Saxena Madhukar Budagavi | a.saxena1@samsung.com, saxena.ankur0@gmail.com m.budagavi@samsung.com | Samsung |  |  |
| (“P”: Proponent, “C”: Cross-checker) |  |  |  |  |

# Description of Tools to be Tested

## Test A: Search Range of Intra Line Copy

- Proponent: NCTU/ITRI (JCTVC-R0132)

- Cross-checker: Huawei

JCTVC-R0132 proposed a technique of intra line copy which performs intra copying for row-wise or column-wise lines instead of blocks. It firstly divides the current PU equally into 1-pixel-wide lines and then operates intra copying line by line to fetch reference pixels from part of the reconstructed area inside the current frame. Each line has its own BV shared across its three colour components. Pixels within the current CU cannot be used as reference. Full-frame and 4-CTU search areas for intra line copy will be tested respectively to investigate the benefit in comparison to restricted and full-frame intra block copy.

## Test B: Intra Line Copy with Constrained BVs

- Proponent: NCTU/ITRI (JCTVC-R0132)

- Cross-checker: Qualcomm

This test imposes an additional constraint on the intra line copy, which requires that all the reference lines belonging to the current PU should be bounded to a few limited regions inside the reconstructed area of the current frame (Figure 1).

- . All the reference lines of a PU are restricted to some bounded area (i.e. the red box) within the available search area.

## Test C: Self-matching Intra Line Copy

- Proponent: Tongji Univ. (JCTVC-R0281)

- Cross-checker: NCTU/ITRI

In contrast to the intra line copy proposed in JCTVC-R0131, this test additionally relaxes the constraint that the reference lines overlapped with the current CU cannot be used as reference, and thus the usage of closer lines from the current CU is allowed (Figure 2). When overlap occurs, the overlapped part associated with the reference line can be formed by referring to the predictive signals of the current CU.

- . The reference lines can be formed by using both in-CU and off-CU pixels as reference.

# List of Testers and Cross-checkers

| Test | Method | Tester | Cross-checker |
| --- | --- | --- | --- |
| A | Search range of intra line copy | NCTU/ITRI | Huawei |
| B | Intra line copy with constrained BVs | NCTU/ITRI | Qualcomm |
| C | Self-matching intra line copy | Tongji Univ. | NCTU/ITRI |

# Test Conditions

The SCM-2.0 software will be used for all the to-be-tested techniques. Two test conditions formed based on JCTVC-R1015 [4] will be used to evaluate the coding results for both the lossy and lossless operating points.

- Test condition 1: Full-frame search range for intra block copy;

- Test condition 2: Restricted search range (1 current CTU + 3*64-wide region to the left) for intra block copy. (Note that techniques to be tested under this test condition should follow the same restriction on the search range.)

# Evaluation of CE Results

Results of this CE will be evaluated on the basis of BD-rate, encoding and decoding time, average- and worst-case memory access bandwidth, and CABAC throughput.

## Complexity

To measure software runtime, the SCM software with the provided configurations and the proposals implemented on it shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

## Memory Bandwidth

The average-case and the worst-case per pixel memory access bandwidth with respect to various memory access patterns (i.e. 4x2 for the average case and 4x1, 8x1, 4x2, 8x2, and 4x4 for the worst case) must be reported for each test in this CE. A software patch built based on the memory bandwidth tool of SCCE1 (JCTVC-R0031) will be provided to the CE participants in two weeks after the release of the SCM-2.0 software to facilitate the computation of the average per-pixel memory access bandwidth.

## CABAC Throughput

The number of context-coded bins per pixel in the worst case scenario must be reported for each test in the CE. An excel sheet will be provided in two weeks after the release of the SCM-2.0 software to facilitate the measurement of the CABAC throughput.

# Timeline

- T1 (August 1, 2014): Release of SCM-2.0 software.

- T2 (T1 + 1 week = August 8, 2014): Finalization and release of CE description.

- T3 (T1 + 2 weeks = August 15, 2014): Release of software patch for SCM-2.0 to measure memory access bandwidth of intra line copy.

- T4 (T1 + 3 weeks = August 22, 2014): Release of CE software to the participants and start of cross-checking.

- T5 (T4 + 3 weeks = September 12, 2014): Release of CE document, text, and simulation/cross-checking results.

- T6 (October 7, 2014): Submission deadline for the 19th JCT-VC meeting.

# References

- C.-C. Chen, X. Xu, L. Zhang, “HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1122, Valencia, Spain, March 2014.

- C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Juo, “Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0132, June 2014.

- L. Zhao, X. Chen, T. Lin, “Non-SCCE2: An improvement on Intra Line Copy of SCCE2,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0281, June 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1015, June 2014.