| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R1106 |
| --- | --- |

| Title: | Description of Core Experiment 6 (CE6): Improvements of palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang (Primary) Patrice Onno Robert Cohen Vadim Seregin Xiaoyu Xiu Zhan Ma | yuwen.huang@mediatek.com patrice.onno@crf.canon.fr cohen@merl.com vseregin@qti.qualcomm.com xiaoyu.xiu@interdigital.com Zhan.Ma@huawei.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 6 (CE6) on improvements of palette mode.

# Introduction

In the 18th JCT-VC meeting in Sapporo, a simplified combined palette mode in JCTVC-R0348 [1] was first adopted into SCM-2.0 software and draft text, and then the CU-level escape colour flag in JCTVC-R0066 [2] with a simplification in JCTVC-R0051 [3] was adopted. The goal of this Core Experiment (CE) is to further investigate improvements of the existing palette mode in SCM-2.0.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Broadcom | Minhua Zhou | minhua@broadcom.com |
| Canon | Chris Rosewarne Christophe Gisquet Guillaume Laroche Patrice Onno | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Fujitsu | Jianqing Zhu Jiangli Ye Kimihiko Kazui | zhujianqing@cn.fujitsu.com threerabbit@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Huawei | Haoping Yu Meng Xu Wei (Rick) Wang Zhan Ma | haoping.yu@huawei.com m.xu@huawei.com rick.wang.101@gmail.com Wei.Wang.sc@huawei.com Zhan.Ma@huawei.com Zhan.ma@outlook.com |
| Intel | Wenhao Zhang Yi-Jen Chiu | wenhao.zhang@intel.com yi-jen.chiu@intel.com |
| InterDigital | Jiun-Yu Kao Xiaoyu Xiu Yan Ye Yuwen He | jiun-yu.kao@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com yuwen.he@interdigital.com |
| ITRI International | Ching-Chieh Lin Chun-Lung Lin Yao-Jen Chang | JackLin@itri.org.tw JackLin.itri@gmail.com Chunlung@itri.org.tw britpablo@itri.org.tw |
| KDDI | Kei Kawamura | ki-kawamura@kddi.com |
| LG Electronics | Sehoon Yea Sunmi Yoo | sehoon.yea@lge.com sunmi.yoo@lge.com |
| MediaTek | Jing Ye Jungsun Kim PoLin Lai Shan Liu Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yi-Wen Chen Yu-Chen Sun Yu-Wen Huang | jing.ye@mediatek.com jungsun.kim@mediatek.com polin.lai@mediatek.com shan.liu@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yiwen.chen@mediatek.com yc.sun@mediatek.com yuwen.huang@mediatek.com |
| Mitsubishi Electric | Akira Minezawa Kazuyuki Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| MERL | Robert Cohen Xingyu Zhang | cohen@merl.com xzhang@merl.com |
| Microsoft | Jizheng Xu Xun Guo | jzxu@microsoft.com xunguo@microsoft.com |
| NCTU | Chun-Chi Chen Tsui-Shan Chang Wen-Hsiao Peng | cheerchen.cs98g@g2.nctu.edu.tw abaibai1122.cs01g@nctu.edu.tw wpeng@cs.nctu.edu.tw |
| Nokia | Jani Lainema Döne Bugdayci | jani.lainema@nokia.com ext-done.bugdayci@nokia.com |
| Qualcomm | Feng Zou Joel Sole Krishna Rapaka Marta Karczewicz Rajan Joshi Vadim Seregin Wei Pu | fzou@qti.qualcomm.com joels@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com rajanj@qti.qualcomm.com vseregin@qti.qualcomm.com wpu@qti.qualcomm.com |
| Samsung | Ankur Saxena Madhukar Budagavi | a.saxena1@samsung.com saxena.ankur0@gmail.com m.budagavi@samsung.com |
| Sharp | Jane Zhao Kiran Misra Seung-Hwan Kim | jzhao@sharplabs.com misrak@sharplabs.com kimse@sharplabs.com |
| Sony | Jun Xu | Jun.Xu@am.sony.com |
| University of Hanover | Thorsten Laude | laude@tnt.uni-hannover.de |

# Software

All tests are to be built on top of SCM-2.0.

# Test condition

All to-be-tested methods shall be tested under SCM-2.0 common test condition [4], which enables full frame intra block copy (FF-IBC) as anchor.

# To-be-tested methods

## Category A – Colour index run coding

### Test A.1 – Restricted run coding

- Proponent: Nokia, JCTVC-R0045

- Crosschecker: Qualcomm

- Description:

This test limits the amount of palette_run syntax element decoded for the index mode of the current palette implementation. When the colour index is larger than a threshold, the palette_run is not signalled. The issue of the parsing dependency noted for this contribution will be investigated.

### Test A.2 – Run coding for two-colour palette

- Proponent: ITRI, JCTVC-R0076

- Crosschecker: Sharp

- Description:

This test evaluates modifications in the two-colour case of palette mode. The first modification skips coding colour index in “copy left run” mode. Encoder and other related modifications in the two-colour case of palette mode will be also evaluated.

### Test A.3 – Restricted run coding

- Proponent: Canon, JCTVC-R0085

- Crosschecker: Qualcomm

- Description:

This test limits the amount of palette_run syntax element decoded for the index mode of the current palette implementation. When the colour index is larger than a threshold, the palette_run is not signalled. The issue of the parsing dependency noted for this contribution will be investigated.

### Test A.4 – Binarization and context modeling for run coding

- Proponent: MediaTek, JCTVC-R0136

- Crosschecker: Qualcomm

- Description:

In this test, the syntax element, palette_run, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The context selection is conditioned on the palette_type_flag, bin index, and palette index. Both decoded palette index and reconstructed palette index values will be tested for context selection. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy coded in the bypass mode.

### Test A.5 – Contexts for run coding

- Proponent: Qualcomm, JCTVC-R0174 part 1

- Crosschecker: Canon

- Description:

In this test, use of additional contexts depending on the index for coding index runs is tested. Additional contexts for greater than 0, greater than 1 and greater than 2 flags will be tested. The context is dependent on the index value (decoded) or the coded index (parsed). Additionally choice of context based on index value and the palette size will be tested.

### Test A.6 – Contexts for run coding

- Proponent: Qualcomm, JCTVC-R0174 part 2

- Crosschecker: Canon

- Description:

In this test, exponential Golomb and Golomb Rice binarizations and their truncated versions for the palette run values will be tested. Combination of tests A.5 and A.6 will also be tested.

## Category B – Colour index and escape colour coding

### Test B.1 – Context coded CU-level escape colour flag

- Proponent: Qualcomm, JCTVC-R0066

- Crosschecker: InterDigital

- Description:

In this test, use of regular CABAC bin to code the CU level escape flag will be tested. Context determination methods dependent on palette size and block size will be tested.

### Test B.2 – Encoder modification related to CU-based escape colour flag

- Proponent: ITRI, JCTVC-R0075

- Crosschecker: InterDigital

- Description:

If the number of the pixels quantized to a major colour is smaller than or equal to a pre-assigned threshold, the pixels are changed to escape colour pixels. Then the corresponding major colour is removed. An adaptive threshold method will also be tested.

### Test B.3 – Colour index coding with contextualization

- Proponent: University of Hanover, JCTVC-R0113

- Crosschecker: MediaTek

- Description:

In this test, a modified colour index signalling is evaluated. In particular, the most significant bin of the palette_index syntax element is coded with a context instead of bypass.

### Test B.4 – Binarization and context modeling for index coding

- Proponent: MediaTek, JCTVC-R0135

- Crosschecker: Fujitsu

- Description:

In this test, the syntax element, palette_index, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the truncated binary code and the resulting bin string is entropy coded in the bypass mode.

### Test B.5 – Escape colour prediction

- Proponent: InterDigital, JCTVC-R0170

- Crosschecker: ITRI

- Description:

In this test, an escape colour prediction method will be tested. A flag is signalled to indicate if all colour components of the current escape colour pixel are the same as that of the previous escape colour pixel. If not, the current escape colour value is predicted from an entry in the palette table. The index value of the table entry used for prediction and the prediction difference are coded.

## Category C – Additional colour index representation modes

### Test C.1 – Transition copy mode

- Proponent: Canon, JCTVC-R0084

- Crosschecker: MediaTek

- Description:

In this test, an index prediction method based on transitions will be evaluated. The following index value for the last occurrence of a given index value is stored and used as the inferred index value in the new prediction mode.

### Test C.2 – Transition copy mode

- Proponent: MediaTek, JCTVC-R0120

- Crosschecker: Canon

- Description:

In this test, a new index coding mode, transition copy (TC) run mode will be tested. In the TC run mode, the coded adjacent colour index patterns are recorded in the TC table. The TC table can be inherited across CUs. The recoded colour indices are used as the colour index predictors in the TC run mode.

### Test C.3 – Copy-from-previous-row mode

- Proponent: Qualcomm, JCTVC-R0202

- Crosschecker: MediaTek

- Description:

In this test, a mode of “copy from previous rows” for the palette coding will be tested. This method enables copying pixels from previously coded rows beyond the row above in the current CU. This is defined as an additional palette mode besides the existing “copy from left”, “copy above” and “escape” modes. The reference row index is coded in the bitstream to indicate which row is copied from. Restrictions on the number of rows available for ‘copy from previous rows” will be tested.

## Category D – Palette table coding

### Test D.1 – Run-length coding for reuse flags

- Proponent: Qualcomm, JCTVC-R0228

- Crosschecker: MediaTek

- Description:

In this test, a method that uses run-length coding to signal the binary vector predictor for palette will be tested, where the run value, indicating the number of zero elements between the ones, is signalled as described in JCTVC-R0228. Additionally, sending the number of non-zero elements explicitly in the beginning will be tested. Run values are coded using Exponential Golomb coded or Golomb Rice codes or their truncated versions. Additionally, increased palette predictor size will be tested with the run-length coding.

# Timeline

T1: Release of SCM-2.0

T2: Finalization of CE description, T2 = T1 + 1 week

T3: Release of to-be-tested software packages and start crosschecking, T3 = T1 + 3 weeks

T4: Release of text specifications, T4 = T1 + 5 weeks

T5: Release of all simulation results and finish crosschecking, T5 = T1 + 6 weeks

T6: Document upload deadline, Tuesday, October 7, 2014

The 19th JCT-VC meeting will be held in Strasbourg during October 17-24, 2014.

# References

- P. Onno, X. Xiu, Y.-W. Huang, R. Joshi, “Suggested combined software and text for run-based palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0348, 18th Meeting: Sapporo, JP, June-July 2014.

- W. Pu, R. Joshi, M. Karczewicz, J. Sole, V. Seregin, “SCCE3: Test B.13 – Flag to indicate escape and index coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0066, 18th Meeting: Sapporo, JP, June-July 2014.

- Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung, “Non-SCCE3: Adaptive escape scheme for palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R0051, 18th Meeting: Sapporo, JP, June-July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-R1015, 18th Meeting: Sapporo, JP, June-July 2014.