| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R1109_r2 |
| --- | --- |

| Title: | Description of Core Experiment 9 (CE9): IBF/CCP interdependency |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Robert Cohen Shan Liu Jizheng Xu Li Zhang | cohen@merl.com shan.liu@mediatek.com jzxu@microsoft.com lizhang@qti.qualcomm.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 9 (CE9) on intra boundary filtering (IBF) and cross-component prediction (CCP) interdependency.

# Introduction

At the 18th JCT-VC meeting, it was shown in JCTVC-R0114 and JCTVC-R0314 that improvements in coding performance for screen content coding may be obtained by disabling intra boundary filtering for the first (G/Y) component, or by enabling intra boundary filtering for all three components. It was also shown in JCTVC-R0219 that improvements in coding performance for screen content coding may be obtained by compensating for interdependencies between intra boundary filtering (IBF) and cross-component prediction (CCP), in that CCP is using a boundary-filtered first component to predict the non-filtered remaining components.

The purpose of this core experiment is to evaluate the proposed modifications to IBF and CCP in the context of screen content coding, both individually and in combination.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Canon (Australia) | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |
| ITRI | Chun-Lung Lin Ching-Chieh Lin | Chunlung@itri.org.tw JackLin@itri.org.tw, JackLin.itri@gmail.com |
| MediaTek | Shan Liu Xianguo Zhang Kai Zhang Yu-Wen Huang Tzu-Der Chuang | shan.liu@mediatek.com xianguo.zhang@mediatek.com Kai.zhang@mediatek.com yuwen.huang@mediatek.com peter.chuang@mediatek.com |
| MERL | Robert Cohen Xingyu Zhang | cohen@merl.com eexyzhang@ust.hk |
| Microsoft | Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com |
| Mitsubishi Electric | Akira Minezawa Kazuyuki Miyazawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp |
| Qualcomm | Li Zhang Jianle Chen Marta Karczewicz | lizhang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |

# Software

The proposed modifications will be incorporated on top of SCM-2.0.

# Test conditions

The common conditions for screen content coding tests from JCTVC-R1015 shall be used for these experiments.

# Tests

## Individual tool tests

### Test A.1 – Optionally disabling the usage of the intra boundary filters

It is proposed to conditionally disable the boundary filters for Intra horizontal, vertical, and DC modes with flag controls in SPS and slice header.

- Proposal: JCTVC-R0114 [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

- Cross-check(s): Participants from MERL

### Test A.2 – Optionally enabling the usage of the intra boundary filters on all components

It is proposed to enable the intra boundary filter for the second and third components in the 444 chroma sampling format. An enable flag is added at the SPS or PPS level.

- Proposal: JCTVC-R0314 [B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen (MERL)]

- Cross-check(s): Participants from MediaTek

### Test A.3 – Modifying cross-component prediction to compensate for intra boundary filtering

It is proposed to modify the CCP process by adding an offset block to the reconstructed luma (or first component) residual block. This offset block is the difference between the luma prediction block after intra boundary filtering is applied and the same block before intra boundary filtering is applied.

- Proposal: JCTVC-R0219 [X. Zhang, R. Cohen (MERL)]

- Cross-check(s): Participants from Qualcomm

## Combination and additional tests (TBD)

### Test B.1 – Combination of tests A.1 and A.3

IBF can be optionally disabled. When it is enabled, CCP is modified to compensate for IBF

- Proposals: JCTVC-R0114 and JCTVC-R0219

- Cross-check(s): Participants from Microsoft

# Timeline

T1: Release of SCM-2.0: 01 August, 2014

T2: Release of to-be-tested software: 22 August, 2014 (T2 = T1 + 3 weeks)

T3: Release of all simulation results including crosscheck: 12 September, 2014 (T3 = T2 + 3 weeks)

T5: Document upload deadline: 07 October, 2014

Note: The 19th JCT-VC meeting will be in Strasbourg, France, Friday 17 October – Friday 24 October, 2014.

# Cited documents

JCTVC-R0114	Optionally disabling the usage of the intra boundary filters for SCC [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

JCTVC-R0219	Improvement of cross-component prediction [X. Zhang, R. Cohen (MERL)]

JCTVC-R0314	On intra chroma boundary filtering [B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen (MERL)]

JCTVC-R1015	Common conditions for screen content coding tests [H. Yu, R. Cohen, K. Rapaka, J. Xu]