| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0004 |
| --- | --- |

| Title: | HEVC conformance test development (AHG4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Jill Boyce Kimihiko Kazui Adarsh K. Ramasubramonian Yan Ye | Tel: Email: | teruhikos@jp.sony.com jill@vidyo.com kazui.kimihiko@jp.fujitsu.com aramasub@qti.qualcomm.com Yan.Ye@InterDigital.com |
| Source: | AHG on HEVC conformance development |  |  |

_____________________________

# Introduction

The mandates of this AHG are:

- Study the requirements of HEVC conformance testing to ensure interoperability.

- Discuss the work plan needed to develop HEVC v.1, RExt and SHVC conformance testing.

- Study potential testing methodology to fulfil the requirements of HEVC conformance testing.

- Establish and coordinate bitstream exchange activities for HEVC.

- Identify needs for HEVC conformance bitstreams with particular characteristics.

- Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set.

# Activities

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG4] in the subject line for easy grouping.

## ftp site

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

## Bitstream generation guideline

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

## HEVC v.1 conformance

After the Sapporo meeting, the following problems were reported. The revised bitstreams were uploaded at the following site, separating the bitstreams under ballot.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

### general_profile_idc

In the following bitstreams, general_profile_idc has reserved value 0 (7.4.4):

AMP_A_Samsung_6

AMP_B_Samsung_6

AMVP_C_Samsung_6

STRUCT_A_Samsung_6

STRUCT_B_Samsung_6

Those bitstreams were revised after Sapporo meeting to fix the value of general_profile_compatibility_flag. (See JCTVC-R0004) However, the value of the general_profile_idc was set to 0 in the revised bitstreams. The value of this idc must be 1.

### End of slice segment

The spec requires that once decoding end_of_slice_segment_flag with value 1, the remaining bytes in the slice segment data must not be different from zero. (#1283)

The following bitstream violate this constraint.

DBLK_A_MAIN10_VIXS_3

### Extension flag

It was reported that one or more extension flags set in parameter set and/or slice headers (this makes streams non conforming to v1 but may be intentional to test decoders).

EXT_A_ericsson_4

PS_B_VIDYO_3

WPP_A_ericsson_MAIN10_2

WPP_A_ericsson_MAIN_2

WPP_B_ericsson_MAIN10_2

WPP_B_ericsson_MAIN_2

WPP_C_ericsson_MAIN10_2

WPP_C_ericsson_MAIN_2

WPP_D_ericsson_MAIN10_2

WPP_D_ericsson_MAIN_2

WPP_E_ericsson_MAIN10_2

WPP_E_ericsson_MAIN_2

WPP_F_ericsson_MAIN10_2

WPP_F_ericsson_MAIN_2

This issue was discussed during Sapporo meeting. The decision was to keep those in the conformance test suite, but to add the clarification. This issue should be confirmed again during the meeting.

In the conformance spec, the following clarification was added.

"

This bitstream does not conform with Main profile and Main10 profile since sps_extension_flag is equal to 1. However, Main profile and Main 10 profile decoders shall be able to decode this bitstream and shall ignore the SPS extension.

"

This issue should be confirmed again during the meeting.

### Max bins

In the following bitstreams, read_bits(1) called 5145 times, more than maximum of 5*RawCtuBits/3=5120 times (A.3.2 Main).

MAXBINS_A_TI_4

MAXBINS_B_TI_4

MAXBINS_C_TI_4

The revised bitstreams are available at the ftp site.

### Reference of Parameter Sets

In the following bitstream, pps_seq_parameter_set_id is set equal to 0, however, SPS is not encoded at this point.

RPS_C_ericsson_5.bit

In this bitstream, VPS/SPS/PPS are encoded in the following order.

NAL_TYPE 32 SIZE 31 [VPS]

NAL_TYPE 32 SIZE 204 [VPS]

NAL_TYPE 34 SIZE 12 [PPS]

NAL_TYPE 32 SIZE 31 [VPS]

NAL_TYPE 33 SIZE 253 [SPS]

NAL_TYPE 34 SIZE 12 [PPS]

NAL_TYPE 20 SIZE 9515 POC 0 AU 0

In PPS (marked in red), pps_seq_parameter_set_id is equal to 0. Referred SPS is not transmitted at this time. But PPS is activated when slice is decoded. When PPS is activated, SPS is available. Therefore it seems the bitstream is conforming, but it must be confirmed during Strasbourg meeting.

This raises another issue. Does the conformance set need an additional stream with different permutations of ordering of VPS, SPS and PPS? I would not be surprised if many decoders assume the conventional ordering and are thus non-conforming. It seems from the text that the design intention is to permit arbitrary ordering so long as all are seen before activation.

### general_profile_compatibility_flag

In the following bitstreams, the value of general_profile_compatibility_flag[0] is not equal to 0.

SLIST_A_Sony_4.bin

SLIST_B_Sony_8.bin

SLIST_C_Sony_3.bin

The revised bitstreams are available at the ftp site.

### New bitstream

The following new bitstream to test SAO corner case was provided.

SAO_H_Parabola_1

It was identified one weakness in the current bitstream set. The new feature of the bitstream is related to the treatment of CTU corner samples during SAO filtering of the 45 and 135 degree modes when slice_loop_filter_across_slices_enabled_flag = 0. The new bistream checks all four "corner" cases.

HEVC v.1 conformance test was finalized in Sapporo and it should be discuss how such new bitstreams be added to HEVC conformance test set.

### Originally planned bitstream features

The following bitstreams were originally planned to generate and the yellow highlighted bitstreams are not generated yet.

Table: Candidates of HEVC conformance test features

|  | Categories | Sub categories | Feature Code | Bitstream features | Volunteers |
| --- | --- | --- | --- | --- | --- |
|  | Intra coding | Intra prediction | IPRED | Functional stage: Decoding of intra PU. Purpose: Check that the decoder can properly decode all values of intraPredMode for different Luma and Chroma transform block sizes. | C.S. Lim, F. Bossen, J. Lainema, K. Sugimoto |
|  |  | Constraint intra prediction | CIP | Functional stage: Decoding of intra PU in non-intra slice when constrained_intra_pred_flag is set to one Purpose: Check that the decoder can properly perform reference sample substitution process. | C.S. Lim, K. Chono |
|  | Inter frame coding | AMVP | AMVP | Check whether motion vector prediction candidate generation and signalling perform correctly. Check the process when neighboring PUs have various partition mode, prediction mode, reference index, inter_pred_idc, etc.. Related syntax element: mvp_l0_flag, mvp_l1_flag | C. Kim, M. Zhou, Y. -W. Huang |
|  |  | MVs | MV | maximum number of motion vectors permitted per CTU |  |
|  |  |  | MV | vectors reference padded picture edges. | M. Coban |
|  |  |  | MV | clipping of motion vector range in MVP/Merge candidates | M. Coban |
|  |  |  | MV | motion vectors are random, maximum page DRAM misses / minimize cache hits (according to memory bandwidth model... e.g. JCTVC-H007) |  |
|  |  | Merge | MERGE | Merge candidate list index signaling: Check that merge candidate list can deal with every merge index (0..4) | B. Bross |
|  |  |  | MERGE | Parallel merge: test all the possible combinations of log2_parallel_merge_level_minus2 and CTB sizes. | M. Zhou, Y. -W. Huang |
|  |  |  | MERGE | number of maximum merge candidates signalling: test all the values of five_minus_max_merge_cand, i.e. 0, 1, 2, 3, 4 for CTB size of 64x64 | M. Zhou, |
|  |  | WP | WP | weighted_pred_flag and/or weighted_bipred_flag are equal to 1. | T. Chujoh |
|  |  |  | WP | various weighted prediction parameter combinations | T. Chujoh |
|  |  | mvd_l1_zero_flag | MVD | Test mvd_l1_zero_flag | F. Bossen |
|  |  | temporal_mvp | TMVP | Test slice_temporal_mvp_enable_flag | J. Xu |
|  |  | Collocated ref idx | RIDX | Test collocated_ref_idx (single slice or multiple slice/picture with different reference lists.) |  |
|  | HL syntax | slices, PSs | PS | bitstreams with stuffed extensions (vps_extension_data_flag=1, sps_extension_data_flag=1, pps_extension_flag=1, ..) | W. Jang, M. Coban |
|  |  | dependent slice | DSLICE | Check: - dependent slices - WPP with dependent slices - tiles with dependent slices | B. Bross |
|  |  | POC | POC | POC increments > 1 | M. Coban |
|  |  | RPS | RPS |  | M. Coban |
|  |  |  | RPS |  | F. Bossen |
|  |  |  | RPS | Slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Slice header RPS with short-term and long-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | PPS RPS with short-term pictures and inter-RPS prediction | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-term pictures | R. Sjöberg |
|  |  |  | RPS | Mix of PPS and slice header RPS with short-and long-term pictures | R. Sjöberg |
|  |  | RPL | RPL |  | M. Coban |
|  |  | RAP | RAP | all 6 (currently identified) RAP types | F. Bossen |
|  |  | HRD | HRD | maximum number of pictures in DPB | TBD |
|  |  |  | HRD | bitstream with largest possible CPB delay (maximum number of coded bits per picture but within MinCR) | TBD |
|  |  |  | HRD | bumping DPB | R. Sjöberg |
|  |  |  | HRD | Low delay, Sub picture | K. Kazui |
|  | Transform | RQT | RQT | Check all depths of inter & intra transform tree: QuadtreeTUMaxDepthInter : 0..4 QuadtreeTUMaxDepthIntra : 0..4 | B. Bross |
|  | Quantization | dQP | DQP | all depths of CU and diff_cu_qp_delta_depth | W. Wan, T. Suzuki |
|  |  | Scaling list | SLIST | non-default scaling lists | T. Suzuki, M. Zhou |
|  | Other coding tools | AMP | AMP | Check all asymetric partition mode enable | X. Zheng, C. Kim |
|  |  | Transform skip | TSKIP |  | J. Xu |
|  |  | I_PCM | IPCM | Functional stage: Parsing of I_PCM size information at SPS and I_PCM flag at CU. Purpose: Check that the decoder can properly decode pcm_flag for different intra CU sizes. Related Syntax: log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, pcm_flag. | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Parsing and decoding of I_PCM CUs. Purpose: Check that the decoder can properly decode I_PCM CUs with different PCM bit depth. Related syntax: pcm_sample_bit_depth_luma_minus1, pcm_sample_bit_depth_chroma_minus1, pcm_flag | W. Wan, C.S. Lim, K. Chono |
|  |  |  | IPCM | Functional stage: Decoding of I_PCM CUs with loop filter skipping. Purpose: Check that the decoder can properly decode I_PCM CUs by skipping loop filter. Related syntax: pcm_loop_filter_disable_flag, pcm_flag. Note: Combination with cu_transquant_bypass_flag will be tested. | W. Wan, C.S. Lim, K. Chono |
|  |  | Lossless | LOSSLESS | transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles lossless coding. | F. Henry |
|  | Parallel processing tools | Tile | TILE | Maximum number of tiles in a random, non-uniform arrangement that changes from picture to picture, with random chosen value for loop_filter_across_tiles_enabled_flag, deblocking_filter_override_flag, slice_disable_deblocking_filter_flag, slice_loop_filter_across_slices_enabled_flag | A. Fuldseth, M. Zhou |
|  |  | WPP | WPP | entropy_coding_sync_enabled_flag on. Purpose: Check that the decoder handles wavefront parallel processing | F. Henry |
|  |  | WPP and slices | WPP | entropy_coding_sync_enabled_flag on, number of slice varies randomly between 2 and maximum number of slices permitted, each non-first slice is either regular or dependent in a random way. Purpose: Check that the decoder handles wavefront parallel processing and slices | F. Henry |
|  |  | Tile & WPP | WPP | entry point of WPP | Hendry |
|  | Temporal scalability |  | TSCL |  | W. Jang |
|  |  |  | TSCL | range of nested temporal_id's |  |
|  | Deblocking filter |  | DBLK | various cb, cr qp_offsets | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | various beta_offset_div2, tc_offset_div2 | T. Suzuki, R. Sjöberg |
|  |  |  | DBLK | trigger all deblocking decisions | T.Suzuki, R. Sjöberg |
|  |  |  | DBLK | deblocking + tile, etc |  |
|  | SAO |  | SAO | Check all CTU sizes with max. SAO information rate. | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random and extreme SAO offsets values | E. Francois, Y. -W. Huang |
|  |  |  | SAO | Random SAO merge left/up flag values | E. Francois, Y. -W. Huang |
|  |  | Slice- and component-level control | SAO | Pictures divided into x slices / randomly enabling SAO Y and/or SAO UV per slice | E. Francois |
|  |  | Tile- and component-level control | SAO | Pictures divided into x tiles / randomly enabling SAO Y and/or SAO UV per tile | E. Francois |
|  | Entropy coding |  |  |  |  |
|  |  | Sign data hiding | SDH | sign_data_hiding_flag on, transquant_bypass_enable_flag on, cu_transquant_bypass_flag is set to 0 and 1 randomly at CU level Purpose: Check that the decoder handles sign data hiding with lossless coding. | H. Henry |
|  |  | High through put binalizaion | HTB |  | M. Coban |
|  |  | cabac_init_idc | CAINIT |  | A. Segall |
|  |  | Max number of bins | MAXBIN |  | V. Sze |
| Corner case bitstream |  |  |  | Maximum number of slices permitted for Profile & Level combination; Each slice has unique short_term_ref_pic_set(), ref_pic_list_modification() and pred_weight_table() with maximum num_long_term_pics | C. Fogg |
|  |  |  |  | CTU size is 64x64 with no cu splits, and only one coded transform unit in the lower right 4x4 pixel corner of the CTU with delaQp signalled (as raised by P. Kaspenberg on the JCT reflector 12-Sept-2012) | C. Fogg |
|  |  |  |  | A wide range of values for pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, pic_crop_bottom_offset | C. Fogg |
|  |  |  |  | inferred splits along non-CTU multiple picture borders |  |
|  |  |  |  | combinations of transform_skip_enabled_flag, transquant_bypass_enable_flag, cu_transquant_bypass_flag |  |
|  |  |  |  | something to test the vast universe that is: POC signalling |  |
|  | Video resolution |  |  | pic_width_in_luma_samples = pic_height_in_luma_samples = (1 << Log2MinCbSize), with various CTU sizes. |  |

## RExt conformance

### Initial plan

- Bit depth & chroma format to cover in RExt conformance are as follows.

- 4:0:0, 4:2:0, 4:2:2, 4:4:4

- 8, 10, 12, 16b

Profiles in RExt are as follows.

Monochrome 12, Monochrome 16, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, and Main 4:4:4 16 Intra profiles

The initial idea of bitstream features is summarized in the following Table. Some of the bitstreams had already been generated and are available at the following site.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/RExt/

Table: Candidate features of RExt conformance

| Chroma format | Bit depth | Category | Sub category | Bitstream | Volunteers | Candidates |
| --- | --- | --- | --- | --- | --- | --- |
| 4:2:2 | 10 | CU/TU/PU |  |  |  |  |
| 4:2:2, 4:4:4 |  |  | CBF |  |  | Qualcomm/Canon/Various? |
| 4:2:2, 4:4:4 |  | Intra coding | Intra smoothing disable |  | Fujitsu | Fujitsu ? |
| 4:2:2 |  |  | Adjustment to intra chroma prediction angle | ADJUST_IPRED_ANGLE_A_RExt_Mitsubishi_1 | A.Minezawa | Mitsubishi |
| 4:2:2, 4:4:4 |  | Inter frame coding | WP (high_precision_offsets_enabled_flag) |  | Qualcomm & Docomo |  |
| 4:4:4 |  |  | Cross component prediction | CCP_8bit_RExt_QCOM CCP_10bit_RExt_QCOM CCP_12bit_RExt_QCOM | Qualcomm | HHI/Qualcomm |
| 4:4:4 | L/C 10/8 8/10 |  | Different bit depth for luma and chroma | Bitdepth_A_RExt_Sony_1 Bitdepth_B_RExt_Sony_1 | Sony |  |
|  |  | Transform |  |  | Sony |  |
|  |  | Quantization | Scaling list |  | Sony | Sony ? |
|  |  |  | Chroma_qp_adjustment |  | Black Berry | Black Berry ? Apple ? |
|  |  | Loop filter | Deblocking |  |  | ? |
|  |  |  | SAO | SAO_A_RExt_MediaTek_1 | Media Tek | Samsung ? Media Tek ? |
|  |  | Entropy coding | Rice parameter initialization |  | Qualcomm | Qualcomm, Sony ? |
|  |  |  | Persistent Rice parameter tool | PERSIST_RPARAM_A_RExt_Sony_1 | Sony |  |
|  |  |  | CABAC bypass alignment |  | Sony | Qaulcomm ? Sony ? |
| 4:4:4 | 16 | Precision | Extended precision |  | Sony |  |
|  |  | HL syntax |  |  |  |  |
| 4:2:2, 4:4:4 | 10, 12, 16 | Others | PCM | IPCM_A_RExt_NEC IPCM_B_RExt_NEC | NEC | NEC |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip rotation |  | BlackBerry ? MediaTek | Microsoft, Qualcomm (N0288) |
| 4:4:4 | 8, 10, 12, 16 |  | Transform skip context | TSCTX_8bit_I_RExt_SHARP_1, TSCTX_8bit_RExt_SHARP_1, TSCTX_10bit_I_RExt_SHARP_1, TSCTX_10bit_RExt_SHARP_1, TSCTX_12bit_I_RExt_SHARP_1, TSCTX_12bit_RExt_SHARP_1 | Sharp | Qualcomm (N0044) Sharp |
| 4:4:4 | 8, 10, 12, 16 |  | RDPCM (implicit & explicit) | ExplicitRdpcm_A_BBC_1 ExplicitRdpcm_B_BBC_1 | Samsung BBC(explicit) | Media Tek (P0154) |
| 4:4:4 | 8, 10, 12, 16 |  | Max transform skip block size |  | BBC | Many. Perhaps BBC? |
| 4:2:2 | 10 |  | Various combination of tools/parameters | Main_422_10_A_RExt_Sony_1 Main_422_10_B_RExt_Sony_1 | Sony |  |
| 4:4:4 | 8/16 |  | Various combination of tools/parameters |  | Sony |  |
| 4:0:0 | 8/16 |  |  |  | Fujitsu ? |  |

Basic features can be combines into bitstreams to reduce the number of conformance bitstreams. Volunteers identified so far are as follows.

- O. Nakagami (Sony)

- D. Flynn (RIM)

- K. Sharman (Sony)

- K. Kazui (Fujitsu)

- D. Nicholson (VITEC)

- A. Minezawa (Mitsubishi)

- K. Chono (NEC)

- Yuwen (yuwen.huang@mediatek.com) (MediaTek)

- M. Naccari (BBC)

- J. Sole (Qualcomm)

- P. Onno (Canon)

- E. Alshina (Samsung)

- K. Tsukuba (Sharp)

### Plan

#### Procedure

- Same as HEVC v.1 conformance

- See bitstream generation guideline in JCTVC-O1010

#### Consideration

Consider to remove the tool from the text if no conformance stream, same as HEVC v.1

## SHVC conformance

This section discusses some considerations for testing the conformance of SHVC decoders.

An SHVC bitstream can contain multiple layers and multiple output layer sets. Each output layer set specified in the bitstream is a conformance point that needs to be tested.

For each bitstream, we define a decoded layer YUV file of a particular layer in the bitstream as the YUV file containing all the decoded pictures in that particular layer, irrespective of whether it is an output layer or not and irrespective of the value of PicOutputFlag, in its output order.

Potential requirements for conformance streams (All mandatory unless mentioned as Optional):

- Bitstream file (name.bit)

- Description of the bitstream (name.txt)

- Output picture log for each output layer set defined in the bitstream (name-OLS#.opl)

  - Log file with one row for each picture in the bitstream that is output (in the same order that they are output) including the following information of the picture:

    - NUH Layer ID

    - POC

    - MD5 checksum

- For each layer in the bitstream, MD5 check sum of the (entire) decoded layer YUV file of the layer.name-L#. md5

- Trace file (name.trc) (optional)

- Picture decoding hash SEI message in bitstream (optional)

All of the above files should be contained with a name.zip file.

Full conformance is tested by comparing the output picture log file, which ensures that the proper pictures are output, in the proper order, with the proper YUV values.  Comparison with the decoded layer YUV MD5 files are not strictly required for conformance, because a conforming decoder is not required to output decoded pictures which are not indicated for output, but are provided as a tool for implementers.

A preliminary list of SHVC conformance bitstreams to be prepared has been identified. It is included in an attached spreadsheet.

So far, 11 companies have volunteered to participate in generation of SHVC conformance bitstreams: Ericsson, ETRI, InterDigital, Fraunhofer HHI, Nokia, Qualcomm, Samsung, Sharp, Sony, Technicolor, and Vidyo. Additional companies are requested to volunteer.

It is recommended that at this meeting the preliminary list of conformance bitstreams be reviewed and volunteers identified for each bitstream.

# Recommendations

The AHG recommends

- to approve the corrections of HEVC v.1 conformance bitstreams

- to discuss and clarify the open questions to HEVC v.1 conformance bitstreams

- to discuss how new bitstreams for HEVC v.1 be added

- to discuss the further plan of HEVC extensions conformance test

- to update the guideline of the bitstream generation for RExt and SHVC conformance